3GPP TSG-RAN WG4 Meeting #84bis 											      R4- 1711412

Dubrovnik, Crotia, 9 – 13 Oct, 2017

Source: 	Huawei, HiSilicon

Title: 	TP for TR 36.758: Specific issue for TDD operation in 3300-3400MHz band – Channel numbering for E-UTRA

Agenda Item:	8.12.1

Document for:	Approval

# Introduction

During last 3GPP TSG RAN Meeting #76 in West Palm Beach, USA June 5 - 8, 2017, a new WI about the study of the TDD in the C-band (3300-3400 MHz) has been approved.

This contribution provides the TP for the TR 36.758, about the channel numbering.

# Discussion

## Background

The text below is to define the UE sensitivity in the 3300-3400MHz band for Africa countries.

# Text proposal for TR

<Unchanged sections omitted>

# 10	Channel numbering for E-UTRA, MSR

For 3300-3400MHz Band the E-UTRA frequency band is defined as in Table 10-1.

Table 10-1 E-UTRA frequency bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 52 | 3300 MHz | – | 3400 MHz | 3300 MHz | – | 3400 MHz | TDD |

EARFCN for 3300-3400MHz Band is defined as in Table 10-2.

Table 10-2: EARFCN allocated for E-UTRA Band 52

| E-UTRA Operating Band | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FDL_low [MHz] | NOffs-DL | Range of NDL | FUL_low [MHz] | NOffs-UL | Range of NUL |
| 52 | 3300 | 59140 | 59140 - 60139 | 3300 | 59140 | 59140 - 60139 |

<Unchanged sections omitted>

# Conclusion

It is proposed:

Proposal: Add the text as proposed in the paragraph “Text proposal for TR” in the TR [4]

# References

[1] RP-171506 “New LTE band for 3.3-3.4 GHz for Africa”, Huawei, HiSilicon

[2] R4-1708452 “Regulation background for TDD C-Band (3300-3400MHz) for Africa”; Huawei, HiSilicon

[3] R4-1709188 “LS to African Telecommunications Union about Regulation background to protect incumbent users in the band: 3.3-3.4GHz”, 3GPP RAN WG4

[4] TR 36.756 “3rd Generation Partnership Project; Technical Specification Group Radio Access Networks; TDD operating in 3300-3400MHz band in Africa for LTE (Release 15)”

[5] R4-1710124 “Response to the LS R4-1709188 from 3GPP to ATU about regulations background to protect incumbent users in the band”, African Telecommunications Union