3GPPRAN4#84-Bis	R4-1711514

Dubrovnik, Croatia, October 9th-13th, 2017

Title: 	Initial timing error and timing advance adjustment accuracy in NR

Source: 	Qualcomm Incorporated

Agenda item:	9.7.6.1 - UE transmit timing, UE timer accuracy and timing advanced [NR_newRAT]

Document for:	Discussion

# Introduction

In this paper, we review the initial transmit timing error and timing adjust accuracy for LTE and propose extensions to NR. Initial timing error depends on the DL BW used for timing estimation along with some margin for estimation error and delay difference in UL/DL.

# Discussion

## Initial Transmit Timing Error

In LTE, initial transmit timing error from 36.133 is reproduced here in Table 3

Table 1: Initial Timing Error for LTE

| Downlink Bandwidth (MHz) | Te_ |
| --- | --- |
| 1.4 | 24*TS |
| ≥3 | 12*TS |
| Note: TS is the basic timing unit defined in TS 36.211 |  |

UL timing error is based on DL timing estimation. The error in DL timing estimation depends on the BW of the signal used. In LTE, CRS is used to obtain DL timing, which for 3MHz case would have a BW of 2.7MHz.

The minimum resolution for DL timing would be the inverse of signal BW. Table 2 shows the minimum timing error for LTE 3MHz case. The margin specified in the table the difference of min error from the 36.133 specification of 12Ts. This margin would comprise of both estimation error in DL timing and RF delay difference in UL vs DL.

Table 2: Initial Transmit Timing Error for LTE 3MHz

| DL BW | CRS BW | Minimum Timing Error (in Ts) | Margin (in Ts) |
| --- | --- | --- | --- |
| 3 | 2.7 | 5.688889 | 6.311111 |

In order to derive initial transmit timing error requirements for NR, we would be able to scale the minimum timing error that is based on the BW of the signal used for DL estimation. The margin scaling should not change between LTE and NR.

In NR, one possibility is to use PSS/SSS signal for DL timing estimation. Table 3 lists the PSS/SSS BW and the corresponding minimum timing error for each SCS.

Table 3: Initial Transmit Timing Error for NR

| SCS | PSS/SSS BW | Min Timing Error (Ts) | Margin | Initial Timing Error (Ts) |
| --- | --- | --- | --- | --- |
| 15 | 1.905 | 8.062992 | 6.311111 | 14 |
| 30 | 3.81 | 4.031496 | 6.311111 | 10 |
| 60 | 7.62 | 2.015748 | 6.311111 | 8 |
| 120 | 15.24 | 1.007874 | 6.311111 | 7 |

In case PBCH DMRS is used for DL timing estimation, we can come up with a similar table based on PBCH BW. There are still on going discussion on PBCH design in RAN1. Once that is done, we can revisit the uplink timing specification.

## Timing Advance Adjustment Accuracy

The timing advance adjustment accuracy is the relative error after applying TA command. In LTE, this number is defined as 4Ts across all BW’s. We see no reason to scale this with respect to SCS.

# Conclusions

Proposal 1: For NR, the initial transmit timing error should be

| SCS | Initial Timing Error (Ts) |
| --- | --- |
| 15 | 14 |
| 30 | 10 |
| 60 | 8 |
| 120 | 7 |

Proposal 2: For NR, the timing advance adjustment accuracy requirement should be kept same as LTE  +/-4 Ts