3GPP TSG-RAN WG4 RAN4-84Bis	R4-1711806

Dubrovnik, Croatia, 9-13 October, 2017

Source:		Ericsson

Title:			Proposal on radiated immunity - exclusion band

Agenda item:		8.27.3.6

Document for:	Approval

# Introduction

This document provides a modification to the proposals presented in R4-1710938 [1].

# Discussion

Based on the results presented in [1] and the feedback collected during the discussion, the following proposals are presented for approval:

Proposal 1: For RI testing of OTA AAS BS applies around the DUT, except fot the half sphere around the main lobe direction).

Proposal 2: For the frequency range 690 MHz to 6 000 MHz test level shall be 3 V/m, except for frequency bands where colocation bands has been declared.

In the existing TS 36.104, TS 36.113 the exclusion for OOB blocking and RI are the same (20 MHz).

Proposal 3: For RI testing of OTA AAS BS, the RX exclusion band shall be specified according to limits defined in the RF discussion.

# Conclusion

Based on the discussion in Section 2, it is proposed to approve the following proposals:

Proposal 1: For RI testing of OTA AAS BS applies around the DUT, except for the half sphere around the main lobe direction.

Proposal 2: For the frequency range 690 MHz to 6 000 MHz test level shall be 3 V/m, except for frequency bands where colocation bands has been declared.

Proposal 3: For RI testing of OTA AAS BS, the RX exclusion band shall be specified according to limits defined in the RF discussion.

# References

R4-1710938, TP on radiated immunity – exclusion bands. Ericsson.

R4-1707480,   Discussion on RI testing of OTA AAS BS and the consequences for receiver blocking, Ericsson, RAN4-#84.

R4-1710937, Discussion on extended exclusion band for immunity (assuming spatial exclusion). Ericsson..