3GPP TSG-RAN WG4 #84bis	R4- 1711858

Dubrovnik, Croatia, 9th –13th October 2017

Source:	Ericsson

Title:	DRAFT TS 37.105 Specification text for Section 10.6 OTA RX Blocking

Agenda item:	8.27.2

Document for:	Approval

# 1.	Introduction

In this contribution a text proposal is presented for OTA RX blocking section of TS 37.105 and we would like to encourage feedback from other companies on the proposed text.

# 2.	Discussion

Based on the time line proposed timeline OTA blocking sections of the TS shall be drafted.  It may be too early to draft any specification text since only a few contributions have been presented on this topic [3,4].

This contibution is a basis to start drafting OOB specification text for the TS.  Some challenges with an OTA OOB requirement has been presented [3] and as such the table shown below in the TP shall be considered only as a baseline.  To reach a value of -15 dBm may not be a practical solution.  One approach to consider would to be used a fixed level at the test range antenna connector and a fixed distance to the eAAS BS.  The assumed eAAS BS orientation would also need to be further considered.  This aspects would need to be captured and hence only a [TBD] in the column below in the TP.

# 3.	References

3GPP TS 37.105 v14.0.0

R4-1706126, “WF on Spec drafting”, Huawei

R4-1707477, “Further discussion on OOB blocking challenges”, Ericsson

R4-1705649, Receiver out of band blocking”, Huawei

[Start of Text Proposal]

## 10.6	OTA Blocking

### 10.6.1	General

The blocking characteristics are a measure of the receiver unit ability to receive a wanted signal at the  RIB at its assigned channel in the presence of an unwanted interferer.

### 10.6.2	Minimum requirement for MSR operation

#### 10.6.2.1	General minimum requirement

The wanted signal and a fixed out of band blocking interferer OTA power level [EIRP] are from the same direction.  The OTA Out of Band blocking requirements are valid over the receiver RoAoA associated with [minSENS].

For a wanted and an interfering signal coupled to the RIBusing the parameters in table 10.6.2.1-1, the following requirements shall be met:

-	For any E-UTRA carrier, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channel defined in 3GPP TS 36.104 [8], subclause 7.2.1.

-	For any UTRA FDD carrier, the BER shall not exceed 0,001 for the reference measurement channel defined in 3GPP TS 25.104 [6], subclause 7.2.1.

-	For any UTRA TDD carrier, the BER shall not exceed 0,001 for the reference measurement channel defined in 3GPP TS 25.105 [7], subclause 7.2.1.2.

For multi-band RIB, the requirement applies for each supported operating band. The in-band blocking frequency ranges of all supported operating bands according to Table 10.6.2.1-1 shall be excluded from the requirement.

Table 10.6.2.1-1: Blocking performance requirement

| Operating Band Number | Centre Frequency of Interfering Signal (MHz) |  |  |  | Interfering Signal mean power (dBm) | Interfering Signal distance (m) | Wanted Signal mean power (dBm) | Type of Interfering Signal | [TBD] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-7, 9-11, 13, 14, 18, 19, 21-23, 24, 27, 30, 33-45, 65, 66, 68 | [1 ] (FUL_high +20) |  | to to | (FUL_low -20) 12750 | [TBD] | [TBD] | P[minSENS]] + x dB (NOTE) | CW carrier | [TBD] |
| 8, 26, 28 | [1 ] (FUL_high +10) |  | to to | (FUL_low -20) 12750 | [TBD] | [TBD] | P [minSENS] + x dB (NOTE) | CW carrier | [TBD] |
| 12 | [1] (FUL_high +13) |  | to to | (FUL_low -20) 12750 | [TBD] | [TBD] | P [minSENS] + x dB (NOTE) | CW carrier | [TBD] |
| 17 | [1 ] (FUL_high +18) |  | to to | (FUL_low -20) 12750 | [TBD] | [TBD] | P[minSENS] + x dB (NOTE) | CW carrier | [TBD] |
| 20 | [1 ] (FUL_high +20) |  | to to | (FUL_low -11) 12750 | [TBD] | [TBD] | P[minSENS] + x dB (NOTE) | CW carrier | [TBD] |
| 25 | [1 ] (FUL_high +15) |  | to to | (FUL_low -20) 12750 | [TBD] | [TBD] | P[minSENS] + x dB (NOTE) | CW carrier | [TBD] |
| 31 | [1 ] (FUL_high +5) |  | to to | (FUL_low -20) 12750 | [TBD] | [TBD] | P[minSENS] + x dB (NOTE) | CW carrier | [TBD] |
|  |  | NOTE: PREFSENS depends on the RAT, the BS class and the channel bandwidth, see subclause 7.2.2. "x" is equal to 6 dB in case of E-UTRA or UTRA wanted signals. |  |  |  |  |  |  |  |

[End of Text Proposal]