3GPP TSG-RAN WG4 Meeting #86	R4-1802214

Athens, Greece, 26 February – 2 March 2018

Source: 	T-Mobile, C Spire, US Cellular, Dish

Title: 					On Extended-Band 12 BS output power

Agenda Item: 			6.12.3.

Document for:	Approval

# 1	Introduction

Part 1 of the WI approved in RAN#77 [1] is a study phase to look at the co-existence impacts between E block and A block. In RAN4 Meeting #85, simulation results were presented [2], and in RAN4 Meeting #86 another set of simulation results were presented in [3]

# 2	Discussion

[2] has provided the urban macro downlink coexistence simulation results using the agreed simulation assumptions and parameters for E-UTRA and NB-IoT coexistence studies. [3] has provided the urban macro downlink co-existence simulation results using different inter-side distances between victim and aggressor.

The simulation results in [2] have shown that the downlink SINR of the victim UE are comparable in the 3 scenarios until the very low end where the SINR in Scenarios 2 and 3 are decreased (<0.5dB at 5%-tile).. The simulation results in [3] have shown that the cell edge SINR degrades and respective outage increases as the power for each NB-IoT carrier increases.

Extended-band 12 interested companies had discussed and agree to address the concern based on the Nokia simulation [2] and Dish simulation [3], and accordingly to limit the maximum power of the NB-IoT deployed in the lowest 1 MHz of the band. A Text Proposal for TR 36.716 is provided in Annex A.

# 3	Conclusion

In this contribution, we discussed Extended-band 12 co-existence and provide a text proposal.

Proposal: It is proposed that the text proposal in Annex A is approved.

# 4	References

# [1]   RP-172111 “New WI: Extended-Band12 new E-UTRA Band”, T-Mobile USA, US Cellular, and C-Spire

# [2]   R4-1712682 “Urban Macro Downlink Coexistence Simulation Results for Extended-Band12”, Nokia

# [3]   R4-1802027 “Co-existence for B29/B12 Extended band New E-UTRA band”, Dish network

# Annex A 		Text Proposal for TR 36.716

<Begin Changes>

x.x maximum BS power of NB-IoT deployed in the lowest 1 MHz

When NB-IoT is deployed in the lowest 1 MHz paired spectrum, specifically 698-699 MHz uplink and 728-729 MHz downlink, the maximum BS power is 43dBm shared among the NB-IoT carriers. (e.g. the maximum BS power per NB-IoT carrier is 43 dBm for a single carrier deployment and 37 dBm for a four carrier deployment)

<End Changes>