3GPP TSG-RAN WG4 #87	R4-1806435

Busan, Korea, 21 - 25 May 2018

Agenda Item:	7.5.8.2

Souce:		Samsung

Title:	Network performance analysis for spherical coverage

Document for:	Discussion and decision

# Introduction

In the last meeting, the min peak EIRP for FR2 has been agreed. The only remaining issue is the 50%-tile requirement for EIRP CDF.

Agreement: • For Rel-15 • For 28GHz • Min peak EIRP is 22.4 dBm • 50%-tile requirement for EIRP CDF is FFS • For 39GHz • Min peak EIRP is 20.6 dBm • 50%-tile requirement for EIRP CDF is FFS Note: for 50%-tile, the values are FFS.

A summary of the network performance analysis for spherical coverage performed by all participating companies so far has been provided in R4-1805930 [1]. In this paper, we provide further network performance sensitivity to spherical coverage at 50%-tile point for information.

# Network performance sensitivity to EIRP CDF at 50%-tile

In this section, we conduct analysis of network performance sensitivity to spherical coverage by providing the network performance results for different EIRP values at the 50%-tile point. In particular, based on the simulation data for UE assumption 1 (see Appendix), we consider deviation of -3dB, -2dB, 0dB, 2dB and 3dB from 13.8dBm (-8.6dB from the peak EIRP) at 20%tile point. The EIRP CDFs are shown in Figure 1.

The simulation results for urban macro with ISD of 200m and 200MHz bandwidth are summarized in Table 1. The UL throughput changes with respect to that for 0dB deviation are recorded. It can be observed that with deviation of -3dB to +3dB at the 50%-tile, the mean UL throughput change ranges from -6.17% to 7.23%, whereas the 5%-tile UL throughput change ranges from -30.94% to 27.88%. The change in the mean throughput is significantly less compared to that in the 5%-tile throughput.

Figure 1: EIRP CDFs for network performance sensitivity analysis

Table 1: Result for urban macro with ISD of 200m and 200MHz bandwidth

| Deviation from a ref |  | UL throughput change wrt that for 0dB deviation |  |
| --- | --- | --- | --- |
|  |  | Mean | 5%-tile |
| 50%-tile | -3dB | -6.17% | -30.94% |
|  | -2dB | -4.42% | -18.77% |
|  | 0dB | 0% | 0% |
|  | +2dB | 4.79% | 20.66% |
|  | +3dB | 7.23% | 27.88% |

# Conclusions

In this contribution, we provide our network performance sensitivity to spherical coverage at 50%-tile point for information purpose.

Observation: With deviation of -3dB to +3dB at the 50%-tile of EIRP CDF, the mean UL throughput change ranges from -6.17% to 7.23%, whereas the 5%-tile UL throughput change ranges from -30.94% to 27.88%. The change in the mean throughput is significantly less compared to that in the 5%-tile throughput.

# Reference

- R4-1805930   Summary of network performance analysis for spherical coverage, Samsung

# Appendix: UE assumptions for spherical coverage