3GPP TSG-RAN WG4 Meeting #87	R4-1807487

# Busan, KR, 21st – 25th May, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR: TRP measurement method for spurious emissions in shielded anechoic chamber |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA |  |  |  |  |  |  | Date: |  | 2018-05-15 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For adding a measurement method for TRP for spurious emissions in shielded anechoic chamber |  |  |  |  |  |  |  |  |
| Summary of change: |  | A TRP measurement method is added in an annex |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There are no TRP measurement methods in the technical report |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Appendix |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# Annex M TRP evaluation for unwanted emissions

## M.5 TRP measurement procedure for spurious emissions

In order to get an accurate value of the TRP, a large set of angular points may be required (e.g. if Δθref and Δϕref are very small), especially for spurious emissions at higher frequencies. Therefore, there is a need to have a method that allows for sparse spherical scanning or a set of one-dimensional scans. One important observation is that spurious emissions coming from different sources (e.g. array elements, or other radiation source as leakage through slots or holes in the chassis) are likely to be uncorrelated in which case the requirement on the number of measurement points can be relaxed compared to the reference angular steps.

A Sparsity Factor (SF) is used to describe the relation between the chosen grid and the reference grid (i.e. the grid defined by the reference angular steps). SF is utilized to quantify grid-related errors due to sparse sampling. By definition:

(ref)

For example, if  ,  and we select to use  and  , then SF = 10.

Based on the grid type and the sparsity factor SF, a systematic correction, denoted ΔTRP, can be calculated [1].

The measurement procedure is the following

- Calculate reference angular steps Δθref and Δϕref using (ref).

- Select appropriate measurement antenna and test distance, see [ref]

- For each emission frequency, select the grid type: full sphere or orthogonal cuts

- For each emission frequency, set the grid spatial sampling step Δθ and Δϕ.

  - If  D≤4λ , or if set Δθ=Δϕ=15∘

  - If D>4λ

    - for TRP assessment with a margin, set Δθref≤Δθ≤15∘ and Δϕref≤Δϕ≤15∘. Calculate the sparsity factor (SF) using (ref). When calculating reference angular steps, the maximum frequency for the considered emission measurement shall be used.

    - for error-free TRP assessment, set Δθ≤Δθref and Δϕ≤Δϕref, corresponding to sparsity factor SF=1.

- For each emission frequency, measure the total power density (ref) on the selected grid.

- For each emission frequency, calculate the TRPgrid from the measured power density values choosing suitable method as described in [ref] then add appropriate ΔTRP from the table below in order to obtain the estimated value for the TRP:

Table 4-1. TRP systematic correction for spurious emissions – ΔTRP

| Grid type: | 2-cuts | 3-cuts | Full Sphere |
| --- | --- | --- | --- |
| ΔTRP (dB) | TBD | TBD | TBD |
| Note |  |  | SFmax corresponds to Δϕ=15∘ |
| These values are the results of a large statistical investigation, as described in [2], based on an assumed maximum correlation factor ρmax=[0.33]). The statistical confidence is 95%. The values do not apply for harmonics of the wanted signal, characterized by a larger correlation factor. |  |  |  |

References:

[1]	R4-1804994 On TRP systematic correction when using sparse sampling, Ericsson, April 2018

[2]	Fridén, J.; Razavi, A. and Stjernman, A. “Angular sampling, Test Signal, and Near Field Aspects for Over-the-Air Total Radiated Power Assessment in Anechoic Chambers” Submitted to a Journal, 2018. Available online:  https://arxiv.org/abs/1803.10993