3GPP TSG-RAN WG4 Meeting #87															R4-1808085

Busan, Korea (Republic of), 21 - 25 May 2018

Source: 	Nokia

Title: 					n8 A-MPR

Agenda Item: 			7.2.2.1.1

Document for:	Discussion

# 1	Introduction

This contribution presents simulation results for n8 A-MPR based on agreed WF [1] and [2].

# 2	Discussion

- Protection requirement: -40dBm/MHz @ 860 – 890MHz, [2]

- The following scenarios are sufficient [1]

- 5MHz @ 900-905MHz,

- 10MHz @ 905-915MHz and

- 15MHz @ 900-915MHz

- Inner and outer allocation regions according to NR 38.101 Section 6.2.2.

Table 1. A-MPR evaluation results for 5/1015 MHz carriers on n8

| Allocation type |  | Outer (max MPR) | Inner (max MPR) |
| --- | --- | --- | --- |
| LCRB |  | All | ≤LCRBmax/2 |
| Distance from channel edge |  | <LCRB/2 from edge | ≥LCRB/2 from edge |
| WF type | modulation | MPR For all BW and SCS |  |
| DFT-s-OFDM | pi/2-BPSK | 0.5 / 1.5 / 3.0 | -1.0 / -1.0 / 3.5 |
|  | QPSK | 2.0 / 2.5 / 5.5 | -1.0 / -1.0 / 4.5 |
|  | 16QAM | 2.0 / 2.5 / 5.0 | 0.0 / 0.5 / 5.5 |
|  | 64QAM | 2.0 / 3.0 / 5.5 | 1.5 / 1.5 / 5.0 |
|  | 256QAM | 3.0 / 3.5 / 5.5 | 3.0 / 3.0 / 5.5 |
| CP-OFDM | QPSK | 3.0 / 4.0 / 7.5 | 0.5 / 1.0 / 7.5 |
|  | 16QAM | 3.0 / 4.0 / 7.0 | 1.5 / 2.0 / 8.0 |
|  | 64QAM | 3.0 / 4.0 / 7.5 | 3.0 / 3.0 / 8.0 |
|  | 256QAM | 5.5 / 5.5 / 7.5 | 5.0 / 5.0 / 8.0 |

In here, it should be noted that for 5 MHz channel with 20 MHz frequency offset between the lower band edge and lower channel edge from the lower edge of band n8 (@ 900 – 905 MHz), A-MPR would be necessary only with the very widest allocations of DFT-S-OFDMA using QPSK modulation as shown Figure 1.

Figure 1. Backoff, if exceeding MPR for BW = 5 MHz and frequency offset between lower band edge and lower channel edge = 5 MH with 15 kHz SCS and DFT-S-FDMA using QPSK modulation.

For this BW = 5 MHz scenario, all other deltas between simulated backoff and MPR are equal to or less than zero.

For BW=10 MHz and BW = 15 MHz scenarios described above, backoff, if exceeding MPR figures are shown in the following attachment files:

# 3	Conclusion

In this contribution we have presented n8 A-MPR results.

# 4	References

[1] R4-1803744 NR A-MPR evaluation scenarios for n1 and n8 for Japan, RAN4#86bis

[2] R4-1702772