3GPP TSG-RAN WG4 Meeting #90	R4-1900791

# Athens, GR, 25th Feb – 1st March, 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on L1-RSRP measurement for reporting (Section 9.5.4.2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-02-12 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The evaluation period is incomplete for CSI-RS L1-RSRP reporting in FR2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the condition of N=1 for CSI-RS L1-RSRP reporting |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The case of non-Rx beam sweeping (N=1) is missing in CSI-RS for L1-RSRP reporting requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 9.5.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS38.521-2 and TS38.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change>>

9.5.4.2	CSI-RS based L1-RSRP Reporting

The UE shall be capable of performing L1-RSRP measurements based on the configured CSI-RS resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TBM_Measurement_Period_CSI-RS.

The value of TBM_Measurement_Period_CSI-RS is defined in Table 9.5.4.2-1 for FR1 with

-	M=TBD for measurement on periodic and semi-persistent CSI-RS resources

-	M=1 for aperiodic CSI-RS resources

The value of TBM_Measurement_Period_CSI-RS is defined in Table 9.5.4.2-2 for measurement on periodic and semi-persistent CSI-RS resources in FR2 with

-	M=TBD and

-	N=1,

-	if the CSI-RS for L1-RSRP reporting is QCL-TypeD with SSB for L1-RSRP beam reporting, and the CSI-RS for L1-RSRP reporting and SSB for L1-RSRP beam reporting are TDM’d, and the CSI-RS for L1-RSRP reporting is not in a resource set configured with repetition ON.

Or

-	if the CSI-RS for L1-RSRP reporting is QCL-TypeD with CSI-RS for L1-RSRP beam reporting with repetition parameter ON, and the CSI-RS for L1-RSRP reporting and CSI-RS for L1-RSRP beam reporting are TDM’d, and the CSI-RS for L1-RSRP reporting is not in a resource set configured with repetition ON.

-	N=8, otherwise.

The value of TBM_Measurement_Period_CSI-RS is defined in Table 9.5.4.2-2 for aperiodic CSI-RS resources in FR2 with

-	M=1 and N=1

<< End of Change>>