3GPP TSG-RAN WG4 Meeting # 90	R4-1901533

# Athens, Greece,  25 February - 1 March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.113 | CR | CRNum | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.113: editorial corrections to incorrect reference number and references |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-02-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Reference number is incorrect in the main body of the text and and reference [28] appears to be incorrect. Furthermore, reference [12] was deleted but is still referenced in the text. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove reference [12] from the text and reference number is corrected and reference [28] is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect reference number and references remain. Therefore, the text is ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.1, 9.2.2, 9.5, 9.6.2, 9.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

# 2	References

<unmodified references omitted>

[28]	ETSI EN 301 489-50: “Electromagnetic compatibility (EMC) standard for radio equipment and services; Part 50: Specific conditions for cellular communication base station (BS), repeater and ancillary equipment; Harmonised standard covering the essential requirements of article 3.1(b) of the Directive 2014/53/EU

<End of modified section>

<Start of next modified section>

## 7.1	Emission

Table 7.1-1: Emission requirements applicability

| Phenomenon | Application | Equipment test requirement |  | Reference subclause in the present document | Reference Standard |
| --- | --- | --- | --- | --- | --- |
|  |  | BS equipment | Ancillary equipment |  |  |
| Radiated emission | Enclosure | applicable for BS type 1-C and BS type 1-H (Note) | not applicable | 8.2.1 | ITU-R SM.329 [24] |
| Radiated emission | Enclosure of ancillary equipment | not applicable] | applicable | 8.2.2 | CISPR 32 [11] |
| Conducted emission | DC power input/output port | applicable | applicable | 8.3 | CISPR 32 [11], - |
| Conducted emission | AC mains input/output port | applicable | applicable | 8.4 | CISPR 32 [11] |
| Conducted emission | Telecommunication port | applicable | applicable | 8.5 | CISPR 32 [11] |
| Harmonic current emissions | AC mains input port | applicable | applicable | 8.6 | IEC 61000-3-2 [13] or IEC 61000-3-12 [14] |
| Voltage fluctuations and flicker | AC mains input port | applicable | applicable | 8.7 | IEC 61000-3-3 [15] ] or IEC 61000-3-11 [16] |
| NOTE: Radiated emission requirements for BS type 1-O and BS type 2-O are described in subclause 8.2.1. |  |  |  |  |  |

### 9.2.2	Test method and level

The test method shall be in accordance with IEC 61000-4-3 [18]:

-	For transmitters, receivers and transceivers the following requirements shall apply:

-	The test level shall be 3 V/m amplitude modulated to a depth of 80 % by a sinusoidal audio signal of 1 kHz;

-	The stepped frequency increments shall be 1 % of the momentary frequency;

-	The test shall be performed over the frequency range 80 MHz  - 6000 MHz; with the exception of the exclusion band for receivers (see subclause 4.4);

-	Responses in stand-alone receivers or receivers which are part of transceivers occurring at discrete frequencies which are narrow band responses, shall be disregarded, see subclause 4.3;

-	The frequencies selected during the test shall be recorded in the test report.

-	In the range of angles except the operational range of angles of the BS Type 1-O and 2-O antenna (i.e. except for the half sphere around the DUT radiating direction as depicted on figure 9.2.2-1) and for the frequency range above 690 MHz (according to the test method in ETSI EN 301 489-50 [28]), the EMC RF electromagnetic field immunity requirement applies.

Figure 9.2.2-1: EMC RF electromagnetic field immunity requirement testing directions for 1-O and 2-O BS (horizontal plane depicted)

<End of modified section>

<Start of next modified section>

## 9.5	RF common mode (0.15 MHz - 80 MHz)

The test shall be performed on AC mains power input/output ports.

This test shall be performed on signal ports, telecommunication ports, control and DC power input/output ports, which may have cables longer than 3 m.

Where this test is not carried out on a port or any other ports because the manufacturer declares that it is not intended to be used with cables longer than stated above, a list of ports which were not tested shall be included in the test report.

This test shall be performed on a representative configuration of the equipment, the associated ancillary equipment, or representative configuration of the combination of radio and ancillary equipment.

NOTE:	This test can also be performed using the intrusive method, where appropriate, see IEC 61000-4-6 [21].

### 9.5.1	Definition

This test assesses the ability of radio equipment and ancillary equipment to operate as intended in the presence of a radio frequency electromagnetic disturbance.

### 9.5.2	Test method and level

The test method shall be in accordance with IEC 61000-4-6 [21]:

-	The test signal shall be amplitude modulated to a depth of 80 % by a sinusoidal audio signal of 1 kHz;

-	The stepped frequency increments shall be 50 kHz in the frequency range 150 kHz to 5 MHz and 1% frequency increment of the momentary frequency in the frequency range 5 MHz to 80 MHz;

-	The test level shall be severity level 2 as given in IEC 61000-4-6 [21] corresponding to 3 V rms, at a transfer impedance of 150 Ω;

-	The test shall be performed over the frequency range 150 kHz - 80 MHz;

-	The injection method to be used shall be selected according to the basic standard IEC 61000-4-6 [21];

-	Responses of stand-alone receivers or receivers which are part of transceivers occurring at discrete frequencies which are narrow band responses, shall be disregarded, see subclause 4.3;

-	The frequencies of the immunity test signal selected and used during the test shall be recorded in the test report.

<End of modified section>

<Start of next modified section>

### 9.6.2	Test method and level

The following requirements shall apply.

The test method shall be in accordance with IEC 61000-4-11 [22].

The test levels shall be:

-	Voltage dip: 0 % residual voltage for 0.5 cycle;

-	Voltage dip: 0 % residual voltage for 1 cycle;

-	Voltage dip: 70 % residual voltage for 25/30 cycles (at 50/60 Hz);

-	Voltage interruption: 0 % residual voltage for 250/300 cycles (at 50/60 Hz).

<End of modified section>

<Start of next modified section>

### 9.7.2	Test method and level

The test method shall be in accordance with IEC 61000-4-5 [20].

The requirements and evaluation of test results given in subclause 9.7.2.1 (telecommunication ports, outdoor cables), subclause 9.7.2.2 (telecommunication ports, indoor cables) and subclause 9.7.2.3 (AC power ports) shall apply, but no test shall be required where normal functioning cannot be achieved, because of the impact of the CDN on the EUT.

#### 9.7.2.1	Test method for telecommunication ports directly connected to outdoor cables

The test level for telecommunications ports, intended to be directly connected to the telecommunications network via outdoor cables, shall be 1 kV line to ground as given in IEC 61000-4-5 [20], however, in telecommunications centres 0,5 kV line to ground shall be used. In this case the total output impedance of the surge generator shall be in accordance with the basic standard IEC 61000-4-5 [20].

The test generator shall provide the 1.2/50 s pulse as defined in IEC 61000-4-5 [20].

#### 9.7.2.2	Test method for telecommunication ports connected to indoor cables

The test level for telecommunication ports, intended to be connected to indoor cables (longer than 10 m) shall be 0.5 kV line to ground. In this case the total output impedance of the surge generator shall be in accordance with the basic standard IEC 61000-4-5 [20].

The test generator shall provide the 1.2/50 s pulse as defined in IEC 61000-4-5 [20].

#### 9.7.2.3	Test method for AC power ports

The test level for AC power input ports shall be 2 kV line to ground, and 1 kV line to line, with the output impedance of the surge generator as given in IEC 61000-4-5 [20].

In telecommunication centres 1 kV line to ground and 0.5 kV line to line shall be used.

The test generator shall provide the 1.2/50 s pulse as defined in IEC 61000-4-5 [20].

<End of modified section>