3GPP TSG-RAN4 Meeting #91	R4-1905362

# Reno, USA, 13th May – 17th May, 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on interruption due to TCI state switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-4-25 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For RRC-based TCI state switching, the interruption due to RRC reconfiguration shall be introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the interruption requiremnt due to TCI state switching. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The interruption due to TCI state switching is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 8.2.1.2.8 and new section 8.2.2.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change #1>>

8.2.1.2.8		Interruptions due to active TCI state switch

When gNB (re)configures one or more active TCI state configuration(s) to a UE, if the UE is not capable of per-FR gap, an interruption on the active serving cells of up to X5 slot defined in Table 8.2.1.2.8-1, is allowed during the RRC reconfiguration procedure [2]. If the UE is capable of per-FR gap, an interruption on the active serving cells within the same FR as the reconfigured serving cell of up to X5 slot is allowed during the RRC reconfiguration procedure [2]. The interruption is for both uplink and downlink of the active serving cells.

Table 8.2.1.2.8-1: Interruption length X5 due to RRC-based TCI state switch

|  | NR Slot length (ms) | Interruption length X4 slot |  |
| --- | --- | --- | --- |
|  |  | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 2 | 3 |
| 2 | 0.25 | 5 |  |
| 3 | 0.125 | 9 |  |

<< End of Change #1>>

<< Start of Change #2>>

8.2.2.2.6		Interruptions due to active TCI state switch

When gNB (re)configures one or more active TCI state configuration(s) to a UE, if the UE is not capable of per-FR gap, an interruption on the active serving cells of up to XTCI slot defined in Table 8.2.2.2.6-1, is allowed during the RRC reconfiguration procedure [2]. If the UE is capable of per-FR gap, an interruption on the active serving cells within the same FR as the reconfigured serving cell of up to X slot is allowed during the RRC reconfiguration procedure [2]. The interruption is for both uplink and downlink of the active serving cells.

Table 8.2.2.2.6-1: Interruption length XTCI

|  | NR Slot length (ms) | Interruption length X (slots) |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |
| 2 | 0.25 | 3 |
| 3 | 0.125 | 5 |

<< End of Change #2>>