3GPP TSG-RAN4#91						     	              R4-1906837

Reno, US, 13-17 May 2019

Source:	Qualcomm Incorporated

Title:	Discussion on the feasibility of simultaneous transmission approach for 2AoA cases

Agenda item: 	10.1.3

Document for:	Approval

# 1. Introducion

In RAN4#90bis meeting, TDM transmission approach for 2AoA cases was agreed to use in FR2 RRM testing in Rel-15. For simultaneous transmission approach, the detail on how to control the SINR at baseband was captured in TR38.810 [1]. However, there is no consensus on the feasibility of simultaneous transmission approach to generate the testable side conditions.

In this paper, we provide the further analysis on antenna gain assumption and discuss the feasibility of generating the testable side condition. The company draft CR[2] is proposed to capture the SINR calculation table and antenna gain assumptions for simultaneous transmission approach into TR38810 for future study.

# 2. Discussion

# 2.1 Simultaneous transmission for Mode 1

From [3], we can give the SINR equation at baseband for simultaneous transmission approach as follows:

SINR1BB≈S1*G1*LossS2+NG2*Loss+N*G1*Loss=1S2S1+1SNR1*G2G1+1SNR1                                                 (1)

where SNR1 is the SNR from probe 1, and the key parameter affecting the SINR at baseband is G2/G1. Based on the discussion in the last meeting, we had the conclusion that the lower bound of SINR can be derived with the assumption of maximum antenna gains difference G2/G1. In TR38.810, it was agreed that for power class 3 UE in Band n260, the lower bound of G1/G2 can be roughly assessed to be below [-10] dB. To make the expression of antenna gains difference G2/G1 clearer in TR38.810, we further analyze this as follows.

We can assume the maximum gain difference is the case in which interference comes from peak direction meanwhile the signal comes from the direction with Nth (e.g. 50%) spherical coverage value. Therefore, we can have the following assumptions on maximum gain difference for fine/rough beam types:

- Fine beam: the maximum G2/G1 is X, where X is derived based on EIS spherical coverage requirement (i.e. difference between the peak EIS and Nth percentile EIS values. Value of N according to power class.

- Rough beam: the maximum G2/G1 is X+Z-Y, where value Z characterizes the antenna gain difference between “fine” and “rough” RX beams within Nth percentile EIS directions, and value Y characterizes the antenna gain difference between the fine and rough beams in the ‘fine’ RX beam peak direction. Based on the agreement in [1], Y=7dB for PC3.

  - Antenna gain for rough beam on beam peak direction: Antenna gain for fine beam peak direction-Y

  - Antenna gain for rough beam with non-beam peak direction: Antenna gain for fine peak direction-X-Z

  - Therefore, it can be derived that the maximum G2/G1 is X+Z-Y

With n260, if we assume X is 12.6dB, Y is 7dB and Z is [6]dB (need further discussion in other paper), then we can obtain the lower bound of G1/G2 is -11.6dB which aligns with the conclusion of [-10]dB in TR38.810.

Observation 1: The lower bound of G2/G1 is X+Z-Y which aligns with the conclusion of [-10]dB in TR38.810.

Proposal 1: The lower bound of feasible maximum SINR for simultaneous transmission can be calculated based on the maximum antenna gain difference of X and X+Z-Y for fine and rough beam respectively with Mode 1.

# 2.2 Simultaneous transmission for Mode 2

From [2], the SINR equation at baseband without artificial noise can be given by:

SINR1≈S1S2*G2/G1                                                                               (2)

Same as for Mode 1, we can have the similar proposal for Mode 2:

Proposal 2: The lower bound of feasible maximum SINR for simultaneous transmission can be calculated based on the maximum antenna gain difference of X and X+Z-Y for fine and rough beam respectively with Mode 2.

# 2.3 Testable side condition

In [4], we provided the preliminary analysis on the SINR range for 2AoAs. In this section, we further analyze the feasibility of generating the testable side condition with simlatanous transmission under the antena gain difference assumptions propsed in this paper. Based on the attached SINR calculator table, we can have the following lower bound of maximum feasible SINR with Mode 1 and Mode 2 respectively.

Table 1: SINR of probe 1 for 2AoAs case with Mode 1, X=12.6, Y=7, Z=[6]

|  | Beam type | Lower bound of maximum feasible SINR (dB) |
| --- | --- | --- |
| n260 | Fine | -3 |
|  | Rough | -8 |

Table 2: SINR of probe 1 for 2AoAs case with Mode 2, X=12.6, Y=7, Z=[6]

|  | Beam type | Lower bound of maximum feasible SINR (dB) |
| --- | --- | --- |
| n260 | Fine | -2 |
|  | Rough | -6 |

From the results in above tables, we can observe that the lower bound of maximum feasible SINR for Mode 1 is -8dB which does not satisfy the testable side condition of -6dB for PC3 n260.

Observation 2: The lower bound of maximum feasible SINR for Mode 1 is -8dB with the worst case assumption, which does not satisfy the testable side condition of -6dB for PC3 n260 with the current TE capability.

While for Mode 2, the lower bound of maximum feasible SINR is -6dB and therefore it can be used for 2AoA RRM testing. It should note that when the lower bound of maximum feasible SINR is calculated, we assume SNR for Probe 2 is -6dB. And in this case, the SINR for Probe 2 can be derived by the similar equation as (2).

SINR2≈S2S1*G1/G2                                                                               (3)

That is since the lower bound of SINR1 is -6dB for Mode 2, and with the assumption of X+Z-Y=11.6dB (X=12.6, Y=7, Z= [6]), we can derive that SINR2 is about 5.6dB (-6dB+11.6dB). Therefore, we have the following proposal:

Proposal 3: At least the simultaneous transmission approach with Mode 2 can be used for 2AoA RRM testing cases.

# 3 Conclusion

In this paper, we provide the details on setup and parameters for 2AoA RRM testing. We have the following observations and proposals:

Observation 1: The lower bound of G2/G1 is X+Z-Y which aligns with the conclusion of [-10]dB in TR38.810.

Proposal 1: The lower bound of feasible maximum SINR for simultaneous transmission can be calculated based on the maximum antenna gain difference of X and X+Z-Y for fine and rough beam respectively with Mode 1.

Proposal 2: The lower bound of feasible maximum SINR for simultaneous transmission can be calculated based on the maximum antenna gain difference of X and X+Z-Y for fine and rough beam respectively with Mode 2.

Observation 2: The lower bound of maximum feasible SINR for Mode 1 is -8dB with the worst case assumption, which does not satisfy the testable side condition of -6dB for PC3 n260 with the current TE capability.

Proposal 3: At least the simultaneous transmission approach with Mode 2 can be used for 2AoA RRM testing cases.

# 4 References

- R4-1904786	, “Draft CR to TR 38.810 on FR2 RRM test methods”, Qualcomm Incorporated, April. 2019.

- R4-1906838, “Draft CR to TR 38.810 on the feasibility of simultaneous transmission approach”, Qualcomm Incorporated, May. 2019.

- R4-1904089, “Setup and parameters for 2AoA RRM testing”, Qualcomm Incorporated, Feb. 2019.

- R4-1900753, “Noc level definition and SINR range for 2AoA RRM testing”, Qualcomm Incorporated, Feb. 2019.