3GPP TSG-RAN WG4 Meeting #92                                         R4-1908075

Ljubljana,SI, 26th – 30th May, 2019

Agenda Item:   9.5.2

Source:        Samsung

Title:           Co-existence simulation result for layout 1

Document for:  Discussion

# Introduction

In RAN4#91 meeting simulation assumption was discussed in AH meeting [1] and agreed for IAB co-existence study in [2]. This contribution provides assessment results and observation based on those results for layout 1.

# Simulation assumptions

In layout 1 it is assumed with half-duplexer TDM restriction and no multiple hops considered. In our simulator, IAB node is dropped randomly always in the cycle with center of 40m and 20m radius. In each cell of donor gNB only one IAB node is active for TRX. Furthermore, the only TRX between donor gNB and IAB-MT is the target of evaluation.

For the minimum distance assumed between Macro BS and UE, Micro BS and UE, the tentative agreement in last meeting is 10meters for both. However, the traditional assumptions would be 35m for Macro BS-to-UE, and 5 meter between Micro BS-to-UE. Even though this may not impact significantly on results, it would be good to align them with existing assumption if no special reason to change the distance.

The assumptions for ACLR and ACS are not captured in agreed WF. What we used in our simulator is shared in tables below:

Table 1:  ACIR for FR1 simulation

| Case | TX ACLR(dB) | RX ACS(dB) |
| --- | --- | --- |
| gNB -> IAB-MT IAB-DU -> IAB-MT | 45 | 33-45 |
| IAB-MT -> gNB IAM-MT -> IAB-DU | 30-45 | 45 |
| IAB-DU -> gNB gNB -> IAB-DU | 45 | 45 |
| IAB-DU -> UE | 45 | 33 |
| UE -> IAB-DU | 30 | 45 |

Table 2: ACIR for FR2 simulation

| Case | TX ACLR(dB) | RX ACS(dB) |
| --- | --- | --- |
| gNB -> IAB-MT IAB-DU -> IAB-MT | 28 | 23 or 23.5 |
| IAB-MT -> gNB IAM-MT -> IAB-DU | In the range of 17-28 | 23.5 |
| IAB-DU -> gNB gNB -> IAB-DU | 28 | 23.5 |
| IAB-DU -> UE | 28 | 23 |
| UE -> IAB-DU | 17 | 23.5 |

In initial simulation we only consider two cases for layout 1 simulation for both FR1 and FR2 as case 1 and case 2. Please note the system on adjacent channel is assumed without IAB deployed.

- Case 1: IAB-MT UL TX for its ACLR

  - Baseline interference for victim system: co-channel interference with the same system, adjacent channel interference without IAB

  - New interference case: co-channel interference with the same system, IAB-MT TX interference on adjacent channel .

- Case 2: IAB-MT DL RX for its ACS

  - Baseline interference: co-channel interference from gNB TX

  - New interference case: co-channel interference from gNB TX, adjacent channel interference from aggressor system

Figure 1: case1                           Figure 2: case 2

All the other simulations are configured according to agreed WF.

# Simulation results

Table 1： Simulation result for FR1 case 1

| MT UL Power control | IAB active ratio | ACLR and ACS assumption | Average user throughput (Kbps) | Performance gain(%) | 5-tile edge user throughput (Kbps) | Performance gain(%) |
| --- | --- | --- | --- | --- | --- | --- |
| ON | Baseline Without IAB | UE ACLR:30dB NB ACS: 45dB | 178316.610 |  | 124311.810 |  |
|  | 50% | IAB-ACLR:45 | 178508.463 | 0.1 | 125261.390 | 0.76 |
|  |  | IAB-ACLR:40dB | 178449.322 | 0.0 | 125202.030 | 0.71 |
|  |  | IAB-ACLR:35dB | 178286.383 | -0.01 | 124960.680 | 0.52 |
|  |  | IAB-ACLR:30 dB | 177875.628 | -0.2 | 123979.100 | -0.27 |
|  | 100% | IAB-ACLR:45 dB | 178724.6640 | 0.23 | 125144.100 | 0.67 |
|  |  | IAB-ACLR:40 dB | 178609.036 | 0.16 | 125068.910 | 0.61 |
|  |  | IAB-ACLR:35 dB | 178287.981 | -0.02 | 124338.870 | 0.02 |
|  |  | IAB-ACLR:30 | 177476.038 | -0.47 | 122574.490 | -1.4 |
| OFF | 15% | IAB-ACLR:45 | 147828.481 | -17 | 8411.190 | -93 |
|  | 100% | IAB-ACLR:45 | 42724.596 | -76 | 0 | -100 |

Table 2： Simulation result for FR1 case 2

| ACLR and ACS assumption | Average user throughput (Kbps) | Performance gain (%) | 5-tile edge user throughput (Kbps) | Performance gain(%) |
| --- | --- | --- | --- | --- |
|  | Average user throughput (Kbps) |  | 5-tile edge user throughput (Kbps) |  |
| Baseline Without adjacent interference | 497210.976 |  | 307731.350 |  |
| gNB-ACLR:45dB IAB-ACS:45dB | 497102.447 | -0.02 | 307724.500 | -0.002 |
| gNB-ACLR:45dB IAB-ACS:40dB | 496984.062 | -0.05 | 307691.050 | -0.01 |
| gNB-ACLR:45dB IAB-ACS:33dB | 496267.414 | -0.19 | 307582.560 | -0.05 |

Table 3: Simulation result for FR2 case 1

| MT UL Power control | IAB active ratio | ACLR and ACS assumption | Average user throughput (Kbps) | Performance gain(%) | 5-tile edge user throughput (Kbps) | Performance gain(%) |
| --- | --- | --- | --- | --- | --- | --- |
| PC ON | Baseline Without IAB | UE-ACLR:17 NB-ACS:23 | 384481.189 |  | -86% |  |
|  | 50% | IAB-ACLR:28 | 384987.850 | 0.13 | 315356.890 | 0.88 |
|  |  | IAB-ACLR:24 | 384332.881 | -0.04 | 312180.210 | -0.14 |
|  |  | IAB-ACLR:20 | 382981.938 | -0.39 | 304495.700 | -2.6 |
|  |  | IAB-ACLR:17 | 381242.956 | -0.84 | 293709.770 | -6.0 |
|  |  | IAB-ACLR:17 | 381242.956 | -0.84 | 293709.770 | -6.0 |
|  | 100% | IAB-ACLR:28 | 384656.711 | 0.05 | 312080.440 | -0.17 |
|  |  | IAB-ACLR:24 | 383341.828 | -0.3 | 304864.010 | -2.48 |
|  |  | IAB-ACLR:20 | 380633.219 | -1 | 289547.780 | -7.38 |
|  |  | IAB-ACLR:17 | 377156.531 | -1.9 | 270657.450 | -13.42 |
| PC OFF | 15% | IAB-ACLR:28 | 338502.819 | -12 | 40755.780 | -87 |
|  | 100% | IAB-ACLR:28 | 150927.608 | -61 | 0 | -100 |

Table 4: Simulation result for FR2 case 2

| ACLR and ACS assumption | Average user throughput (Kbps) | Performance gain (%) | 5-tile edge user throughput (Kbps) | Performance gain (%) |
| --- | --- | --- | --- | --- |
| Baseline (Without adjacent interference) | 1103569.946 | - | 740795.830 | - |
| NB-ACLR:45dB IAB-ACS:23dB | 1095643.812 | -0.72 | 738704.620 | -0.28 |

# Summary

According to simulation results presented in this contribution there are several observations as following:

For IAB-MT transmission on UL, UL power control shall be supported otherwise there would be destructive impact on reception of victim gNB on adjacent channel.

With UL power control and beamforming assumed for IAB-MT, performance degradation of victim system is not sensitive to IAB-MT ACLR and active ratio of IAB. For FR1 IAB-MT ACLR can be in the range of 30 to 45 dB range. And for FR2 IAB-MT is suggested to align with gNB ACLR to ensure the cell edge performance of victim system.

Based on beamforming of IAB-MT, the performance degradation of IAB MT reception is ignorable with interference from NR operation on adjacent channel.

# Reference

[1] R4-1907824 AH meeting minutes for IAB ad-hoc

[2] R4-1907825 WF on simulation assumption for IAB co-existence study