3GPP TSG- RAN WG4 Meeting #92	R4-1908182

# Ljubljana, Slovenia, August 26th – 30th, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draft | rev |  | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification CR on BWP switch delay requirements (8.6) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - UE needs a usable TCI state after BWP switch. Otherwise, UE does not have any TCI-state to be used, even for receving MAC-CE for TCI-state activation. - It is ambiguous that whether the current delay requirement applies to the case where DCI/timer-based BWP switch is performed only on a single CC or on multiple CCs at the same time. Since the requirement for BWP switch for multiple CC will be discussed in R16. It is reasonable to consider the current requirement is only for DCI/timer-based BWP switch on a single CC - RRC-based BWP switch include 2 cases. One is to change the active BWP, and the other is to change the parameter in the active BWP. Current wording may lead to the mis-understanding that the requirement only covers the 1st case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that if UE does not have the information on the indicated TCI-state to decode PDCCH and activated TCI-state to decode PDSCH in the new BWP, UE re-uses those TCI-states in old BWP till new MAC CE is received to update the TCI-states for PDCCH and PDSCH - Clarify the current requirement applies only to the case that DCI/timer-based BWP switch happens in only one single CC. - Clarify RRC-based BWP swtich covers 2 cases. One is to change the active BWP, and the other is to change the parameter in the active BWP |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirement may be infeasible if constraint on TCI-state is not added. There will be ambiguity in the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of 1st change>

## 8.6	Active BWP switch delay

### 8.6.1	Introduction

The requirements in this section apply for a UE configured with more than one BWP on PCell or any activated SCell in standalone NR or NE-DC, PCell, PSCell or any activated SCell in MCG or SCG in NR-DC, or PSCell or any activated SCell in SCG in EN-DC. UE shall complete the switch of active DL and/or UL BWP within the delay defined in this section.

### 8.6.2	DCI and timer based BWP switch delay

The requirements in this section only apply to the case that the BWP switch is performed on a single CC.

For DCI-based BWP switch, after the UE receives BWP switching request at DL slot n on a serving cell, UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch on the first DL or UL slot occurs right after the beginning of DL slot n+ TBWPswitchDelay.

The UE is not required to transmit UL signals or receive DL signals during time duration TBWPswitchDelay on the cell where DCI-based BWP switch occurs. The UE is not required to follow the requirements defined in this section when performing a DCI-based BWP switch between the BWPs in disjoint channel bandwidths or in partially overlapping channel bandwidths.

For timer-based BWP switch, the UE shall start BWP switch at DL slot n, where n is the beginning of a DL subframe (FR1) or DL half-subframe (FR2) immediately after a BWP-inactivity timer bwp-InactivityTimer [2] expires on a serving cell, and the UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch on the first DL or UL slot occurs right after the beginning of DL slot n+ TBWPswitchDelay.

The UE is not required to transmit UL signals or receive DL signals after bwp-InactivityTimer [2] expires on the cell where timer-based BWP switch occurs.

Depending on UE capability bwp-SwitchingDelay [2], UE shall finish BWP switch within the time duration TBWPswitchDelay defined in Table 8.6.2-1.

Table 8.6.2-1: BWP switch delay

|  | NR Slot length (ms) | BWP switch delay TBWPswitchDelay (slots) |  |
| --- | --- | --- | --- |
|  |  | Type 1Note 1 | Type 2Note 1 |
| 0 | 1 | 1 | 3 |
| 1 | 0.5 | 2 | 5 |
| 2 | 0.25 | 3 | 9 |
| 3 | 0.125 | 6 | 18 |
| Note 1: Depends on UE capability. Note 2: If the BWP switch involves changing of SCS, the BWP switch delay is determined by the larger one between the SCS before BWP switch and the SCS after BWP switch. |  |  |  |

If UE does not have the information on the indicated TCI-state to decode PDCCH and activated TCI-state to decode PDSCH in the new BWP, UE re-uses those TCI-states in old BWP before new MAC CE is received to update the TCI-states for PDCCH and PDSCH.

### 8.6.3	RRC based BWP switch delay

For RRC-based BWP switch, after the UE receives RRC reconfiguration involving active BWP switching or parameter change of its active BWP, UE shall be able to receive PDSCH/PDCCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch occurs on the first DL or UL slot right after the beginning of DL slot n+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length, where

DL slot n is the last slot containing the RRC command, and

TRRCprocessingDelay is the length of the RRC procedure delay in  millisecond as defined in clause 12 in TS 38.331 [2], and

TBWPswitchDelayRRC=[6]ms is the time used by the UE to perform BWP switch.

The UE is not required to transmit UL signals or receive DL signals during the time defined by TRRCprocessingDelay+TBWPswitchDelayRRC on the cell where RRC-based BWP switch occurs.

### <End of 1st change>