3GPP TSG-RAN WG4 Meeting #92		R4-1908197

# Ljubljana, SI, 26th– 30th Aug, 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR |  | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on Corrections to CSI reporting test case parameters in FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  |  | Date: |  | 2019-08-16 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Configuration for aperiodic resources and report is incomplete |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected and updated parameters for aperiodic resources and reporting |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test configuration is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.2.2.1 8.3.2.2.1 8.4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS/TR 38.521-4 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of Change

##### 8.2.2.2.2	CQI reporting under fading conditions

<UNCHANGED PART SKIPPED>

Table 8.2.2.2.2.1-1 Test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 100 |  |  |  |
| Subcarrier spacing |  |  | kHz | 120 |  |  |  |
| Duplex Mode |  |  |  | TDD |  |  |  |
| TDD Slot Configuration |  |  |  | FR2.120-2 Annex A.1.3 |  |  |  |
| SNRBB |  |  | dB | 6 | 7 | 12 | 13 |
| Propagation channel |  |  |  | TDLA30-35 |  |  |  |
| Antenna configuration |  |  |  | 2×2 ULA High |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Aperiodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 8 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |  |  |  |
|  | CSI-RS interval and offset |  | slot | Not configured |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Aperiodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 2 |  |  |  |
|  | CDM Type |  |  | fd-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 6 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig interval and offset |  | slot | Not configured |  |  |  |
|  | aperiodicTriggeringOffset |  |  | 0 |  |  |  |
| CSI-IM configuration | CSI-IM RE pattern |  |  | 1 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (8, 13) |  |  |  |
|  | CSI-IM timeConfig interval and offset |  | slot | 8/1 |  |  |  |
| ReportConfigType |  |  |  | Aperiodic |  |  |  |
| CQI-table |  |  |  | Table 1 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 8 |  |  |  |
| csi-ReportingBand |  |  |  | 111111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | Not configured |  |  |  |
| ZP CSI-RS trigger |  |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |  |  |
| CSI request |  |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |  |  |
| reportTriggerSize |  |  |  | 1 |  |  |  |
| CSI-AperiodicTriggerStateList |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  | 000001 |  |  |  |
|  |  | RI Restriction |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  | PUSCH |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 1.375 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  | As specified in Table A.4-1, TBS.1-1 |  |  |  |

<UNCHANGED PART SKIPPED>

##### 8.3.2.2.1	Single PMI with 2TX TypeI-SinglePanel Codebook

For the parameters specified in Table 8.3.2.2.1-1, and using the downlink physical channels specified in Annex C.5.1, the minimum requirements are specified in Table 8.3.2.2.1-2.

Table 8.3.2.2.1-1: Test parameters (single layer)

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 100 | 100 |
| Subcarrier spacing |  | kHz | 120 | 120 |
| TDD DL-UL configuration |  |  | FR2.120-2 as specified in Annex A.1.3 | FR2.120-1 as specified in Annex A.1.3 |
| Propagation channel |  |  | TDLA30-35 | TDLA30-35 |
| Antenna configuration |  |  | 2 x 2 ULA Low | 2 x 2 ULA Low |
| Beamforming Model |  |  | As specified in Annex B.4.1 | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 | 4 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 4, (8,-) | Row 4, (8,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (13,-) | (13,-) |
|  | CSI-RS interval and offset | slot | Not configured | Not configured |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 2 | 2 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 3, (6,-) | Row 3, (6,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (13,-) | (13,-) |
|  | CSI-RS interval and offset | slot | Not configured | Not configured |
|  | aperiodicTriggeringOffset |  | 0 | 0 |
| CSI-IM configuration | CSI-IM RE pattern |  | Pattern 1 | Pattern 1 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (8,13) | (8,13) |
|  | CSI-IM timeConfig interval and offset | slot | 8/1 | 5/1 |
| ReportConfigType |  |  | Aperiodic | Aperiodic |
| CQI-table |  |  | Table 1 | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured | Not configured |
| cqi-FormatIndicator |  |  | Wideband | Wideband |
| pmi-FormatIndicator |  |  | Wideband | Wideband |
| Sub-band Size |  | RB | 8 | 8 |
| csi-ReportingBand |  |  | 111111111 | 111111111 |
| CSI-Report interval and offset |  | slot | 8/1 | 5/1 |
| ZP CSI-RS trigger |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel |
|  | Codebook Mode |  | 1 | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A | N/A |
|  | CodebookSubsetRestriction |  | 001111 | 001111 |
|  | RI Restriction |  | N/A | N/A |
| Physical channel for CSI report |  |  | PUSCH | PUSCH |
| CQI/RI/PMI delay |  | ms | 1.375 | 1.75 |
| Maximum number of HARQ transmission |  |  | 4 | 4 |
| Measurement channel |  |  | R.PDSCH.5-8.1 TDD | R.PDSCH.5-7.1 TDD |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (0.125 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4)], this reported PMI cannot be applied at the eNB downlink before slot#(n+4)]. Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |  |

<UNCHANGED PART SKIPPED>

## 8.4	Reporting of Rank Indicator (RI)

<UNCHANGED PART SKIPPED>

Table 8.4.2.2-1: RI Test (TDD)

| Parameter |  |  |  |  | Unit |  | Test 1 |  | Test 2 |  | Test 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  |  | MHz |  | 100 |  | 100 |  | 100 |
| Subcarrier spacing |  |  |  |  | kHz |  | 120 |  | 120 |  | 120 |
| Duplex Mode |  |  |  |  |  |  | TDD |  | TDD |  | TDD |
| TDD Slot Configuration |  |  |  |  |  |  | FR1.120-2 |  | FR1.120-2 |  | FR1.120-2 |
| SNR |  |  |  |  | dB |  | 0 |  | 16 |  | 16 |
| Propagation channel |  |  |  |  |  |  | TDLA30-35 |  | TDLA30-35 |  | TDLA30-35 |
| Antenna configuration |  |  |  |  |  |  | ULA Low 2x2 |  | ULA Low 2x2 |  | XP High 2x2 |
| Beamforming Model |  |  |  |  |  |  | As defined in Annex B.4.1 |  | As defined in Annex B.4.1 |  | As defined in Annex B.4.1 |
| ZP CSI-RS configuration |  | CSI-RS resource Type |  |  |  |  | Aperiodic |  | Aperiodic |  | Aperiodic |
|  |  | Number of CSI-RS ports (X) |  |  |  |  | 4 |  | 4 |  | 4 |
|  |  | CDM Type |  |  |  |  | FD-CDM2 |  | FD-CDM2 |  | FD-CDM2 |
|  |  | Density (ρ) |  |  |  |  | 1 |  | 1 |  | 1 |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  |  | Row 4, (8,-) |  | Row 4, (8,-) |  | Row 4, (8,-) |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  |  |  | (13,-) |  | (13,-) |  | (13,-) |
|  |  | CSI-RS interval and offset |  |  | slot |  | not configured |  | not configured |  | not configured |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Aperiodic | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  |  | 2 | 2 | 2 |
|  | CDM Type |  |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 3 (6,-) | Row 3 (6,-) | Row 3 (6,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | (13,-) | (13,-) | (13,-) |
|  | NZP CSI-RS-timeConfig interval and offset |  | slot | not configured | not configured | not configured |
|  | aperiodicTriggeringOffset |  |  | 0 | 0 | 0 |
| CSI-IM configuration |  | CSI-IM RE pattern |  |  |  |  | Pattern 1 |  | Pattern 1 |  | Pattern 1 |
|  |  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  |  | (8,13) |  | (8,13) |  | (8,13) |
|  |  | CSI-IM timeConfig interval and offset |  |  | slot |  | 8/1 |  | 8/1 |  | 8/1 |
| ReportConfigType |  |  |  |  |  |  | Aperiodic |  | Aperiodic |  | Aperiodic |
| CQI-table |  |  |  |  |  |  | Table 1 |  | Table 1 |  | Table 1 |
| reportQuantity |  |  |  |  |  |  | cri-RI-PMI-CQI |  | cri-RI-PMI-CQI |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  |  |  |  | not configured |  | not configured |  | not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  |  |  |  | not configured |  | not configured |  | not configured |
| cqi-FormatIndicator |  |  |  |  |  |  | Wideband |  | Wideband |  | Wideband |
| pmi-FormatIndicator |  |  |  |  |  |  | Wideband |  | Wideband |  | Wideband |
| Sub-band Size |  |  |  |  | RB |  | 8 |  | 8 |  | 8 |
| csi-ReportingBand |  |  |  |  |  |  | 111111111 |  | 111111111] |  | 111111111 |
| CSI-Report interval and offset |  |  |  |  | slot |  | not configured |  | not configured |  | not configured |
| ZP CSI-RS trigger |  |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| CSI request |  |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  |  | 1 | 1 | 1 |
| CSI-AperiodicTriggerStateList |  |  |  |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration |  |  | Codebook Type |  |  |  | typeI-SinglePanel |  | typeI-SinglePanel |  | typeI-SinglePanel |
|  |  |  | Codebook Mode |  |  |  | 1 |  | 1 |  | 1 |
|  |  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  |  | N/A |  | N/A |  | N/A |
|  |  |  | CodebookSubsetRestriction |  |  |  | 010000 for fixed rank 2, 010011 for following rank |  | 000011 for fixed rank 1, 010011 for following rank |  | 000011 for fixed rank 1, 010011 for following rank |
|  |  |  | RI Restriction |  |  |  | N/A |  | N/A |  | N/A |
| Physical channel for CSI report |  |  |  |  |  |  | PUSCH |  | PUSCH |  | PUSCH |
| CQI/RI/PMI delay |  |  |  |  | ms |  | 1.375 |  | 1.375 |  | 1.375 |
| Maximum number of HARQ transmission |  |  |  |  |  |  | 1 |  | 1 |  | 1 |
| RI Configuration |  |  |  |  |  |  | Fixed RI = 2 and follow RI |  | Fixed RI = 1 and follow RI |  | Fixed RI = 1 and follow RI |

End of Change