3GPP TSG-RAN WG4 Meeting #92	R4-1908366

Ljubljana, Slovenia, 26th – 30th Aug., 2019

Agenda item:		9.6.3

Source:	MediaTek Inc.

Title:	Discussion on IDLE mode CA measurement

Document for:		Discussion

# 1	Introduction

In Xi'an #105-Bis meeting [1], RAN2 agreed to introduce NR IDLE/INACTIVE CA measurement. The related agreements are listed as follows:

Agreements 1: NR early measurements can be configured in both NR RRCRelease message and NR system information. FFS: Whether there are differences in the configuration that can be provided by RRCRelease and SI. 2: Introduce some indication about the cell's early measurement support in NR system information. 3: To control the duration of UE performing both IDLE and INACTIVE measurements, a single validity timer (similar to measIdleDuration in LTE euCA) is mandatory indicated only in NR RRCRelease message, i.e. not included in NR SIB. 4: For both IDLE and INACTIVE early measurements, the following IEs can be optionally configured per NR frequency in both NR RRCRelease message and NR SIB: - A list of frequencies and optionally cells (similar to measCellList in LTE euCA) the UE is required to perform early measurements. - A cell quality threshold (similar to qualityThreshold in LTE euCA) the UE is required to report the measurement results only for the cells which met the configured thresholds. FFS: A validity Area (similar to validityArea in LTE euCA) to indicate the list of cells within which UE is required to perform early measurements. If the UE reselects to a cell outside this list, the early measurements are no longer required (same as timer expiry). o If it is absent, the UE will not have area limitation of early measurements. For SSB based measurements: 5: For both IDLE and INACTIVE early measurements, SSB frequencies to be measured can be located out of sync raster 6: For both IDLE and INACTIVE early measurements, RSRP and RSRQ can be configured as cell and beam measurement quantity. 7: For both IDLE and INACTIVE early measurements, the configuration parameters provided per SSB frequency follow the same principles as those provided in SIB2/4 for the purposes of Idle/Inactive mobility. (Details differences can be discussed at stage 3 level) 8: As LTE euCA, cell / beam SINR is not introduced as measurement quantity in NR early measurement configuration in Rel-16. For SSB based beam level measurement configurations: 9: The UE is required to report the beam with the highest measurement quantity FFS: Whether additional beams can be reported. 10: For both IDLE and INACTIVE early measurements, the UE can be configured with one of the 3 beam reporting types 1) No beam reporting; 2) Only beam identifier 3) Both beam identifier and quantity FFS: Whether to support CSI-RS based NR early measurements 11: LTE UE in IDLE mode, IDLE with suspended, and INACTIVE can be configured with NR early measurements to support fast setup of (NG)EN-DC (i.e. euCA is extended to support NR measurements). Details are FFS

In this paper, we discuss the related RAN4 requirement for NR IDLE/INACTIVE CA measurement.

# 2	Discussion

In current RAN4 spec, TS36.133 Section 4.9 has specified the IDLE mode CA measurement for LTE SA mode UE during Rel-15 euCA WI. For UE which supports ca-IdleModeMeasurements, UE shall perform measurement on the configured overlapping and non-overlapping inter-frequency carriers for IDLE mode measurement reporting shown as Table 1.

Table 1: Measurements of inter-frequency CA candidate cells

| Serving cell signal quality | Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ |  | Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ |  |
| --- | --- | --- | --- | --- |
| Overlapping with carriers for re-selection | Overlapping | Non-overlapping | Overlapping | Non-overlapping |
| # of early measurement reporting carriers | 3 (0 or 1 inter-frequency carrier may be a non-overlapping carrier) |  | 1 (0 or 1 inter-frequency carrier may be a non-overlapping carrier) |  |
| Measurement delay | 4.2.2.4 | N/A | N/A | N/A |
| Measurement accuracy | 9.1.3B.2 & 9.1.6B.2 | 9.1.3B.3 & 9.1.6B.3 | 9.1.3B.3 & 9.1.6B.3 | 9.1.3B.3 & 9.1.6B.3 |

Where the definition of overlapping and non-overlapping carriers are:

- Overlapping: a carrier configured by higher layer for early measurement reporting and inter-frequency mobility measurements.

- Non-overlapping: a carrier configured by higher layer for early measurement reporting while not configured for inter-frequency mobility measurements.

Besides, TS36.133 Section 4.2.2.9 specifies that UE shall be capable of monitoring a total of at least 8 carrier frequency layers in RRC_IDLE state meaning that, according the current spec, the overall measurement capability for UE who supports IDLE mode CA measurement may be extended by 1 carriers (number of non-overlapping carrier), compare to the measurement capability specified in IDLE mode cell re-selection. Here is the example, when UE already monitors 8 carrier frequency layers in E-UTRA measurements RRC_IDLE state, and Network configures only non-overlapping carriers for early measurement reporting. The overall number of monitoring carriers may become at least 8+1 =9 carriers when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ.

Observation 1: In LTE euCA, Comparing to the measurement capability specified in IDLE mode cell re-selection, for UE which supports IDLE mode CA measurement, the overall measurement capability may be extended for 1 carriers.

In our understanding, UE power consumption is very important in IDLE mode. Even with IDLE mode CA measurement, we still prefer not to increase the baseline on UE IDLE measurement capability in Rel-15. Therefore, we suggest

Proposal 1: In NR, supporting IDLE/INACTIVE mode CA measurement should not extend the measurement capability defined in Rel-15.

We then discuss the restrictions on the early measurement reporting, i.e., 0 or 1 inter-frequency carrier may be a non-overlapping carrier. Assuming that Srxlev ≤ SnonIntraSearchP, and UE is monitoring 8 carriers and the RSRP for these carriers are as shown in Table 2.

Table 2: Example of IDLE/INACTIVE mode CA measurement

|  | Non-overlapping carriers | Overlapping carriers |
| --- | --- | --- |
| RSRP Ranking | F1: -82dBm | F4: -90dBm |
|  | F2: -86dBm | F5: -91dBm |
|  | F3: -89dBm | F6: -93dBm |
|  |  | F7: -95dBm |
|  |  | F8: -97dBm |

Since UE can only report 0 or 1 carrier from the set of non-overlapping carrier, UE may report:

- “1” non-overlapping carrier, e.g., UE reports F1, F4, F5.

  - F2 and F3 with higher RSRPs than all overlapping carriers are not reported.

- “0” non-overlapping carrier, e.g., UE reports F4, F5, F6.

  - F1, F2 and F3 with higher RSRPs than all overlapping carriers are not reported.

In our understanding, this is strange that UE cannot report the carrier with better RSRP to network. Therefore, we suggest to remove such restriction and propose

Proposal 2: In NR, RAN4 to remove the restriction on the number of overlapping and non-overlapping carriers in early measurement reporting and only specify the total number of carriers.

We also notice that based on the current spec, UE monitors more carriers for IDLE mode CA measurement when one of the serving cell signal quality is no larger than a certain threshold, i.e., Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ.

Observation 2: UE monitors more carriers for IDLE mode CA measurement when one of the serving cell signal quality is no larger than a certain threshold.

However, in this case, UE might prefer to allocate more resources (time and power) for cell re-selection. On the other hand, when serving cell signal quality is good, i.e., Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, UE has less intention to perform frequent measurement for cell re-selection. In this case, UE can spend more time and power on candidate CA carriers. So we suggest

Proposal 3: RAN4 to discuss whether to follow the LTE IDLE mode CA measurement rules, i.e., UE monitors more carriers when one of the serving cell signal quality is no larger than a certain threshold and measures less carriers for CA measurement otherwise.

# 4	Summary

In this contribution, we have the following observations:

Observation 1: In LTE euCA, Comparing to the measurement capability specified in IDLE mode cell re-selection, for UE which supports IDLE mode CA measurement, the overall measurement capability may be extended for 1 carriers.

Observation 2: UE monitors more carriers for IDLE mode CA measurement when one of the serving cell signal quality is no larger than a certain threshold.

And we propose

Proposal 1: In NR, supporting IDLE/INACTIVE mode CA measurement should not extend the measurement capability defined in Rel-15.

Proposal 2: In NR, RAN4 to remove the restriction on the number of overlapping and non-overlapping carriers in early measurement reporting and only specify the total number of carriers.

Proposal 3: RAN4 to discuss whether to follow the LTE IDLE mode CA measurement rules, i.e., UE monitors more carriers when one of the serving cell signal quality is no larger than a certain threshold and measures less carriers for CA measurement otherwise.

# Reference

- R2-1905501, Report of 3GPP TSG RAN2#105bis meeting, Xi'an, China, April, 2019.