3GPP TSG-WG RAN4 Meeting #92	R4-1908981

# Ljubljana, Slovenia, 26th – 30th August, 2019

Source: 	Nokia, Nokia Shanghai Bell, [Verizon]

Title: 					TP for TR 37.716-21-11 Introduction of DC_5A-66A_n66A_BCS0

Agenda Item: 			9.4.2 [DC_R16_1BLTE_1BNR_3DL2UL-Core]

Document for:	Approval

# Introduction

This TP introduces two-band EN-DC configurations of DC_5A-66A_n66A_BCS0 to TR 37.716-21-11.

**************************** Start of changes ********************************************

## 5.1.X	DC_2A-5A_n66A

### 5.1.X.1	Operating bands for DC

Table 5.1.X.1-1: Band combinations EN-DC (two bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_5-66_n66 | CA_5-66 | n66 | No |

### 5.1.X.2	Configuration for DC

Table 5.1.X.2-1: Inter-band EN-DC configurations (two bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_5-66_n66 | DC_5A_n66A | CA_5A-66A | n66A |

### 5.1.X.3	Coexistence studies

Based on co-existence studies of DC_5A_n66A in TR 37.863-01-01, the second order IMD may fall into band 5/n5.

### 5.1.X.4	∆TIB and ∆RIB values

For DC_5-66_n66, the TIB,c and RIB,c values are derived from CA_5-66 in TS 36.101.

Table 5.X.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-66_n66 | 5 | 0.3 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |

Table 5.X.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_5-66_n66 | 5 | 0 |
|  | 66 | 0 |
|  | n66 | 0 |

### 5.1.X.5	REFSENS requirements

The MSD due to IMD products are already specifed for 2DL2UL DC combos (DC_5A_n66A) in TS 38.101-3, where band 5 MSD is specified. To avoid the specification complexity, band n5 MSD is not specified as the same MSD is applied if band 5 carrier frequency is replaced by band n5 carrier frequency.

**************************** End of changes ********************************************