3GPP TSG-RAN WG4 Meeting #92	R4-1909154

Ljubljana, Slovenia, 26 – 30 August 2019

Agenda Item:	8.12.3.3

Source:	Ericsson

Title:	Discussion on channel quality reporting for NB-IoT

Document for:	Discussion

# 1	Introduction

RAN plenary #83 revised the WID for Rel-16 NB-IoT [1] and added to support DL channel quality support in CONNECTED as well as MSG3-based quality report for non-anchor carrier.

Improved multi-carrier operation: - Specify support of Msg3 quality reporting for non-anchor access [RAN1, RAN2, RAN4] - Specify support for quality reporting in connected mode for anchor and non-anchor carriers. The quality report is not carried in the physical layer. [RAN1, RAN2, RAN4]. - Specify signalling to indicate on a non-anchor carrier for paging a set of subframes which will contain NRS even when no paging NPDCCH is transmitted [RAN1, RAN2, RAN4]

RAN4#91 discussed the channel quality reporting in MSG3 and agreed with the way forward as follows [2]:

- Msg3 based channel quality reporting - FFS whether T1 defined in R14 applies for MSG3 DL quality report in non-anchor carriers - RAN4 shall take into account further RAN1/RAN2 progress on reporting message type details

We discuss further on the channel quality reporting for NB-IoT based on the latest agreements by RAN1/RAN2 as listed in Appendix.

# 2	Discussion

## 2.1	MSG3-based channel quality report for non-anchor

Figure 1	DL channel quality reporting on non-anchor for NB-IOT

According to RAN1 agreements, the difference from the MSG3-based DL channel quality reporting in Rel-14 is to assume the quality measurement on non-anchor carrier, and the possible scenario is illustrated in Figure 1. In the figure, we consider two measurement periods for the DL quality measurement: T1 and T2. T1 is the period before the NPRACH transmission, and T2 is the period from the beginning of RAR reception (including NPDCCH transmission period) to the MSG3 transmission. When RAN1/RAN2/RAN4 discussed the measurement resources for Rel-14 DL quality report, it was under the agenda of Rel-15 NB-IoT and some chipset vendors had already completed the design for Rel-14 chipset. 3GPP therefore accepted to assume both T1 and T2 as the measurement period in order to minimize the impact to the Rel-14 chipsets.

For the DL channel quality report on non-anchor carrier, on the other hand, it is for Rel-16 UEs. This means we can assume chipset vendors can start the design after the specification freeze. We therefore propose RAN4 assumes the measurement period for non-anchor carrier should be set to T2.

There is another reason we propose to limit to T2. For non-anchor carrier, the eNB does not need to transmit NRS except for the subframes where Paging or NPDCCH/NPDSCH is scheduled. Since the random access procedure timing depends on UE scheduling, eNB cannot expect when UE start the random access procedure and therefore eNB cannot transmit NRS for DL channel quality measurement for T1.

Proposal 1: RAN4 assumes the measurement period for DL channel quality reporting in IDLE for non-anchor is between the beginning of MSG2 transmission to the beginning of MSG3 transmission.

## 2.2	DL quality reporting in CONNECTED mode for anchor and non-anchor carriers

For channel quality report in connected mode, RAN1 agreed to reuse the same definition as Rel-14 NB-IoT MSG3-based channel quality report, i.e., the number of repetitions that the UE needs to decode hypothetical NPDCCH with BLER of 1%.

Regarding the measurement resources, RAN1 also agreed to use at least the carrier assigned to monitor in USS for NPDCCH and the associated NPDSCH. RAN1 is still discussing whether other carrier(s) are used for the DL channel quality measurement.

RAN4 received LS from RAN1 [3] where

RAN1 assumes the following aspects are decided by RAN2 and RAN4. - number of candidate values for channel quality reports - mapping value of reported channel quality - report in RRC message or MAC CE

According to RAN2 agreements, the DL quality report in connected mode is not periodic or on-demand. They also agreed to use MAC CE to report the DL channel quality of the configured carrier in RRC connected mode. We expect RAN2 will define a new MAC CE message format in the next step and identify the maximum number of bits used for the DL channel quality reporting. Based the available bits, RAN4 will discuss the number of candidate values and mapping. In our view RAN4 needs to wait for RAN2 decision.

Proposal 2: For DL quality reporting in CONNECTED mode, RAN4 need to wait further the conclusion for measurement resources and message contents.

# 3	Summary

Proposal 1: RAN4 assumes the measurement period for DL channel quality reporting in IDLE for non-anchor is between the beginning of MSG2 transmission to the beginning of MSG3 transmission.

Proposal 2: For DL quality reporting in CONNECTED mode, RAN4 need to wait further the conclusion for measurement resources and message contents.

# References

- RP-190757, “WID revision: Additional enhancements for NB-IoT”, Huawei, HiSilicon.

- R4-1907734, “Way forward on R16 enhancement for NB-IoT RRM​”, Huawei, HiSilicon, Nokia, Ericsson, Qualcomm.

- R1-1907624, “LS on channel quality report in connected mode for NB-IoT”, RAN1.

# Appendix (TS36.133)

#### 6.6.2.6	MSG3-based channel quality report for UE Category NB1

The requirements in this clause shall apply for UE supporting DL channel quality reporting for UE Category NB1 as defined in TS 36.331 [2] section 5.3.3.3, 5.3.3.3a, and 5.3.7.4.

The DL channel quality provides the serving eNB with information about the minimum NPDCCH repetition level to satisfy the hypothetical NPDCCH block error rate of 1% with the parameters specified in Table 6.6.2.6-1.

Table 6.6.2.6-1: NPDCCH transmission parameters for downlink quality reporting.

| Parameters | Values |
| --- | --- |
| DCI format | Format N1 |
| Number of information bits (excluding CRC) | 23bits |
| System bandwidth | 200kHz |
| Antenna configuration | 2x1 |
| Aggregation level | 2 |
| DRX | OFF |

The reported NPDCCH repetition level shall be derived from the channel quality measured in the period T1 or T2 in the carrier where the random access response is transmitted, where

-	T1 is the period before NPRACH transmission used for NRSRP measurement for enhanced coverage level estimation

-	T2 is the period from the beginning of the random access response to the beginning of PUSCH format 1 for DL channel quality reporting.

The NPDCCH repetition level for CQI-NPDCCH-NB and CQI-NPDCCH-Short-NB is chosen from the supported NPDCCH repetition levels [3]. The report mapping is defined in 9.1.22.15.

The UE shall satisfy the downlink channel quality measurement accuracy requirements as specified in 9.1.22.16.

## RAN1 agreements

RAN1#94 Agreement For channel quality report in Msg3 on non-anchor access, the channel quality definition is denoted by the number of repetitions that the UE needs to decode hypothetical NPDCCH with BLER of 1% - FFS: Whether the details on the hypothetical NPDCCH are specified or not Working Assumption For channel quality report in Msg3 on non-anchor access, UE performs the channel quality measurement on the carrier it monitors to receive Msg2 (i.e. RAR) - FFS: Whether the UE performs measurement on other carriers Agreement For non-anchor access, RAN1 further studies how UEs report the measured channel quality. RAN1#94bis Agreement RAN1 does not define search space for hypothetical NPDCCH for channel quality report in Msg3 on non-anchor access. Agreement From RAN1 point of view, specification support for measurement period for non-anchor access in RAN1 specifications is not needed Agreement RAN1 does not define measurement reference resource for non-anchor access. For further study: The following scenarios with regards to downlink channel quality reporting in msg3 for non-anchor carrier access. - For EDT/non-EDT, msg3 associated with PDCCH order PRACH, IDLE - PUR RAN4#95 Agreement In case 4 bits is used for a non-anchor carrier, all repetition i.e. 12 candidate values {1,2,4,8,16,32,64,128,256,512,1024,2048} can be reported in Msg3. Agreement In case of 2 bits is used for a non-anchor carrier, 3 candidate values can be reported in Msg3. Select one of the following alternatives for determining the 3 values: - Depending on Rmax, the maximum number of repetitions for NPDCCH Type 2 CSS. - Depending on R, "DCI subframe repetition number" indicated in DCI format N1 for Msg2 scheduling. Depending on Rdecoded, based on the number of repetitions for NPDCCH scheduling Msg2 where UE decodes successfully. RAN1#96 For further study in future meetings: For channel quality report in Msg3 on non-anchor access, RAN1 identified the following scenarios in which UE may monitor a carrier that is different from the carrier in which the UE receives Msg2 (i.e. RAR): - Idle mode - Non-EDT random access procedure - EDT procedure - Connected mode - NPDCCH ordered NPRACH procedure - Random access procedure to apply for UL-SCH resource Agreement For the measurement on carrier(s) other than the one UE is receiving RAR for non-anchor access, if supported, RAN1 to select one or more among the following candidates: - Paging carrier - Anchor carrier - Carrier(s) configured by CarrierConfigDedicated-NB in connected mode - Other carriers configured by network with implicit or explicit signaling The following issues are identified for RAN1 further study - Whether amount of time or gap is needed for the measurement - Which carrier(s) UE reported if more than one carrier is measured - What kind of NRS UE can use for the measurement and whether needs indication Agreement In case of 2 bits are used for a non-anchor carrier, one of 3 candidate values can be reported in Msg3 - Which depends on Rmax, the maximum number of repetitions for NPDCCH Type 2 CSS. For further study in future meetings: If supported, RAN1 further discuss how to derive the candidate values reported in Msg3 if no NPDCCH Type2 CSS on the carrier RAN1#96bis Agreement For channel quality report in connected mode, the channel quality definition is denoted by the number of repetitions that the UE needs to decode hypothetical NPDCCH with BLER of 1%. Agreement For channel quality report in connected mode other than Msg3, UE performs measurement on the carrier it is assigned to monitor in USS for NPDCCH and the associated NPDSCH. - FFS other carrier(s) Agreement For channel quality report in Msg3 for non-anchor access, RAN1 send LS to RAN2 requesting the following information: - Number(s) of available bits for the quality reporting in Msg3 - Whether the quality report in Msg3 is transmitted in RRC or MAC CE Send LS to RAN2 and CC RAN4. Ask, from RAN2, perspective, whether the above might be different for different cases Agreement For channel quality measurement in Msg3 for non-anchor access, the maximum number of carriers reported in Msg3 is no larger than 2. - FFS detail of the maximum number (1 or 2) Agreement eNB can enable/disable channel quality report in Msg3 for non-anchor access via SIB - FFS details - FFS whether the field in Rel-14 for anchor access is reused RAN1#97 Agreement For channel quality measurement in Msg3 for non-anchor access, the maximum number of carriers reported in Msg3 is 1. Agreement For channel quality report in Msg3 on non-anchor access in idle mode, channel quality measurement on the carrier UE monitors to receive Msg2 (i.e. RAR) is supported. - FFS: Support of network indication of which carrier to use to report Msg3 (e.g. explicit signaling in SI) Agreement For channel quality report in connected mode other than Msg3, RAN1 assumes the following aspects are decided by RAN2 and RAN4. - number of candidate values for channel quality reports - mapping value of reported channel quality - report in RRC message or MAC CE LS to RAN2 and RAN4 to convey this agreement is endorsed in R1-1907624 (Zhe, Huawei)

## RAN2 agreements

RAN2#103bis agreements: None. RAN2#104 agreements: Re-use the code points defined in Rel-14. Study the impact of re-using the Rel-14 RRC reporting mechanism and consider whether a MAC mechanism should be used instead. RAN2 further study how to support the use case of enabling measurements in non-anchor carrier while reducing measurement on anchor carrier. RAN2#105 agreements: None. RAN2#105bis agreements: - Channel Quality Reporting: Existing Msg3 signalling is used for reporting downlink channel quality when UE makes access on non-anchor carrier. Existing SIB2 signalling is used to enable UE to report downlink channel quality on non-anchor carrier. The DL channel quality reported in connected mode corresponds to the carrier used for the unicast transmission (i.e. configured by MSG4 or by a subsequent reconfiguration procedure). eNB enables the reporting of the DL Channel quality in connected mode. - FFS whether dedicated or broadcast signaling is used for enabling. Periodic reporting or on-demand reporting are not supported. DL channel quality reported in connected mode is optional for the UE. Working assumption that the same code points as Msg3 reporting is used. FFS whether MAC or RRC is used for reporting. R2-1905263 LS on non-anchor carrier CQI reporting in MSG3 Qualcomm Incorporated LS out Rel-16 NB_IOTenh3-Core To:RAN1 Cc: RAN4 - Non-anchor carrier measurements for RRM: If RAN4 confirms feasibility, and NRS is present in non-anchor carrier, RAN2 can make use of that for RRM measurements in non-anchor carrier. R2-1905264 LS on non-anchor carrier idle mode measurements for RRM Qualcomm Incorporated LS out Rel-16 NB_IOTenh3-Core To:RAN4 Cc: RAN1 RAN2#106 agreements: Channel Quality Reporting: The DL channel quality of the configured carrier in RRC connected mode is reported by MAC CE. Use the same LCID as agreed for eMTC. We need to wait for RAN4 before deciding when to report. FFS whether broadcast or dedicated signalling is used for NW to enable the reporting.