3GPP TSG-RAN WG4 Meeting #92	R4-1909157

Ljubljana, Slovenia, 26 – 30 August 2019

Agenda Item:	8.14.1.5

Source:	Ericsson

Title:	RLM for HST

Document for:	Discussion

# 1	Introduction

RAN4#91 agreed with the WF on RRM enhancement for LTE HST [1]. In WF, it was agreed that:

- In Rel-16 the target velocity for SCC RRM requirements for high speed enhancement is 350km/h, and the target velocity for PCC RRM requirement (single carrier mode) is 500km/h.

- Maximum Doppler shift in RRM simulations is limited to 972Hz, following the agreed UE demodulation assumption on Doppler limitation.

Regarding RLM on this condition, several options have been proposed:

- Option 1: For RLM in high speed scenario, define a new event according to some threshold Qout,E1 which is less than 10%. A new signalling will be introduced for UE to indicate network that it is going to be out-of-sync if PDCCH block error rate is above Qout,E1.

- Option 2: For RLM in high speed scenario, define the threshold Qout_HST as the level at which the downlink radio link cannot be reliably received and shall correspond to [10]% block error rate of a hypothetical PDCCH transmission with transmission parameters specified.

- Option 3: RAN4 specify RLM requirements only for the condition PDCCH is transmitted with transmission diversity when the new very high speed signalling is indicated.

- Option 4: No enhancement is needed

We will show our simulation results and discuss the RLM for HST.

# 2	Discussion

It is well known the radio link monitoring (RLM) is a procedure where UE measures the channel quality based on CRS. If the measured download channel quality is below the threshold Qout, UE indicates out-of-synch to the higher layer. On the other hand, if the measured channel quality exceeds the threshold during the out-of-synch, UE indicates in-synch to the higher layer.

According to TS36.133 7.6.1, the threshold Qout is defined as the level at which the channel quality corresponds to 10% BLER of a hypothetical PDCCH. On the other hand, the threshold Qin is defined as the level at which the channel quality can be significantly more reliably received than at Qout and shall correspond to 2% BLER of a hypothetical PDCCH. The assumed PDCCH parameters are shown in the appendix.

When RAN4 introduces RLM requirements, RAN4 usually performs PDCCH link level simulations based on the assumption parameters and verifies the thresholds Qin (i.e., SNR corresponding to 2% of PDCCH BLER) is significantly high compared with Qout (i.e., SNR corresponding to 10% of PDCCH BLER). According to the way forward [1], We performed the PDCCH simulation with Doppler=972Hz with HST/ETU/EVA channel models as shown in Table 1.

Table 1	Simulation parameters for DPCCH for RLM

| Parameters | Out-of-synch | In-synch |
| --- | --- | --- |
| DCI formats | 1A | 1C |
| Channel BW | 10MHz | 10MHz |
| Aggregation level | AL8 | AL4 |
| Antenna configuration | 1x2, 2x2 |  |
| Propagation channel | HS972, ETU972, EVA972 |  |

Figure 1	PDCCH BLER with Doppler=972Hz.

Table 2	SNR (dB) to achieve 2% (Qin) and 10% (Qout) of PDCCH BLER and the difference between Qin and Qout.

|  | 1Tx |  |  | 2Tx |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Channel Model | Qin | Qout | Diff | Qin | Qout | Diff |
| ETU 972Hz | 3.6 | -2.8 | 6.4 | 2.8 | -3.1 | 5.9 |
| EVA 972Hz | 3.3 | -3.4 | 6.7 | 2.7 | -3.5 | 6.2 |
| HST 972Hz | -3.0 | -6.8 | 3.8 | -2.2 | -6.7 | 4.5 |

Figure 1 shows the PDCCH BLER for the maximum Doppler frequency of 972Hz with path delay profiles ETU, EVA, and HST. Table 2 summarizes the required SNR to achieve 2% (Qin) and 10% (Qout) of PDCCH BLER for each channel model and shows the difference of SNR between Qin and Qout.

In [2], we observed it requires very high SNR to achieve PDCCH BLER < 1% or it may not achieve PDCCH BLER < 1% for the maximum Doppler frequency of 1250 Hz with single Tx transmission. With the assumption of the maximum Doppler frequency of 972Hz, our simulation result shows PDCCH BLER becomes less than 1% even with single Tx configuration. However, it is observed the SNR difference between Qin and Qout is small especially for HST 972Hz with single Tx configuration, 3.8dB.

Observation: SNR difference between Qin and Qout is small especially for HST 972Hz with single Tx transmission scenario.

When RAN4 sets RLM requirements, it is usually required at least 4-5dB for the robust RLM operation. We therefore propose to consider our simulation results and discuss further the RLM requirements for the condition PDCCH is transmitted with single transmission antenna in very high speed condition.

Proposal: RAN4 needs study further the RLM requirements for the condition PDCCH is transmitted with single transmission antenna in very high speed condition.

# 3	Conclusion

Observation: SNR difference between Qin and Qout is small especially for HST 972Hz with single Tx transmission scenario.

Proposal: RAN4 needs study further the RLM requirements for the condition PDCCH is transmitted with single transmission antenna in very high speed condition.

# References

- R4-1907739, “Way forward on RRM enhancement for LTE HST”, NTT DOCOMO.

- R4-1906412, “RLM for HST”, Ericsson.

# Appendix

Table 7.6.1-1 PDCCH/PCFICH transmission parameters for out-of-sync

| Attribute | Value |
| --- | --- |
| DCI format | 1A |
| Number of control OFDM symbols | 2; Bandwidth 10 MHz 3; 3 MHz Bandwidth 10 MHz 4; Bandwidth = 1.4 MHz |
| Aggregation level (CCE) | 4; Bandwidth = 1.4 MHz 8; Bandwidth 3 MHz |
| Ratio of PDCCH RE energy to average RS RE energy | 4 dB; when single antenna port is used for cell-specific reference signal transmission by the PCell or PSCell. 1 dB: when two or four antenna ports are used for cell-specific reference signal transmission by the PCell or PSCell. |
| Ratio of PCFICH RE energy to average RS RE energy | 4 dB; when single antenna port is used for cell-specific reference signal transmission by the PCell or PSCell. 1 dB: when two or four antenna ports are used for cell-specific reference signal transmission by the PCell or PSCell. |
| Note 1: DCI format 1A is defined in clause 5.3.3.1.3 in TS 36.212 [21]. Note 2: A hypothetical PCFICH transmission corresponding to the number of control symbols shall be assumed. |  |

Table 7.6.1-2 PDCCH/PCFICH transmission parameters for in-sync

| Attribute | Value |
| --- | --- |
| DCI format | 1C |
| Number of control OFDM symbols | 2; Bandwidth 10 MHz 3; 3 MHz Bandwidth 10 MHz 4; Bandwidth = 1.4 MHz |
| Aggregation level (CCE) | 4 |
| Ratio of PDCCH RE energy to average RS RE energy | 0 dB; when single antenna port is used for cell-specific reference signal transmission by the PCell or PSCell. -3 dB; when two or four antenna ports are used for cell-specific reference signal transmission by the PCell or PSCell. |
| Ratio of PCFICH RE energy to average RS RE energy | 4 dB; when single antenna port is used for cell-specific reference signal transmission by the PCell or PSCell. 1 dB: when two or four antenna ports are used for cell-specific reference signal transmission by the PCell or PSCell. |
| Note 1: DCI format 1C is defined in clause 5.3.3.1.4 in TS 36.212 [21]. Note 2: A hypothetical PCFICH transmission corresponding to the number of control symbols shall be assumed. |  |