3GPP TSG-RAN WG4 Meeting #92	R4-1909161

Ljubljana, Slovenia, 26 – 30 August 2019

Agenda Item:	7.11.1.2

Source:	Ericsson

Title:	CSI-RS periodicity for contention free random access tests

Document for:	Approval

# 1	Introduction

We discuss the CSI-RS periodicity parameters used for the contention free random access test (CFRA), which is TBD in TS38.133.

# 2	Discussion

One of the TBDs in TS38.133 is the CSI-RS periodicity and offset values as follows [1].

Table A.3.14.1-1: CSI-RS Reference Measurement Channels for SCS=15kHz

|  | CSI-RS.1.1 FDD | CSI-RS.1.2 FDD | CSI-RS.1.3 FDD | CSI-RS.1.4 FDD |
| --- | --- | --- | --- | --- |
| Resource Type | periodic | periodic | aperiodic | aperiodic |
| Resource Set Config |  |  |  |  |
| nzp-CSI-ResourceSetId | 0 | 0 | 0 | 0 |
| repetition | n.a. | off | off | on |
| aperiodicTriggeringOffset | n.a. | n.a. | 6 | 6 |
| trs-Info | n.a. | n.a. | n.a. | n.a. |
| Resource Config |  |  |  |  |
| nzp-CSI-RS-ResourceId | 0 for resource #0 | 10 for resource #0 | 20 for resource #0 | 30 for resource #0 |
|  |  |  |  | 31 for resource #1 |
|  |  |  |  | 32 for resource #2 |
|  |  |  |  | 33 for resource #3 |
|  |  | 11 for resource #1 | 21 for resource #1 | 34 for resource #4 |
|  |  |  |  | 35 for resource #5 |
|  |  |  |  | 36 for resource #6 |
|  |  |  |  | 37 for resource #7 |
| powerControlOffset | 0 | 0 | 0 | 0 |
| powerControlOffsetSS | db0 | db0 | db0 | db0 |
| scramblingID | 0 | 0 | 0 | 0 |
| Period (slots) | TBD | slot10 | n.a. | n.a. |
| Offset | TBD | 1 | n.a. | n.a. |
| qcl-InfoPeriodicCSI-RS | TCI.State.0 | TCI.State.0 | n.a. | n.a. |
|  |  | TCI.State.1 |  |  |
| frequencyDomainAllocation | 000001 | 000001 | 000001 | 000001 |
| nrofPorts | 2 | 1 | 1 | 1 |
| firstOFDMSymbolInTimeDomain | 5 for resource #0 | 6 for resource #0 | 6 for resource #0 | 0 for resource #0 |
|  |  |  |  | 1 for resource #1 |
|  |  |  |  | 2 for resource #2 |
|  |  |  |  | 3 for resource #3 |
|  |  | 10 for resource #1 | 10 for resource #1 | 4 for resource #4 |
|  |  |  |  | 5 for resource #5 |
|  |  |  |  | 6 for resource #6 |
|  |  |  |  | 7 for resource #7 |
| cdm-Type | FD-CDM2 | noCDM | noCDM | noCDM |
| density | 1 | 3 | 3 | 3 |
| startingRB | 0 | 0 | 0 | 0 |
| nrofRBs | 276 | 276 | 276 | 276 |

According to our observation, this RMC is used for the non-contention based random access (contention free random access, or CFRA) test.

According to TS38.213 8.1, PRACH occasions for CFRA is associated with a CSI-RS index. In our understanding the purpose of the CSI-RS configuration CSI-RS.1.1 FDD is to let UE calculate the initial preamble power and know the beam for CFRA. This is fundamentally same purpose as contention-based random access based on SSB. Therefore we propose to set the same periodicity as SSB, that is every 20ms.

TS38.213 8.1 For a PRACH transmission triggered upon request by higher layers, a value of ra-OccasionList [12, TS 38.331], if the value of the random access preamble index field is not zero, indicates a list of PRACH occasions for the PRACH transmission where the PRACH occasions are associated with a CSI-RS index indicated by csi-RS. The indexing of the PRACH occasions indicated by ra-OccasionList is reset per association pattern period.

# 3	Summary

Proposal: For CSI-RS periodicity and slot offset used for non-contention based random access procedure, set the periodicity to 20ms one slot after the QCL’ed SS/PBCH block, that is,

- For SCS=15kHz, period set to ‘slot20’ and offset set to ‘1’.

- For SCS=30kHz, period set to ‘slot40’ and offset set to ‘2’.

- For SCS=120kHz, period set to ‘slot160’ and offset set to ‘8’.

# References

- 3GPP TS38.133, “Requirements for support of radio resource management”, V.15.6.0.