3GPP TSG-RAN WG4 Meeting #92	R4-1909183

Ljubljana, Slovenia, 26th– 30th August, 2019

Source: 	Huawei, HiSilicon

Title:	On MSR new CS

Agenda Item:	12.1.2

Document for:	Approval

1	Introduction

In last RAN plenary meeting #83, a new WID [1] was approved to introduce to the MSR specifications support for [GSM+E-UTRA+NR] and [UTRA+E-UTRA+NR] and [GSM+UTRA+E-UTRA+NR]. For the new MSR CS(s), the following objectives are identified.

- Decide whether these combinations should be supported by a single new CS or multiple CS

- Identify RAT combinations to be supported by each introduced CS

In last RAN4 meeting, Way Forward on New Capability Sets were captured in [3]. In this contribution we provide our view on the options.

2	Discussion

As approved in [3], the introduction of CS20 is still for further decision

- Introduce the following Capability Sets:

  - CS18 GSM+E-UTRA+NR

  - CS19 UTRA+E-UTRA+NR

  - [CS20 GSM + UTRA + E-UTRA + NR]

  - Add a statement in the conformance specification that if two or more CS are supported, duplicated tests only need to be performed once.

- For CS20, decide whether or not to include considering:

  - Whether conformance for all RATs in a band is clear

  - If CS20 is included, testing of CS20 is a combination of CS18 and CS 19 testing

    - Tests defined for each configuration are performed only once

If both CS18 and CS 19 are supported, the test cases can simply sum of test cases for the individual CS. From the applicability table as proposed in [4], it can be found that some test cases are duplicated which can be performed only once, e.g. some single RAT tests. For the requirements use multi-RAT test configurations, e.g. TC23 and TC24, it might be difficult to further reduce test configurations to keep the coverage. Hence it is possible to define the conformance test for the case both CS 18 and CS 19 are supported at an antenna connector.

Proposal: CS18 and CS19 is proposed to introduce in MSR specification to support NR and GSM/UTRA multi-RAT operation.

| Capability Set | GSM + E-UTRA3 + NR (CS 18) | UTRA + E-UTRA3 + NR (CS 19) | CS 18 + CS 19 |
| --- | --- | --- | --- |
| BS test case | BC2 | BC1 and BC2 |  |
| 6.2 Base Station output power | - | - |  |
| Base Station maximum output power | C, NI, NG: TC23 CNC, NCNI, NCNG: NTC23 C/NC, C/NCNI, C/NCNG: NTC23, TC23 | C, NI, NG: TC24 CNC, NCNI, NCNG: NTC24 C/NC, C/NCNI, C/NCNG: NTC24, TC24 | + |
| E-UTRA for DL RS power | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| NB-IoT for DL RS power | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| UTRA FDD primary CPICH power | N/A | (TS 25.141) | + |
| UTRA FDD secondary CPICH power | N/A | (TS 25.141) | + |
| 6.3 Output power dynamics | - | - | - |
| E-UTRA | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| NB-IoT | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| NR | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) |
| UTRA FDD | N/A | (TS 25.141) | + |
| 6.4 Transmit ON/OFF power | - | - |  |
| Transmitter OFF power | N/A | N/A | N/A |
| Transmitter transient period | N/A | N/A | N/A |
| 6.5 Transmitted signal quality | - | - |  |
| 6.5.1 Modulation quality | - | - |  |
| E-UTRA | C: TC23 NI, NG: (Note 4) CNC: TC23 NCNI, NCNG: (Note 4) C/NC: NTC23, TC23 C/NCNI, C/NCNG: (Note 4) | C: TC24 NI, NG: (Note 4) CNC: TC24 NCNI, NCNG: (Note 4) C/NC: NTC24, TC24 C/NCNI, C/NCNG: (Note 4) | + |
| NB-IoT | N/A (Note 4) | N/A (Note 4) | N/A |
| NR | C: TC23 CNC: TC23 C/NC: NTC23, TC23 | C: TC24 CNC: TC24 C/NC: NTC24, TC24 | + |
| UTRA FDD | N/A | C: TC24 CNC: TC24 C/NC: NTC24, TC24 | + |
| GSM/EDGE | C: TC23 CNC: TC23 C/NC: NTC23, TC23 | N/A | + |
| 6.5.2 Frequency error | - | - |  |
| E-UTRA | Same TC as 6.5.1 | Same TC as 6.5.1 | + |
| NB-IoT | N/A (Note 4) | N/A (Note 4) | + |
| NR | Same TC as 6.5.1 | Same TC as 6.5.1 | + |
| UTRA FDD | N/A | Same TC as 6.5.1 | + |
| GSM/EDGE | Same TC as 6.5.1 | N/A | + |
| 6.5.3 Time alignment error | - | - |  |
| E-UTRA | (TS 36.141) NI, NG: (Note 4) NCNI, NCNG: (Note 4) C/NCNI, C/NCNG: (Note 4) | (TS 36.141) NI, NG: (Note 4) NCNI, NCNG: (Note 4) C/NCNI, C/NCNG: (Note 4) | (TS 36.141) NI, NG: (Note 4) NCNI, NCNG: (Note 4) C/NCNI, C/NCNG: (Note 4) |
| NB-IoT | N/A (Note 4) | N/A (Note 4) |  |
| NR | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) |
| UTRA FDD | N/A | (TS 25.141) | + |
| 6.6 Unwanted emissions | - | - |  |
| 6.6.1 Transmitter spurious emissions | - | - |  |
| (Category A) | C, NI, NG: TC23 CNC, NCNI, NCNG: NTC23 C/NC, C/NCNI, C/NCNG: NTC23, TC23 | C, NI, NG: TC24 CNC, NCNI, NCNG: NTC24 C/NC, C/NCNI, C/NCNG: NTC24, TC24 | + |
| (Category B) | C, NI, NG: TC23 CNC, NCNI, NCNG: NTC23 C/NC, C/NCNI, C/NCNG: NTC23, TC23 | C, NI, NG: TC24 CNC, NCNI, NCNG: NTC24 C/NC, C/NCNI, C/NCNG: NTC24, TC24 | + |
| Protection of the BS receiver of own or different BS | C, NI, NG: TC23 CNC, NCNI, NCNG: NTC23 C/NC, C/NCNI, C/NCNG: NTC23, TC23 | C, NI, NG: TC24 CNC, NCNI, NCNG: NTC24 C/NC, C/NCNI, C/NCNG: NTC24, TC24 | + |
| Additional spurious emissions requirements | C, NI, NG: TC23 CNC, NCNI, NCNG: NTC23 C/NC, C/NCNI, C/NCNG: NTC23, TC23 | C, NI, NG: TC24 CNC, NCNI, NCNG: NTC24 C/NC, C/NCNI, C/NCNG: NTC24, TC24 | + |
| Co-location with other Base Stations | C, NI, NG: TC23 CNC, NCNI, NCNG: NTC23 C/NC, C/NCNI, C/NCNG: NTC23, TC23 | C, NI, NG: TC24 CNC, NCNI, NCNG: NTC24 C/NC, C/NCNI, C/NCNG: NTC24, TC24 | + |
| 6.6.2 Operating band unwanted emissions | - | - |  |
| General requirement for Band Categories 1 and 3 | N/A | C, NI, NG: TC24 CNC, NCNI, NCNG: NTC24 C/NC, C/NCNI, C/NCNG: NTC24, TC24 SC: (Note 3) | + |
| General requirement for Band Category 2 | C, NI, NG: TC23 CNC, NCNI, NCNG: NTC23 C/NC, C/NCNI, C/NCNG: NTC23, TC23 SC: (Note 3) | N/A | + |
| Additional requirements | Compliance stated by manufacturer declaration | Compliance stated by manufacturer declaration | Compliance stated by manufacturer declaration |
| 6.6.3 Occupied bandwidth | - | - |  |
| Minimum requirement | (TS 36.141) (TS 38.141-1) | (TS 25.141) (TS 36.141) (TS 38.141-1) | (TS 25.141) (TS 36.141) (TS 38.141-1) |
| 6.6.4 Adjacent Channel Leakage power Ratio (ACLR) | - | - |  |
| E- UTRA | C: TC23 CNC: NTC23 C/NC: NTC23, TC23 | C: TC24 CNC: NTC24 C/NC: NTC24, TC24 | + |
| NB-IoT | NI: TC23 NG: TC23 NCNI: NTC23 NCNG: NTC23 C/NCNI, C/NCNG: NTC23, TC23 | NI: TC24 NG: TC24 NCNI: NTC24 NCNG: NTC24 C/NCNI, C/NCNG: NTC24, TC24 | + |
| NR | C: TC23 CNC: NTC23 C/NC: NTC23, TC23 | C: TC24 CNC: NTC24 C/NC: NTC24, TC24 | + |
| UTRA FDD | N/A | C: TC24 CNC: NTC24 C/NC: NTC24, TC24 | + |
| Cumulative ACLR | CNC: NTC23 C/NC: NTC23 | CNC: NTC24 C/NC: NTC24 | + |
| 6.7 Transmitter intermodulation | - | - |  |
| General requirement | Same TC as used in 6.6 | Same TC as used in 6.6 | + |
| Additional requirement (BC1 and BC2) | CNC: NTC23 C/NC: NTC23 | CNC: NTC24 C/NC: NTC24 | + |
| Additional requirement (BC3) | N/A | N/A |  |
| 7.2 Reference sensitivity level | - | - |  |
| E-UTRA | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| NB-IoT | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| NR | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) |
| UTRA FDD | N/A | (TS 25.141) | + |
| GSM/EDGE | (TS 51.021) | N/A | + |
| 7.3 Dynamic range |  |  |  |
| E-UTRA | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| NB-IoT | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| NR | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) |
| UTRA FDD | N/A | (TS 25.141) | + |
| 7.4 In- band selectivity and blocking | - | - |  |
| General blocking requirement | C, NI, NG: TC23 CNC, NCNI, NCNG: NTC23 C/NC, C/NCNI, C/NCNG: NTC23, TC23 | C, NI, NG: TC24 CNC, NCNI, NCNG: NTC24 C/NC, C/NCNI, C/NCNG: NTC24, TC24 | + |
| General narrowband blocking requirement | C, NI, NG: TC23 CNC, NCNI, NCNG: NTC23 C/NC, C/NCNI, C/NCNG: NTC23, TC23 | C, NI, NG: TC24 CNC, NCNI, NCNG: NTC24 C/NC, C/NCNI, C/NCNG: NTC24, TC24 | + |
| Additional BC3 blocking requirement | N/A | C, NI, NG: TC24 CNC, NCNI, NCNG: NTC24 C/NC, C/NCNI, C/NCNG: NTC24, TC24 | + |
| 7.5 Out-of-band blocking | - | - |  |
| General requirement | C, NI, NG: TC23 CNC, NCNI, NCNG: NTC23 C/NC, C/NCNI, C/NCNG: NTC23, TC23 | C, NI, NG: TC24 CNC, NCNI, NCNG: NTC24 C/NC, C/NCNI, C/NCNG: NTC24, TC24 | + |
| Co-location requirement | C, NI, NG: TC23 CNC, NCNI, NCNG: NTC23 C/NC, C/NCNI, C/NCNG: NTC23, TC23 | C, NI, NG: TC24 CNC, NCNI, NCNG: NTC24 C/NC, C/NCNI, C/NCNG: NTC24, TC24 | + |
| 7.6 Receiver spurious emissions | - | - |  |
| General requirement | C, NI, NG: TC23 CNC, NCNI, NCNG: NTC23 C/NC, C/NCNI, C/NCNG: NTC23, TC23 | C, NI, NG: TC24 CNC, NCNI, NCNG: NTC24 C/NC, C/NCNI, C/NCNG: NTC24, TC24 | + |
| Additional requirement for BC2 (Category B) | C, NI, NG: TC23 CNC, NCNI, NCNG: NTC23 C/NC, C/NCNI, C/NCNG: NTC23, TC23 | C, NI, NG: TC24 CNC, NCNI, NCNG: NTC24 C/NC, C/NCNI, C/NCNG: NTC24, TC24 | + |
| 7.7 Receiver intermodulation | - | - |  |
| General intermodulation requirement | C, NI, NG: TC23 CNC, NCNI, NCNG: NTC23 C/NC, C/NCNI, C/NCNG: NTC23, TC23 | C, NI, NG: TC24 CNC, NCNI, NCNG: NTC24 C/NC, C/NCNI, C/NCNG: NTC24, TC24 | + |
| General narrowband intermodulation requirement | C, NI, NG: TC23 CNC, NCNI, NCNG: NTC23 C/NC, C/NCNI, C/NCNG: NTC23, TC23 | C, NI, NG: TC24 CNC, NCNI, NCNG: NTC24 C/NC, C/NCNI, C/NCNG: NTC24, TC24 | + |
| 7.8 In-channel selectivity | - | - | - |
| E-UTRA requirement | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| NB-IoT | (TS 36.141) | (TS 36.141) | (TS 36.141) |
| NR | (TS 38.141-1) | (TS 38.141-1) | (TS 38.141-1) |

3	Conclusions

In this contribution we provide further discussion on MSR new CS.

Proposal: CS18 and CS19 is proposed to introduce in MSR specification to support NR and GSM/UTRA multi-RAT operation.

4	References

[1] RP-190642, New WID on introduction of GSM, UTRA, E-UTRA and NR capability set(s) (CS(s)) to the multi-standard radio (MSR) specifications

[2] R4-1903858, On MSR new CS, Huawei

[3] R4-1907807, WF on New capability sets for MSR, Ericsson, Huawei, Vodafone, Nokia