3GPP TSG RAN WG4 Meeting #92	R4-1909504

Ljubljana, Slovenia, August 26-30, 2019

Agenda Item:	7.10.6.1

Source: 	Ericsson

Title:	Analysis of one-shot timing adjustment requirements

Document for:	Discussion

- Introduction

In the last RAN4 meeting (RAN4#92) requirements related to one shot timing adjustment under the beam switch was agreed and incorporated in section 7.1.2.2 of TS 38.133. However, there are two main open issues:

- Threshold (H) values above which the UE adjusts its transmission timing in one adjustment

- Interruption time due to one-shot timing adjustment

The following related to interruption time was agreed in the WF [1]:

  - Interruption due to UE autonomous timing adjustment:

    - When UE perform one shot timing adjustment, interruption is allowed for both DL and UL transmission.

    - Depending on the value of H, the interruption duration is FFS.

    - The location of the interruption in time is FFS.

In this paper we provide analysis and address the above two open issues in order to finalize the remaining requirements.

# Analysis of Threshold

In section 7.1.2.2 the timing adjustment requirements based on one-shot or gradual change and its relation with the threshold, H, are specified as follows:

When the magnitude of the T exceeds H then the UE shall adjust its transmission timing in one adjustment only once provided that the following conditions are met at the UE. Otherwise when the the magnitude of the T ≤ H then the UE shall adjust its transmission timing according to the rules defined in clause 7.1.2.1.

Where:

- T = |T1-T2|

- T1 is the DL reception reference time of the former beam at the UE just before UE Rx beam switch

- T2 is the DL reception reference time of the new beam at the UE just after the UE Rx beam switch

- One shot UE transmit timing adjustment = 2*(T1-T2)

- The UE transmit timing immediately after the one shot adjustment: T2-(NTA+NTA_offset)Tc + 2*(T1-T2)

This means, H, is the threshold value beyond which the UE shall apply one shot adjustment in the uplink timing. If the threshold, H, is large then more often the UE will have to apply gradual adjustment. This in turn will take significantly longer for the UE to adjust the UE transmit timing to appropriate level i.e. to fully compensate its due to the beam switch. The delay in bringing the UE transmit timing to appropriate level means the base station reception quality will be degraded. The CP length also needs to account for timing drift due to other factors such as multipaths etc. Secondly the uplink error at the base station due to beam switch will also be caused due to other factors such as UE initial timing error (Te), TA resolution step errors, TA adjustment error from the previous beam etc. Therefore, in our opinion the threshold, H, should be small fraction of the CP length.

## Impact of existing UE timing error on BS reception error

Without considering any beam switch the existing timing error at BS reception (TBSE) comprises of at least UE initial timing error (Te), TA resolution step errors, TA adjustment error and can be expressed as follows:

TBSE = Te + TACE + TAAE                                                                                                            (1)

Where:

- Te = UE transmit timing error (section 7.1.2, TS 38.133)

- TACE = TA command resolution error (section 4.2, TS 38.213)

- TAAE = TA adjustment accuracy (section 7.3.2, TS 38.133)

Table 1: TA command resolution error and TA adjustment accuracy as function of UL SCS

| UL SCS [kHz] | TACE [Tc] | TAAE [Tc] |
| --- | --- | --- |
| 15 | ± 512 | ± 256 |
| 30 | ± 256 | ± 256 |
| 60 | ± 128 | ± 128 |
| 120 | ± 64 | ± 32 |

The existing timing error at the BS reception (TBSE), without considering any beam based on Eq. (1) is shown in table 2. The results show that the BS timing error is quite substantial e.g. as large as 39% of UL CP for 60 kHz UL SCS.

Table 2: Relation between CP length and existing timing errors at the BS reception

| SSB SCS (kHz) | UL SCS (kHz) | UE timing error (Te) [Tc] | BS error (TBSE) [Tc] | BS error (TBSE) as percentage of UL CP length (%) |
| --- | --- | --- | --- | --- |
| 15 | 15 | 768 | 1536 | 17 |
|  | 30 | 640 | 1152 | 25 |
|  | 60 | 640 | 896 | 39 |
| 30 | 15 | 512 | 1280 | 14 |
|  | 30 | 512 | 1024 | 22 |
|  | 60 | 448 | 704 | 31 |
| 120 | 60 | 224 | 480 | 21 |
|  | 120 | 224 | 320 | 28 |
| 240 | 60 | 192 | 448 | 19 |
|  | 120 | 192 | 288 | 25 |

## 2.2. Impact of different H values on BS reception error

Let us now considering the impact of beam switching on the BS reception errors.

When T ≤ Te then the UE will NOT apply one-shot timing adjustment. In this case there will be an additional bias of 2*T in terms of timing error at the BS reception. Therefore, the total BS error (BSRE_beam) considering beam switching aspect can be expressed by (2):

TBSE_beam = TBSE + 2*T                                                            (2)

Where TBSE is defined in Eq. (1).

### Example # 1: H = Te

Consider an example with H=Te. For H = Te, 2*T = 2*Te. Therefore, the total timing error at the BS reception according to (2) will be sum of TBSE and 2*Te.

Table 3 show the total timing error at the BS reception in terms of absolute values (in Tc) and also in terms of percentage of the UL CP length. The BS reception error as percentage of the UL CP is extremely large. In several cases (e.g. 60 kHz, 120 kHz) almost the entire CP or substantial part of it is lost due to the timing errors. The UL CP length at the base station also needs to account for timing drift due to multipaths. Therefore the level of error shown in table 3 is not acceptable from the BS performance point of view.

Table 3: Total timing error at BS when T ≤ H; where H = Te

| SSB SCS (kHz) | UL SCS (kHz) | H=Te [Tc] | BS error (TBSE_beam) [Tc] | BS error (TBSE_beam) as percentage of UL CP length (%) |
| --- | --- | --- | --- | --- |
| 15 | 15 | 768 | 3072 | 33 |
|  | 30 | 640 | 2432 | 53 |
|  | 60 | 640 | 2176 | 94 |
| 30 | 15 | 512 | 2304 | 25 |
|  | 30 | 512 | 2048 | 44 |
|  | 60 | 448 | 1600 | 69 |
| 120 | 60 | 224 | 928 | 40 |
|  | 120 | 224 | 768 | 67 |
| 240 | 60 | 192 | 832 | 36 |
|  | 120 | 192 | 672 | 58 |

### Example # 2: H = Te/2

Now consider another example with H=Te/2. For H = Te, 2*T = Te. Therefore, the total timing error at the BS reception according to (2) will be sum of TBSE and Te.

Table 4 show the total timing error at the BS reception in terms of absolute values (in Tc) and also in terms of percentage of the UL CP length. The total BS reception timing errors as percentage of the UL CP compared to those in previous example (#2) is substantially reduced. Even though in some cases the error is still high but considering also UE implementation aspect, H = Te/2 can be acceptable.

Table 4: Total timing error at BS when T ≤ H; where H = Te/2

| SSB SCS (kHz) | UL SCS (kHz) | H=Te [Tc] | BS error (TBSE_beam) [Tc] | BS error (TBSE_beam) as percentage of UL CP length (%) |
| --- | --- | --- | --- | --- |
| 15 | 15 | 768 | 2304 | 25 |
|  | 30 | 640 | 1792 | 39 |
|  | 60 | 640 | 1536 | 67 |
| 30 | 15 | 512 | 1792 | 19 |
|  | 30 | 512 | 1536 | 33 |
|  | 60 | 448 | 1152 | 50 |
| 120 | 60 | 224 | 704 | 31 |
|  | 120 | 224 | 544 | 47 |
| 240 | 60 | 192 | 640 | 28 |
|  | 120 | 192 | 480 | 42 |

# Proposed Values of Threshold

Based on the above analysis provided in section 2 it is proposed to H = Te/2. for different SCS are shown in table 5.

Table 5: The value of H

| Frequency Range | SCS of SSB signals (KHz) | SCS of uplink signals s(KHz) | H [Tc] |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 384 |
|  |  | 30 | 320 |
|  |  | 60 | 320 |
|  | 30 | 15 | 256 |
|  |  | 30 | 256 |
|  |  | 60 | 224 |
| 2 | 120 | 60 | 112 |
|  |  | 120 | 112 |
|  | 240 | 60 | 96 |
|  |  | 120 | 96 |

- Observation # 1: The threshold, H, beyond which the UE applies single shot adjustment should be small fraction of UL CP length to prevent BS reception problem.

- Proposal # 1: The threshold, H, beyond which the UE applies single shot timing adjustment shall be Te/2.

# Analysis of interruption

As discussed in the previous section the value of H should be small fraction of CP. With H = Te/2 the one-shot adjustment will be done when time difference is much smaller than CP length. Therefore, even in the worst case where the difference between the old beam and the new beam becomes much larger than H (e.g. in order of CP length such as 0.57µs for 120 kHz SCS) the interruption should not be larger than 1 symbol. However, one problem is that the beam switching is done by the UE autonomously. Therefore, it is not possible to define reference time when the actual interruption will occur.

One possibility is to specify that the UE is allowed an interruption of 1 symbol in UL and 1 symbol in DL at specific symbol within the slot e.g. the last symbol of the slot. The rationale is that it is better to avoid interruption in the first symbol, which carriers the control channel. Howevr, the interruption may occur on all serving cells and even on FR1 serving cell if the UE does not support per-FR gap. Furthermore, due to MRTD it cannot be guaranteed that the interruption occurs in the last symbol of the slot in all the serving cells. Due to the fact that one-shot timing adjustment may rarely cause interruption and it is not realistic to define exact location in time where the interruption occurs, we propose not to introduce any interruption requirement.

- Observation # 2: Upon applying one-shot timing adjustment the UE may rarely cause interruption.

- Proposal # 2: No interruption requirement due to one-shot adjustment is specified.

- Summary

In this paper we have analysed the remaining issues in order to finalize the one-shot UE transmit timing requirements. The following are the main proposals:

- Observation # 1: The threshold, H, beyond which the UE applies single shot adjustment should be small fraction of UL CP length to prevent BS reception problem.

- Proposal # 1: The threshold, H, beyond which the UE applies single shot adjustment shall be Te/2.

| Frequency Range | SCS of SSB signals (KHz) | SCS of uplink signals s(KHz) | H [Tc] |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 384 |
|  |  | 30 | 320 |
|  |  | 60 | 320 |
|  | 30 | 15 | 256 |
|  |  | 30 | 256 |
|  |  | 60 | 224 |
| 2 | 120 | 60 | 112 |
|  |  | 120 | 112 |
|  | 240 | 60 | 96 |
|  |  | 120 | 96 |

- Observation # 1: Upon applying one-shot timing adjustment the UE may rarely cause interruption.

- Proposal # 2: No interruption requirement due to one-shot timing adjustment is specified.

A CR to update the requirements based on the above proposals is provided in [2].

# References

- R4-1907203, Way forward on UE timing adjustment with Beam switching, Ericsson, Intel, Mediatek, Nokia, Nokia Shanghai Bell, Qualcomm.

- R4-1909505, Correction to one shot UE timing adjustment requirements (7.1.2.2), Ericsson.