3GPP TSG-RAN WG4 Meeting #93                                      R4-1913323

Reno, NV, USA, 18th – 22nd Nov, 2019

Agenda item:			9.11.2.1

Source:			Samsung

Title:	Further Discussion on L1-SINR Measurement and Reporting

Document for:		Discussion

# 1	Introduction

The Rel-16 MIMO enhancement work item was approved in RAN#80 with a list of enhancement areas including L1-SINR measurement and reporting. Back to very early discussion of Rel-16 eMIMO, RAN1 has concluded that L1-SINR is confirmed to be supported while L1-RSRQ is excluded to be measured and reported.

During last RAN4 meeting, the following agreement is achieved for L1-SINR measurement and reporting related RRM requirement [3]:

<Agreement from RAN4#92Bis RRM Session> - L1-SINR Measurement: - L1-SINR measurement procedure: - Measurement period: - For L1-SINR measurements based on CMR only, define L1-SINR measurement period as a number of CMR periodicities and reuse the requirements on L1-RSRP measurement period for both FR1 and FR2. FFS for L1-SINR measurements based on CMR and IMR. - Measurement restriction: - Define L1-SINR measurement restriction requirement by applying the same principles as for L1-RSRP measurement. - Scheduling restriction: - Define L1-SINR scheduling restriction requirement by applying the same principles as for L1-RSRP measurement. - UE capability on L1-SINR: - L1-SINR measurement requirement should be based on applicability rule depending on UE capability to support this feature. - SINR measurement reporting table: - Extend current L3 SS-SINR measurement reporting table for L1 SS-SINR and L1 CSI-SINR usage, with maintaining the range and quantization of SINR mapping. - Introduce differential SS-SINR and CSI-SINR measurement (for L1 reporting) report mapping table. - Side condition for L1-SINR measurement - CMR only scenarios: reuse side condition for L1-RSRP measurement - CMR + IMR scenarios: FFS <Way Forward for L1-SINR Measurement> - L1-SINR measurement procedure: - FFS measurement period requirement for L1-SINR measurements based on CMR and IMR. - Scenarios of L1-SINR measurement for RRM requirement: - In RAN4#93, RAN4 study and conclude which scenario(s) shall be included for L1-SINR measurement requirement and evaluation (i.e., measurement period and measurement accuracy). - Down-selecting scenarios to define L1-SINR measurement requirement from following possible scenarios: - Scenario 1: L1-SINR measured on CMR only (when dedicated IMR is not configured) - Scenario 1A: CSI-RS based CMR (only with density 3 REs/RB) - Scenario 2: L1-SINR measured on CMR+IMR (when dedicated IMR is configured) - Scenario 2A: SSB based CMR + ZP-IMR - Scenario 2B: CSI-RS based CMR + ZP-IMR - Scenario 2C: SSB based CMR + NZP-IMR (only with density 3 REs/RB) - Scenario 2D: CSI-RS based CMR + NZP-IMR (only with density 3 REs/RB) - Other options are not excluded, depending on RAN1 agreement. - At least the following aspects are identified to be needed for consideration: - Question-1: Whether to define separate requirements for CMR only and CMR+IMR? - Question-2: Whether to define separate requirements for SSB-based CMR and CSI-RS based CMR? - Question-3: Whether to define separate requirements for different IMR configuration?

Hence, in this discussion paper, we would like to further provide our analysis and view for outstanding issues for L1-SINR measurement and reporting related RRM requirement.

# Discussion

To continue the RAN4 discussion in RAN4#92Bis, we would like to further share our understanding of RAN1-defined L1-SINR and corresponding impact on RAN4 RRM core requirement in TS38.133.

#### 2.1 Contents in Beam Measurement Reports

In addition to the content analysed in our previous discussion paper [4], additional agreements are achieved for L1-SINR reporting content in beam measurement reports in RAN1#92Bis:

<RAN1 agreement> Agreement made in RAN1#98Bis - The bit length is fixed to be 4 bits for differential L1-SINR - The step size is fixed to be 1 dB for differential L1-SINR

Considering RAN4 agreement that “Extend current L3 SS-SINR measurement reporting table for L1 SS-SINR and L1 CSI-SINR usage, with maintaining the range and quantization of SINR mapping.” And “Introduce differential SS-SINR and CSI-SINR measurement (for L1 reporting) report mapping table.” We can conclude that the following differential L1-SINR table is needed to be introduced.

Proposal-1: For L1-SINR measurement, RAN4 introduce differential SS-SINR and CSI-SINR measurement as below table:

| Reported value | Measured quantity value (difference in measured SINR from largest SINR) | Unit |
| --- | --- | --- |
| DIFFSINR_0 | 0≥ΔSINR>-1 | dB |
| DIFFSINR_1 | -1≥ΔSINR >-2 | dB |
| DIFFSINR_2 | -2≥ΔSINR >-3 | dB |
| DIFFSINR_3 | -3≥ΔSINR >-8 | dB |
| DIFFSINR_4 | -4≥ΔSINR >-10 | dB |
| DIFFSINR_5 | -5≥ΔSINR >-12 | dB |
| DIFFSINR_6 | -6≥ΔSINR>-14 | dB |
| DIFFSINR_7 | -7≥ΔSINR >-16 | dB |
| DIFFSINR_8 | -8≥ΔSINR >-18 | dB |
| DIFFSINR_9 | -9≥ΔSINR >-20 | dB |
| DIFFSINR_10 | -10≥ΔSINR >-22 | dB |
| DIFFSINR_11 | -11≥ΔSINR >-24 | dB |
| DIFFSINR_12 | -12≥ΔSINR >-26 | dB |
| DIFFSINR_13 | -13≥ΔSINR >-28 | dB |
| DIFFSINR_14 | -14≥ΔSINR >-30 | dB |
| DIFFSINR_15 | -15≥ΔSINR | dB |

#### 2.2 Scenarios of L1-SINR measurement

We would like to firstly go back to RAN1 agreement till last meeting, and based on that, the possible scenario is summarized.

<RAN1 agreement> Agreement made in RAN1#97 - When dedicated IMR is not configured, - If CMR is based on CSI-RS, when L1-SINR is configured, and interference measurement is performed using CMR with CSI-RS only with density 3 REs/RB for 1-port CSI-RS is used - Spec does not require UE to use SSB for interference measurement - Note: CSI-RS above is CSI-RS for BM - When dedicated IMR is configured, - NW can configure interference measurement for L1-SINR with either of the following options - ZP-IMR only - NZP-IMR only - (WA) ZP-IMR and NZP IMR (interference measurement is taken on both) - Maximum Number of ZP IMR is 1 - If IMR is configured based on NZP IMR only, when L1-SINR is configured, interference measurement is performed only with density 3 REs/RB CSI-RS - If IMR is configured based on ZP IMR only, when L1-SINR is configured, interference measurement is performed using ZP IMR - FFS: interference measurement is performed using CMR additionally - Support of L1-SINR is optional - FFS: Support of NZP IMR and ZP IMR are separate UE capabilities - Note: CSI-RS above is CSI-RS for BM Agreement made in RAN1#98 For L1-SINR based beam report, in a CSI-reportConfig, if IMR is configured to be based on ZP-IMR only: - CMR and IMR are 1-to-1 mapped from signaling perspective Agreement made in RAN1#98Bis For NZP-IMR based interference measurement, option 1a is supported - In a CSI-reportConfig, gNB configures a list of N CMR(s) and another list of N IMR(s), and they are 1:1 mapped - For each SINR, interference is measured based on each associated NZP-IMR only - UE may assume that the NZP CSI-RS resource for channel measurement and NZP CSI-RS resource(s) for interference measurement configured for one CSI reporting are QCLed with respect to 'QCL-TypeD’ - FFS: Whether QCL-TypeD can be configured to each NZP IMR - FFS: Each NZP CSI-RS port configured for interference measurement corresponds to an interference transmission layer - FFS: Additional support of option 2a (without RRC signalling impact) Note: There is no consensus in RAN1 on the support of option 2b/2c (which introduces IMR index reporting for L1-SINR)

Based on the above RAN1 discussion, it has been agreed that gNB can configure Channel Measurement Resource (CMR) only or CMR as well as Interference Measurement Resource (IMR) for L1-SINR measurement.

• Scenario 1: L1-SINR measured on CMR only (when dedicated IMR is not configured)

- Scenario 1A: CSI-RS based CMR (only with density 3 REs/RB)

Note: SSB-based CMR can only be used for the scenario where dedicated IMR is configured, and CSI-RS-based CMR can be used for both scenarios where dedicated IMR is configured and not configured. If IMR is not configured and CMR is based on CSI-RS, only CSI-RS with density = 3 REs/RB can be configured.

• Scenario 2: L1-SINR measured on CMR+IMR (when dedicated IMR is configured)

- Scenario 2A: SSB based CMR + ZP-IMR

- Scenario 2B: CSI-RS based CMR + ZP-IMR

- Scenario 2C: SSB based CMR + NZP-IMR (only with density 3 REs/RB)

- Scenario 2D: CSI-RS based CMR + NZP-IMR (only with density 3 REs/RB)

Note: RAN1 agree to introduce ZP-IMR only and NZP-IMR only for interference measurement.

Furthermore, detailed RAN1 discussion on how to configure CMR/IMR provide the conclusion that, CMR/IMR association is based on 1-to-1 mapping for both ZP-IMR only and NZP-IMR only scenarios.

Proposal-1: Accordingly based on RAN1 agreement, RAN4 define RRM requirement for L1-SINR measurement for following scenarios:

• Scenario 1: L1-SINR measured on CMR only (when dedicated IMR is not configured)

- Scenario 1A: CSI-RS based CMR (only with density 3 REs/RB)

• Scenario 2: L1-SINR measured on CMR+IMR (when dedicated IMR is configured)

- Scenario 2A: SSB based CMR + ZP-IMR

- Scenario 2B: CSI-RS based CMR + ZP-IMR

- Scenario 2C: SSB based CMR + NZP-IMR (only with density 3 REs/RB)

- Scenario 2D: CSI-RS based CMR + NZP-IMR (only with density 3 REs/RB)

Furthermore, in previous RAN4 discussion, the following questions are proposed.

- Question-1: Whether to define separate requirements for CMR only and CMR+IMR?

- Question-2: Whether to define separate requirements for SSB-based CMR and CSI-RS based CMR?

- Question-3: Whether to define separate requirements for different IMR configuration?

Similar to L1-RSRP measurement requirement, SSB-based and CSI-RS based measurement requirements needs to be defined separately, while CMR only and CMR+IMR provide another dimension to separate measurement requirements. For both ZP-IMR and NZP-IMR for interference measurement, we expect that the same L1-SINR measurement requirement shall be applied, so no need to further discriminate for different IMR configurations. Accordingly, we propose to use the following structure for L1-SINR measurement.

Proposal-2: L1-SINR measurement requirements for different scenarios shall be defined based on following structure:

- L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured

- L1-SINR reporting with SSB based CMR and dedicated IMR configured

- L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured

##### 2.2.1 L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured (CMR only)

As mentioned above, for Scenario 1A (CSI-RS based CMR and no dedicated IMR configured), the following agreement is achieved during last RAN4 meeting:

<Agreement from RAN4#92Bis RRM Session> - L1-SINR Measurement: - L1-SINR measurement procedure: - Measurement period: - For L1-SINR measurements based on CMR only, define L1-SINR measurement period as a number of CMR periodicities and reuse the requirements on L1-RSRP measurement period for both FR1 and FR2. FFS for L1-SINR measurements based on CMR and IMR. - Side condition for L1-SINR measurement - CMR only scenarios: reuse side condition for L1-RSRP measurement

Based on the above agreement, we expect the measurement period (namely, TL1-SINR_Measurement_Period_CSI-RS_CMR_only) shall be the same as measurement period for CSI-RS based L1-RSRP defined in Section 9.5.4.2.

Proposal-3: For L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured, the measurement period requirements for FR1 and FR2 shall be defined as the below tables:

Table x: Measurement period TL1-SINR_Measurement_Period_CSI-RS_CMR_Only for FR1

| Configuration | TL1-SINR_Measurement_Period_CSI-RS_CMR_Only (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-SINR measurement is transmitted with Density = 3. |  |

Table y: Measurement period TL1-SINR_Measurement_Period_CSI-RS_CMR_Only for FR2

| Configuration | TL1-SINR_Measurement_Period_CSI-RS_CMR_Only (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P*N)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-SINR measurement is transmitted with Density = 3. |  |

Where the variable M, P, N are the same as Section 9.5.4.2 for L1-RSRP reporting.

##### 2.2.2 L1-SINR reporting with SSB based CMR and dedicated IMR configured (SSB CMR+IMR)

For the scenario of L1-SINR reporting with SSB based CMR and dedicated IMR (either ZP-IMR only or NZP-IMR only), RAN1 agree that CMR and IMR are 1-to-1 mapped from signaling perspective for both ZP-IMR only and NZP-IMR only.

Observation-1: For both ZP-IMR only and NZP-IMR only scenarios, CMR/IMR association is based on 1-to-1 mapping.

Accordingly, for SSB-based CMR with IMR configured, it is reasonable to assume that for L1-SINR measurement period as a number of CMR periodicities and reuse the requirements on SSB-based L1-RSRP measurement period for both FR1 and FR2.

Proposal-4: For SSB-based CMR with IMR configured, L1-SINR measurement period as a number of CMR periodicities and reuse the requirements on SSB-based L1-RSRP measurement period for both FR1 and FR2 as defined in Section 9.5.4.1.

##### 2.2.3 L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured (SSB CMR+IMR)

For the scenario of L1-SINR reporting with SSB based CMR and dedicated IMR (either ZP-IMR and NZP-IMR), the CMR/IMR 1-to-1 mapping relationship is also valid, and we propose that CSI-RS based L1-RSRP requirement can be resued:

Proposal-5: For CSI-RS-based CMR with IMR configured, L1-SINR measurement period as a number of CMR periodicities and reuse the requirements on CSI-RS-based L1-RSRP measurement period for both FR1 and FR2 as defined in Section 9.5.4.2.

#### 2.3 Measurement restriction and scheduling availability

In previous meeting’s RAN4 agreement, the same principles as for L1-RSRP measurement is needed to be used for L1-SINR measurement.

<Agreement from RAN4#92Bis RRM Session> - Measurement restriction: - Define L1-SINR measurement restriction requirement by applying the same principles as for L1-RSRP measurement. - Scheduling restriction: - Define L1-SINR scheduling restriction requirement by applying the same principles as for L1-RSRP measurement.

For SSB as reference signal for L1-SINR measurement, SSB is only possible to be configured as CMR. Therefore, the measurement restriction requirement can be defined as:

<Captured from accompanying CR> #### 9.x.5.1 Measurement restriction if SSB configured for L1-SINR Measurement For FR1, when the SSB configured as CMR for L1-SINR measurement is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD, L1-RSRP or L1-SINR measurement, - If SSB and CSI-RS have same SCS, UE shall be able to measure the SSB for L1-SINR measurement without any restriction; - If SSB and CSI-RS have different SCS, - If UE supports simultaneousRxDataSSB-DiffNumerology, UE shall be able to measure the SSB for L1-SINR measurement without any restriction; - If UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both SSB for L1-SINR measurement and CSI-RS. Longer measurement period for SSB based L1-SINR measurement is expected, and no requirements are defined. For FR2, when the SSB configured as CMR for L1-SINR measurement is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD, L1-RSRP or L1-SINR measurement, UE is required to measure one of but not both SSB for L1-SINR measurement and CSI-RS. Longer measurement period for SSB based L1-RSRP measurement is expected, and no requirements are defined.

However, it could be a more complex for CSI-RS which is possible to be configured as CMR or IMR (either ZP-IMR or NZP-IMR). Then, at least the following questions needs to be answered:

For CSI-RS configured as CMR:

Q1: For CSI-RS configured as CMR in FR2, is that possible to configure CSI-RS with “repetition = on”?

Q2: If answer to Q1 is yes, does measurement restriction is applied if this CSI-RS with “repetition = on” is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD, L1-RSRP or L1-SINR measurement.

For simplicity, RAN4 can define requirement based on “yes to Q1 and Q2”, similar to L1-RSRP measurement. However, the scenario of CSI-RS configured as CMR in FR2 with “repetition = on” is somehow questionable.

Proposal-6: For CSI-RS configured as CMR with “repetition = on”, measurement restriction is applied if this CSI-RS is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD, L1-RSRP or L1-SINR measurement.

For CSI-RS configured as NZP-IMR:

Q3: For CSI-RS configured as NZP-IMR in FR2, is that possible to configure CSI-RS with “repetition = on”?

Based on our understanding, the scenario of CSI-RS configured as NZP-IMR with “repetition = on” is questionable, since it is not meaningful to instruct UE to train RX beam for interference, while RX beam refinement should be always based on CMR.

Proposal-7: The scenario of CSI-RS configured as NZP-IMR with “repetition = on” should be excluded in RAN4 requirement.

For CSI-RS configured as ZP-IMR:

Q4: For CSI-RS configured as ZP-IMR in FR2, is that possible to configure CSI-RS with “repetition = on”?

From our understanding, the configuration for CSI-RS configured as ZP-IMR in FR2 with “repetition = on” is wrong configuration.

Proposal-8: The scenario of CSI-RS configured as ZP-IMR with “repetition = on” should be excluded in RAN4 requirement.

Since we expect more discussion on the scheduling availability and measurement restriction for CSI-RS configured as IMR, in our corresponding CR, the related content is marked in square bracket.

# Conclusion

In this paper, we provided our views on L1-SINR measurement and reporting, with the following observations and proposals achieved:

Proposal-1: For L1-SINR measurement, RAN4 introduce differential SS-SINR and CSI-SINR measurement as below table:

| Reported value | Measured quantity value (difference in measured SINR from largest SINR) | Unit |
| --- | --- | --- |
| DIFFSINR_0 | 0≥ΔSINR>-1 | dB |
| DIFFSINR_1 | -1≥ΔSINR >-2 | dB |
| DIFFSINR_2 | -2≥ΔSINR >-3 | dB |
| DIFFSINR_3 | -3≥ΔSINR >-8 | dB |
| DIFFSINR_4 | -4≥ΔSINR >-10 | dB |
| DIFFSINR_5 | -5≥ΔSINR >-12 | dB |
| DIFFSINR_6 | -6≥ΔSINR>-14 | dB |
| DIFFSINR_7 | -7≥ΔSINR >-16 | dB |
| DIFFSINR_8 | -8≥ΔSINR >-18 | dB |
| DIFFSINR_9 | -9≥ΔSINR >-20 | dB |
| DIFFSINR_10 | -10≥ΔSINR >-22 | dB |
| DIFFSINR_11 | -11≥ΔSINR >-24 | dB |
| DIFFSINR_12 | -12≥ΔSINR >-26 | dB |
| DIFFSINR_13 | -13≥ΔSINR >-28 | dB |
| DIFFSINR_14 | -14≥ΔSINR >-30 | dB |
| DIFFSINR_15 | -15≥ΔSINR | dB |

Proposal-1: Accordingly based on RAN1 agreement, RAN4 define RRM requirement for L1-SINR measurement for following scenarios:

• Scenario 1: L1-SINR measured on CMR only (when dedicated IMR is not configured)

- Scenario 1A: CSI-RS based CMR (only with density 3 REs/RB)

• Scenario 2: L1-SINR measured on CMR+IMR (when dedicated IMR is configured)

- Scenario 2A: SSB based CMR + ZP-IMR

- Scenario 2B: CSI-RS based CMR + ZP-IMR

- Scenario 2C: SSB based CMR + NZP-IMR (only with density 3 REs/RB)

- Scenario 2D: CSI-RS based CMR + NZP-IMR (only with density 3 REs/RB)

Proposal-2: L1-SINR measurement requirements for different scenarios shall be defined based on following structure:

- L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured

- L1-SINR reporting with SSB based CMR and dedicated IMR configured

- L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured

Proposal-3: For L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured, the measurement period requirements for FR1 and FR2 shall be defined as the below tables:

- Table x: Measurement period TL1-SINR_Measurement_Period_CSI-RS_CMR_Only for FR1

| Configuration | TL1-SINR_Measurement_Period_CSI-RS_CMR_Only (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-SINR measurement is transmitted with Density = 3. |  |

- Table y: Measurement period TL1-SINR_Measurement_Period_CSI-RS_CMR_Only for FR2

| Configuration | TL1-SINR_Measurement_Period_CSI-RS_CMR_Only (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P*N)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-SINR measurement is transmitted with Density = 3. |  |

- Where the variable M, P, N are the same as Section 9.5.4.2 for L1-RSRP reporting.

Observation-1: For both ZP-IMR only and NZP-IMR only scenarios, CMR/IMR association is based on 1-to-1 mapping.

Proposal-4: For SSB-based CMR with IMR configured, L1-SINR measurement period as a number of CMR periodicities and reuse the requirements on SSB-based L1-RSRP measurement period for both FR1 and FR2 as defined in Section 9.5.4.1.

Proposal-5: For CSI-RS-based CMR with IMR configured, L1-SINR measurement period as a number of CMR periodicities and reuse the requirements on CSI-RS-based L1-RSRP measurement period for both FR1 and FR2 as defined in Section 9.5.4.2.

Proposal-6: For CSI-RS configured as CMR with “repetition = on”, measurement restriction is applied if this CSI-RS is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD, L1-RSRP or L1-SINR measurement.

Proposal-7: The scenario of CSI-RS configured as NZP-IMR with “repetition = on” should be excluded in RAN4 requirement.

Proposal-8: The scenario of CSI-RS configured as ZP-IMR with “repetition = on” should be excluded in RAN4 requirement.

# Reference

[1] RP-192271,	“Revised WID: Enhancements on MIMO for NR”, Samsung.

[2] RP-192270,	“Status Report for WI: Enhancements on MIMO for NR”, Samsung

[3] R4-1912742, “WF on RRM requirement scope for Rel-16 NR eMIMO WI”, Samsung, Huawei.

[4] R4-1911390, “Discussion on L1-SINR measurement and reporting for Rel-16 MIMO enhancement”, Samsung.