3GPP TSG-RAN WG4 Meeting #93                                 R4-1913886

Reno, Nevada, USA, Nov. 18th – 22nd, 2019

Source: 	MediaTek Inc.

Title: 	FR1 CA bandwidth class definition

Agenda Item:	7.5.2.1

Document for:	Approval

# Introduction

The proposal for extending FR1 CA bandwidth class B upper aggregated channel bandwidth limit from 50 MHz to 100 MHz was approved in last RAN4 meeting to mitigate a potential issue in n48 under contiguous CA operation [1]. However, this change would also reverse the CR agreement in RAN4 #88 meeting which was implemented to address the conflict between CA bandwidth class B and class F due to the aggregated channel bandwidth range overlap [2]. Considering that either of the CA bandwidth class B definition before and after the approved change in last RAN4 meeting would have its own associated issue, in this contribution, we propose to remove CA bandwidth class F and reclassify the CA bandwidth class B fallback group from 0 to 2 to resolve both aforementioned issues.

# Discussion

Although the proposal for extending FR1 CA bandwidth class B upper aggregated channel bandwidth limit from 50 MHz to 100 MHz was approved in last RAN4 meeting [1], the concern as raised during the online discussions and previously addressed in the agreed CR in RAN4 #88 meeting [2] would adversely resurface which can clearly be realized by following the FR1 CA bandwidth class definition evolution as shown in Table 2-1 to Table 2-6 and summarized in Table 2-7.

The very first FR1 CA bandwidth class definition (in V15.1.0) as shown in Table 2-1 was essentially following the scheme for E-UTRA CA bandwidth class definition. However, in RAN4 #86bis meeting, the concern for not being able to use 15kHz SCS for all CCs in contiguous CA to support channel bandwidth wider than 100 MHz was raised [3]. It was then proposed to introduce CA bandwidth class F to L to accommodate this potential need which was further classified as a separate fallback group (fallback group 2) to differentiate from CA bandwidth class C to E (fallback group 1). The proposed change was later captured in V15.2.0 as shown in Table 2-2 through a CR in RAN4 #87 meeting [4]. In RAN4 #88 meeting, it was identified that the proposed change in CA bandwidth class B aggregated channel bandwidth upper limit from 100 MHz to 50 MHz [4] was not captured in V15.2.0 which would cause the potential conflict between CA bandwidth class B and class F due to the aggregated channel bandwidth range overlap. The issue was addressed by a CR [2] which modified the CA bandwidth class B aggregated channel bandwidth upper limit from 100 MHz to 50 MHz and was captured in V15.3.0 as shown in Table 2-3. In RAN4 #89 meeting, a CR was agreed to add “NOTE 2” in the CA bandwidth class table to indicate the fallback requirement [5] which was captured in V15.4.0 as shown in Table 2-4. In RAN4 #90 meeting, a CR was agreed to clarify the fallback group for CA bandwidth class A and B where the former was specified as belonging to fallback group 0, 1, 2 and the latter was specified as fallback group 0 [6]. The change was captured in V15.5.0 as shown in Table 2-5. However, in our view specifying the fallback group for bandwidth class A is deemed unnecessary as CA in any fallback group should always start with a single component carrier.

Observation 1: It is not necessary to specify fallback group for bandwidth class A as CA in any fallback group should always start with a single component carrier.

In last RAN4 meeting, the proposal for extending FR1 CA bandwidth class B upper aggregated channel bandwidth limit from 50 MHz to 100 MHz was approved [1] which would reverse the CR agreement in RAN4 #88 meeting [2] and return the FR1 CA bandwidth class definition back to V15.2.0 as shown in Table 2-2.

TS 38.101-1 V15.1.0

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC |
| --- | --- | --- |
| A | CBW ≤ max CHBW | 1 |
| B | 20 MHz ≤ CBW ≤ 100 MHz | 2 |
| C | 100 MHz < CBW ≤ 2 x max CHBW | 2 |
| D | 200 MHz < CBW ≤ 3 x max CHBW | 3 |
| E | 300 MHz < CBW ≤ 4 x max CHBW | 4 |
| NOTE: max CHBW is maximum channel bandwidth supported among all bands in a release |  |  |

Table 2-1 FR1 CA bandwidth class definition in TS 38.101-1 V15.1.0

TS 38.101-1 V15.2.0

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel_CA ≤ BWChannel,max | 1 |  |
| B | 20 MHz ≤ CBW ≤ 100 MHz | 2 |  |
| C | 100 MHz < BWChannel_CA ≤ 2 x BWChannel,max | 2 | 1 |
| D | 200 MHz < BWChannel_CA ≤ 3 x BWChannel,max | 3 |  |
| E | 300 MHz < BWChannel_CA ≤ 4 x BWChannel,max | 4 |  |
| F | 50 MHz < BWChannel_CA ≤ 100 MHz | 2 | 2 |
| G | 100 MHz < BWChannel_CA ≤ 150 MHz | 3 |  |
| H | 150 MHz < BWChannel_CA ≤ 200 MHz | 4 |  |
| I | 200 MHz < BWChannel_CA ≤ 250 MHz | 5 |  |
| J | 250 MHz < BWChannel_CA ≤ 300 MHz | 6 |  |
| K | 300 MHz < BWChannel_CA ≤ 350 MHz | 7 |  |
| L | 350 MHz < BWChannel_CA ≤ 400 MHz | 8 |  |
| NOTE: BWChannel, max is maximum channel bandwidth supported among all bands in a release |  |  |  |

Table 2-2 FR1 CA bandwidth class definition in TS 38.101-1 V15.2.0

TS 38.101-1 V15.3.0

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel_CA ≤ BWChannel,max | 1 |  |
| B | 20 MHz ≤ BWChannel_CA ≤ 50 MHz | 2 |  |
| C | 100 MHz < BWChannel_CA ≤ 2 x BWChannel,max | 2 | 1 |
| D | 200 MHz < BWChannel_CA ≤ 3 x BWChannel,max | 3 |  |
| E | 300 MHz < BWChannel_CA ≤ 4 x BWChannel,max | 4 |  |
| F | 50 MHz < BWChannel_CA ≤ 100 MHz | 2 | 2 |
| G | 100 MHz < BWChannel_CA ≤ 150 MHz | 3 |  |
| H | 150 MHz < BWChannel_CA ≤ 200 MHz | 4 |  |
| I | 200 MHz < BWChannel_CA ≤ 250 MHz | 5 |  |
| J | 250 MHz < BWChannel_CA ≤ 300 MHz | 6 |  |
| K | 300 MHz < BWChannel_CA ≤ 350 MHz | 7 |  |
| L | 350 MHz < BWChannel_CA ≤ 400 MHz | 8 |  |
| NOTE: BWChannel, max is maximum channel bandwidth supported among all bands in a release |  |  |  |

Table 2-3 FR1 CA bandwidth class definition in TS 38.101-1 V15.3.0

TS 38.101-1 V15.4.0

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel_CA ≤ BWChannel,max | 1 |  |
| B | 20 MHz ≤ BWChannel_CA ≤ 50 MHz | 2 |  |
| C | 100 MHz < BWChannel_CA ≤ 2 x BWChannel,max | 2 | 1 |
| D | 200 MHz < BWChannel_CA ≤ 3 x BWChannel,max | 3 |  |
| E | 300 MHz < BWChannel_CA ≤ 4 x BWChannel,max | 4 |  |
| F | 50 MHz < BWChannel_CA ≤ 100 MHz | 2 | 2 |
| G | 100 MHz < BWChannel_CA ≤ 150 MHz | 3 |  |
| H | 150 MHz < BWChannel_CA ≤ 200 MHz | 4 |  |
| I | 200 MHz < BWChannel_CA ≤ 250 MHz | 5 |  |
| J | 250 MHz < BWChannel_CA ≤ 300 MHz | 6 |  |
| K | 300 MHz < BWChannel_CA ≤ 350 MHz | 7 |  |
| L | 350 MHz < BWChannel_CA ≤ 400 MHz | 8 |  |
| NOTE 1: BWChannel, max is maximum channel bandwidth supported among all bands in a release NOTE 2: It is mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration within a fallback group. It is not mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration that belong to a different fallback group |  |  |  |

Table 2-4 FR1 CA bandwidth class definition in TS 38.101-1 V15.4.0

TS 38.101-1 V15.4.0

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel_CA ≤ BWChannel,max | 1 | 0, 1, 2 |
| B | 20 MHz ≤ BWChannel_CA ≤ 50 MHz | 2 | 0 |
| C | 100 MHz < BWChannel_CA ≤ 2 x BWChannel,max | 2 | 1 |
| D | 200 MHz < BWChannel_CA ≤ 3 x BWChannel,max | 3 |  |
| E | 300 MHz < BWChannel_CA ≤ 4 x BWChannel,max | 4 |  |
| F | 50 MHz < BWChannel_CA ≤ 100 MHz | 2 | 2 |
| G | 100 MHz < BWChannel_CA ≤ 150 MHz | 3 |  |
| H | 150 MHz < BWChannel_CA ≤ 200 MHz | 4 |  |
| I | 200 MHz < BWChannel_CA ≤ 250 MHz | 5 |  |
| J | 250 MHz < BWChannel_CA ≤ 300 MHz | 6 |  |
| K | 300 MHz < BWChannel_CA ≤ 350 MHz | 7 |  |
| L | 350 MHz < BWChannel_CA ≤ 400 MHz | 8 |  |
| NOTE 1: BWChannel, max is maximum channel bandwidth supported among all bands in a release NOTE 2: It is mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration within a fallback group. It is not mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration that belong to a different fallback group |  |  |  |

Table 2-5 FR1 CA bandwidth class definition in TS 38.101-1 V15.5.0

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel ≤ BWChannel,max | 1 | 0, 1, 2 |
| B | 20 MHz ≤ BWChannel_CA ≤ 100 MHz | 2 | 0 |
| C | 100 MHz < BWChannel_CA ≤ 2 x BWChannel,max | 2 | 1 |
| D | 200 MHz < BWChannel_CA ≤ 3 x BWChannel,max | 3 |  |
| E | 300 MHz < BWChannel_CA ≤ 4 x BWChannel,max | 4 |  |
| F | 50 MHz < BWChannel_CA ≤ 100 MHz | 2 | 2 |
| G | 100 MHz < BWChannel_CA ≤ 150 MHz | 3 |  |
| H | 150 MHz < BWChannel_CA ≤ 200 MHz | 4 |  |
| I | 200 MHz < BWChannel_CA ≤ 250 MHz | 5 |  |
| J | 250 MHz < BWChannel_CA ≤ 300 MHz | 6 |  |
| K | 300 MHz < BWChannel_CA ≤ 350 MHz | 7 |  |
| L | 350 MHz < BWChannel_CA ≤ 400 MHz | 8 |  |
| NOTE 1: BWChannel, max is maximum channel bandwidth supported among all bands in a release NOTE 2: It is mandatory for a UE to be able to fallback to lower order NR CA bandwidth class configuration within a fallback group. It is not mandatory for a UE to be able to fallback to lower order NR CA bandwidth class configuration that belong to a different fallback group |  |  |  |

Table 2-6 FR1 CA bandwidth class B change approved in RAN4 #92bis meeting

| TS 38.101-1 Revision | CR Tdoc Number | Reason for Change | Summary of Changes |
| --- | --- | --- | --- |
| V15.2.0 | R4-1806677 | CA with 15kHz SCS for all CCs not supported | Added BW class F to L as fallback group 2 |
| V15.3.0 | R4-1809784 | Conflict between class B and F | Changed class B BW upper limit from 100 MHz to 50 MHz |
| V15.4.0 | R4-1815060 | To align with 38.101-2 on fallback requirement | Added NOTE 2 |
| V15.5.0 | R4-1902225 | Missing clarification of fallback group for BW class A | Specified BW class A as fallback group 0,1,2 and BW class B as fallback group 0 |

Table 2-7 summary of FR1 CA bandwidth class definition evolution

To mitigate the issues caused by the CA bandwidth class B and bandwidth class F either with or without aggregated bandwidth range overlap, it is proposed to remove CA bandwidth class F and maintain bandwidth class B with aggregated bandwidth upper limit kept at 100 MHz. In addition, CA bandwidth class B is re-categorized as fallback group 2 and fallback group clarification for bandwidth class A can be removed. The proposed changes are presented in Table 2-8.

Proposal: FR1 CA bandwidth class definition is specified as in Table 2-8.

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel ≤ BWChannel,max | 1 |  |
| B | 20 MHz ≤ BWChannel_CA ≤ 100 MHz | 2 | 2 |
| C | 100 MHz < BWChannel_CA ≤ 2 x BWChannel,max | 2 | 1 |
| D | 200 MHz < BWChannel_CA ≤ 3 x BWChannel,max | 3 |  |
| E | 300 MHz < BWChannel_CA ≤ 4 x BWChannel,max | 4 |  |
|  |  |  | 2 |
| G | 100 MHz < BWChannel_CA ≤ 150 MHz | 3 |  |
| H | 150 MHz < BWChannel_CA ≤ 200 MHz | 4 |  |
| I | 200 MHz < BWChannel_CA ≤ 250 MHz | 5 |  |
| J | 250 MHz < BWChannel_CA ≤ 300 MHz | 6 |  |
| K | 300 MHz < BWChannel_CA ≤ 350 MHz | 7 |  |
| L | 350 MHz < BWChannel_CA ≤ 400 MHz | 8 |  |
| NOTE 1: BWChannel, max is maximum channel bandwidth supported among all bands in a release NOTE 2: It is mandatory for a UE to be able to fallback to lower order NR CA bandwidth class configuration within a fallback group. It is not mandatory for a UE to be able to fallback to lower order NR CA bandwidth class configuration that belong to a different fallback group |  |  |  |

Table 2-8 Proposed FR1 CA bandwidth class definition

# Conclusion

In this contribution, we revisit the issues encountered in current CA bandwidth class B and CA bandwidth class F definitions and propose to remove CA bandwidth class F to resolve the potential issues.

# References

- R4-1912608 “Band n48 CA bandwidth class and CA configuration”, Verizon, Nokia, Ericsson, Qualcomm, 3GPP TSG-RAN WG4 Meeting #92bis, Chongqing, China, October 14th – 18th, 2019

- R4-1809784 “Draft CR to 38.101-1: Corrections on CA bandwidth classes for FR1 in section 5.3A.5”, ZTE Corporation, 3GPP TSG-RAN WG4 Meeting #88, Gothenburg, Sweden, August 20th – 24th, 2018

- R4-1803985 “CA bandwidth class for 15 kHz SCS at FR1”, LG Electronics, 3GPP TSG-RAN WG4 Meeting #86bis, Melbourne, Australia, April 14th – 20th, 2019

- R4-1806677 “CR for updated CA BW Classes to support SCS 15kHz”, LG Electronics, 3GPP TSG-RAN WG4 Meeting #87, Busan, Korea, May 21st – 25th, 2018

- R4-1815060 “draft CR for adding note about the fallback of NR CA in FR1 for TS 38.101-1”, NTT DOCOMO Inc., 3GPP TSG-RAN WG4 Meeting #89, Spokane, Washington, USA, November 12th – 16th, 2018

- R4-1902225 “CR to 38.101-1 on CA BW Classes fallback groups”, Intel Corporation, 3GPP TSG-RAN WG4 Meeting #90, Athens, Greece, February 25th – March 1st, 2019