3GPP TSG-RAN WG4 Meeting #93                                                 R4-1914079

# Reno, USA, 18th – 22nd Nov, 2019

Agenda Item:	9.11.2.2

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements for BFR on SCell

Document for:		Discussion

- Introduction

The RRM core requirement impact for BFR on SCell should be identified [1], and some views are provided in this paper.

- BFR on SCell

For L3 measurement requirement, it can observe that a sharing factor CSSF is introduced in section 9.1.5 [2], and it is scaled up by the number of configured SCell(s).

Observation 1: For L3 measurement, the RRM measurement period is scaled up by the number of configured SCell(s).

In R16, BFR could be performed on multiple SCells, based on UE capability, and it would require to investigate whether to introduce such sharing factor when multiple SCells are configured for BFR procedure.

Proposal 1: RAN4 to study whether to introduce a sharing factor for BFD and CBD measurement period.

In FR2 intra-band CA, UE assumes the same downlink spatial domain transmission filter for the serving cells on the same band. For a given time point, it can observe that if a beam failure has been detected on one serving cell, very likely the beam failure would also occur on other serving cells on the same band, since the same beamforming will be applied. Therefore, it is not necessary to simultaneously perform BFR procedure on multiple serving cells on the same FR2 band.

Observation 2: It is sufficient to perform BFR procedure on one serving cell on the same FR2 band, since UE assumes the same downlink spatial domain transmission filter for the serving cells on the same FR2 band.

Proposal 2: For BFD and CBD requirements in FR2, up to one serving cell is assumed to perform BFR procedure for one FR2 band.

- Summary

In this paper, the update of UE capability for NR-U are provided. We have the following observations and proposals:

Observation 1: For L3 measurement, the RRM measurement period is scaled up by the number of configured SCell(s).

Proposal 1: RAN4 to study whether to introduce a sharing factor for BFD and CBD measurement period.

Observation 1: For L3 measurement, the RRM measurement period is scaled up by the number of configured SCell(s).

Proposal 2: For BFD and CBD requirements in FR2, up to one serving cell is assumed to perform BFR procedure for one FR2 band.

References

[1] WF on RRM Requirement Scope for Rel-16 NR eMIMO WI, Samsung, Huawei

[2] TS38.133, NR; Requirements for support of radio resource management, V15.7.0.