3GPP TSG-RAN WG4 Meeting #93                                         R4-1914087

Reno, US, 18th– 22nd November 2019

Agenda item:	10.16.4	Others 	[NR_n259-Core/Perf]

Source: 		NTT DOCOMO, INC.

Title: 	BS measurement uncertainty and test tolerance for Band n259

Document for:	Approval

- Introduction

In RAN4#92bis, the need for updates related to BS RF/Conformance for the introduction of band n259 was discussed [1]. Conformance testing aspects such as measurement uncertainty (MU) and test tolerance (TT) are still under discussion and no consensus has been reached. In this contribution, we provide our views how to handle them.

2.	Discussion

In [1], the necessity of changes for BS RF conformance specification was summarized and discussed. As mentioned in the paper, it can be seen that some test requirements, MU and TT support only up to 40GHz, which is not enough for support of band n259. Copy and paste the table below that summarizes the need for changes.

Observation 1: In the current specification, some test requirements, MU and TT do not support band n259.

Table 2: Summary of the necessity of changes for BS RF conformance specification

| Section (TS 38.141-2) | Requirement | Frequency range of operating band applicable to test requirement (NOTE 1) | Frequency range applicable to test system uncertainty (MU) for test requirements (Sec. 4.1.2) (NOTE 1) | Frequency range applicable to test tolerance (TT) for test requirements (Annex C) (NOTE 1) | Necessity to update for introduction of n259 |
| --- | --- | --- | --- | --- | --- |
| Radiated transmitter characteristics |  |  |  |  |  |
| 6.2 | Radiated transmit power | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | 24.25 – 29.5 GHz 37 – 40 GHz | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | Yes |
| 6.3 | OTA base station output power | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | 24.25 – 29.5 GHz 37 – 40 GHz | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | Yes |
| 9.4.3 | OTA total power dynamic range | Not mentioned | Not mentioned | Not mentioned | No |
| 6.5.1 | OTA transmitter OFF power | Refer to OTA transient period | 24.25 – 29.5 GHz 37 – 40 GHz | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | Yes |

| Section (TS 38.141-2) | Requirement | Frequency range of operating band applicable to test requirement (NOTE 1) | Frequency range applicable to test system uncertainty (MU) for test requirements (Sec. 4.1.2) (NOTE 1) | Frequency range applicable to test tolerance (TT) for test requirements (Annex C) (NOTE 1) | Necessity to update for introduction of n259 |
| --- | --- | --- | --- | --- | --- |
| Radiated transmitter characteristics |  |  |  |  |  |
| 6.5.2 | OTA transient period | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | - | - | Yes |
| 6.6.2 | OTA frequency error | Not mentioned | Not mentioned | Not mentioned | No |
| 6.6.3 | OTA modulation quality | Not mentioned | Not mentioned | Not mentioned | No |
| 6.6.4 | OTA time alignment error | Not mentioned | Not mentioned | Not mentioned | No |
| 6.7.1 | General for OTA unwanted emissions | Not mentioned (However the definition of ΔfOBUE is not enough) | - | - | Yes (Need to update the definition of ΔfOBUE) |
| 6.7.2 | OTA occupied bandwidth | Not mentioned | Not mentioned | Not mentioned | No |
| 6.7.3 | OTA Adjacent Channel Leakage Power Ratio (ACLR) | 24.25 – 33.4 GHz 37 – 52.6 GHz | 24.25 – 29.5 GHz 37 – 40 GHz | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | Yes |
| 6.7.4 | OTA operating band unwanted emissions | 24.25 – 33.4 GHz 37 – 52.6 GHz | 24.25 – 29.5 GHz 37 – 40 GHz | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | Yes |
| 6.7.5 | OTA transmitter spurious emissions | Not mentioned (Step frequency for Cat.B does not cover n259) | Not mentioned | Not mentioned | Need to check (Need to add n259 to Table 6.7.5.2.5.2.3-2 if necessary) |
| Radiated receiver characteristics |  |  |  |  |  |
| 7.2 | OTA sensitivity | Not mentioned |  | - | No |
| 7.3 | OTA reference sensitivity level | Not mentioned | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | 24.25 GHz < f ≦ 33.4 GHz 37 GHz < f ≦ 52.6 GHz | Yes |
| 7.5.1 | OTA adjacent channel selectivity | 24.25 – 33.4 GHz 37 – 52.6 GHz | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | Not mentioned | Yes |
| 7.5.2 | OTA in-band blocking | Not mentioned (However the definition of ΔfOOB is not enough) | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | Not mentioned | Yes (Need to update the definition of ΔfOOB) |
| 7.6 | OTA out-of-band blocking | Not mentioned | Not mentioned | Not mentioned | No |
| 7.7 | OTA receiver spurious emissions | Not mentioned | Not mentioned | Not mentioned | No |
| 7.8 | OTA receiver intermodulation | Not mentioned | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | Not mentioned | Yes |
| 7.9 | OTA in-channel selectivity | Not mentioned | 24.15 GHz < f ≤ 29.5 GHz 37 GHz < f ≤ 40 GHz | 24.25 GHz < f ≦ 33.4 GHz 37 GHz < f ≦ 52.6 GHz | Yes |
| NOTE 1: “Not mentioned” imply applicable to 24.25 – 52.6GHz. |  |  |  |  |  |

Basically, MU can be calculated based on some uncertainty sources. For example, according to CATR uncertainty budget of EIRP measurement captured in Table 12.3.1.2.2.1-1, TR 38.817-02, it can be seen that uncertainty sources affected by frequency changes are only “RF power measurement equipment” and “Uncertainty of return loss (S11)”. Therefore, it is sufficient to consider these sources to expand the frequency range.

Observation 2: Uncertainty sources affected by frequency changes are only “RF power measurement equipment” and “Uncertainty of return loss (S11)”.

Table 12.3.1.2.2.1-1: Compact antenna test range uncertainty assessment for EIRP measurement

| UID (Note 1) | Uncertainty Source | Uncertainty value 24.25<f <29.5GHz | Uncertainty value 37<f <40GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB) 24.25<f <29.5GHz | Standard uncertainty ui (dB) 37<f <40GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| ----- Ommited----- |  |  |  |  |  |  |  |  |
| E2-2 | RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.5 | 0.7 | Gaussian | 1 | 1 | 0,5 | 0,7 |
| ----- Ommited----- |  |  |  |  |  |  |  |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| ----- Ommited----- |  |  |  |  |  |  |  |  |
| E2-7 | Uncertainty of return loss (S11) measurement of SGH and test receiver (VNA) ports | 0.43 | 0.57 | U-shaped | √2 | 1 | 0,21 | 0,26 |
| ----- Ommited----- |  |  |  |  |  |  |  |  |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  | 0,88 | 1,03 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  | 1,73 | 2,02 |
| Note 1: UID are referenced to 3GPP TR 37.843 [26]. |  |  |  |  |  |  |  |  |

For these uncertainty sources, the following values were proposed in Rel.15 discussion.

RF power measurement uncertainty:

- 0.7 dB was proposed for 37GHz – 40GHz in [2]

- 1.28 dB was proposed for 26GHz - 50GHz in [3, 4].

ncertainty of return loss (S11):

- 0.57 dB was proposed for 37GHz – 40GHz in [2]

- 0.74 dB was proposed for FR2 in [3, 4].

- 0.5 dB was proposed for FR2 in [5]

After online and offline discussion in RAN4 #88, 0.7 dB for RF power measurement uncertainty and 0.57 dB for uncertainty of return loss (S11) were agreed in [6].  Since the FR2 band specified in Rel.15 is up to 40 GHz, the frequency range to which these uncertainties apply is specified as 37-40 GHz. On the other hand, according to [3, 4, 5], the same uncertainty values can be used for the entire FR2.

Observation 3: Some companies assume the same uncertainty source values for the entire FR2 or 26 – 50 GHz.

From the above observations, the existing test requirements, MU and TT can be reused for band n259 since the same uncertainty source values can be used for more than 40GHz.

Proposal 1: The existing test requirement, test system uncertainty (MU) and test tolerance (TT) shall be applied to n259 (39.5 GHz – 43.5 GHz).

3.	Conclusion

In this contribution, we further analyse the necessity to update TS 38.141-2 for the introduction of n259.

Measurement uncertainty (MU) and test tolerance (TT)

Observation 1: In the current specification, some test requirements, MU and TT do not support band n259.

Observation 2: Uncertainty sources affected by frequency changes are only “RF power measurement equipment” and “Uncertainty of return loss (S11)”.

Observation 3: Some companies assume the same uncertainty source values for the entire FR2 or 26 – 50 GHz.

Proposal 1: The existing test requirement, test system uncertainty (MU) and test tolerance (TT) shall be applied to n259 (39.5 GHz – 43.5 GHz).

References

- R4-1912448, “BS RF requirement and test tolerance for n259”, NTT DOCOMO, INC., RAN4 #92bis, October 2019

- R4- 1810506, “Radiated transmit power and TX power dynamic range uncertainty budget for FR2 in CATR”, Ericsson, RAN4 #88, August 2018

- R4-1807130, “Test Equipment uncertainty values for FR2”, Keysight, NSI-MI, RAN4 #87, May 2018

- R4-1809884, “NR FR2 OTA measurement uncertainty value for CATR”, Keysight, NSI-MI, RAN4 #88, August 2018

- R4-1811264, “Proposed MU budgets for OTA EIRP and EIS measurements of BS at mmWave in CATR”, MVG, RAN4 #88, August 2018

- R4-1811635, “WF on MU and TT for transmission in-band TRP emission and directional requirements for FR2”, Ericsson, RAN4 #88, August 2018