3GPP TSG-RAN WG4 Meeting #93                                  R4-1914107

Reno, USA, 18th– 22nd November, 2019

Agenda item:  11.3.1

Source:       China Unicom

Title:          TP for TR 37.815: Conclusion of SI for FDD+TDD EN-DC HPUE

Document for:	 Approval

- Introduction

The study item of high power UE (power class 2) for EN-DC (1 LTE FDD band + 1 NR TDD band) has been extended several RAN4 meetings according to the initial work plan. Six options for reporting UE capability to solve SAR issue had been discussed. Based the agreed WF[1] during last RAN4 meeting, It is supposed to conclude a solution by selecting one of them in RAN4#93.

The main target of the SI is to study the feasible solution to comply with the SAR requirement for two cases in the SI scope, i.e. case 1 for 23dBm LTE + 23dBm NR and case 2 for 23dBm LTE + 26dBm NR.

Base on discussion in RAN#93bis, the group prefers to report one duty cycle as UE capability in Rel-16. With this capability, network can help to guarantee the UE not exceeding the SAR limits at the UE maximum output power. The specific signalling design and the value range can be further considered during the WI stage. Meanwhile, the UE based solution should also be considered as the reported maximum duty cycle capability cannot always be guaranteed by the network scheduling. A combined UE based, and network based solution to better improve the UL performance is suggested in the SI study.

Required changes in TS 38.101-3 were also analysed during the SI study, the identified requirements shall be specified for specific FDD+TDD EN-DC band combinations in the following WI.

In the SI study, only one band combination, i.e. DC_3_n78 is proposed as an example combination to facilitate the study. Once the SI is finished, a WI will be proposed to standardize the SAR solution as well the specifying requirements for proposed band combinations. It is expected that more band combinations will be proposed in the WI stage, thus it is proposed to treat the FDD+TDD EN-DC HPUE as a basket WI.

- References

[1] R4-1913077 WF for EN-DC PC2 FDD-TDD China Unicom, CHTTL, OPPO, China Telecom, Huawei, HiSilicon

[2] R4-1910338 WF for EN-DC PC2 FDD-TDD China Unicom, CHTTL, China Telecom

[3] R4-1909060 Effective use of power of PC2 UE with two different UL bands NTT DOCOMO, INC.

[4] R4-1908591 Further discussion on solutions for EN-DC FDD-TDD High Power UE CHTTL, China Unicom

[5] R4-1908721	 Increasing EN-DC power for FDD-TDD PC2 and PC3 Ericsson

[6] R4-1908862	 Discussion on different schemes for PC2 EN-DC (FDD+TDD) vivo

[7] R4-1909080	 About NSA FDD-TDD HPUE SAR OPPO

[8] R4-1909908 Further consideration on SAR solutions for FDD+TDD HPUE Huawei, HiSilicon

[9] R4-1910740	 About NSA FDD-TDD HPUE SAR solutions OPPO

[10] R4-1910812 further discussion on option3 for PC2 EN-DC (FDD+TDD) vivo

[11] R4-1911264 Discussion on the UE capabilibity solutions for EN-DC FDD-TDD High Power UE CHTTL

[12] R4-1911649 Discussion on NSA FDD-TDD HPUE SAR solutions	China Unicom

[13] R4-1911716 Discussion of HPUE ENDC FDD+TDD NTT DOCOMO, INC.

[14] R4-1912484 Comparison of options for FDD-TDD PC2 duty cycle capability signaling Qualcomm Incorporated

[15] R4-1912520 On SAR solutions for FDD+TDD HPUE Huawei, HiSilicon

- Text Proposal

<Start of TP>

- 9	Summary of the SI

The main target of the SI is to study the feasible solution to comply with the SAR requirement for two cases in the SI scope, i.e. case 1 for 23dBm LTE + 23dBm NR and case 2 for 23dBm LTE + 26dBm NR.

Base on discussion in RAN#93bis, the group prefers to report one duty cycle as UE capability in Rel-16. With this capability, network can help to guarantee the UE not exceeding the SAR limits at the UE maximum output power. The specific signalling design and the value range can be further considered during the WI stage. Meanwhile, the UE based solution should also be considered as the reported maximum duty cycle capability cannot always be guaranteed by the network scheduling. A combined UE based, and network based solution to better improve the UL performance is suggested in the SI study.

Required changes in TS 38.101-3 were also analysed during the SI study, the identified requirements shall be specified for specific FDD+TDD EN-DC band combinations in the following WI.

In the SI study, only one band combination, i.e. DC_3_n78 is proposed as an example combination to facilitate the study. Once the SI is finished, a WI will be proposed to standardize the SAR solution as well the specifying requirements for proposed band combinations. It is expected that more band combinations will be proposed in the WI stage, thus it is proposed to treat the FDD+TDD EN-DC HPUE as a basket WI.

<End of TP>