3GPP TSG-RAN WG4 Meeting #93												  	     		    R4-1914267

Reno, USA, 18 – 22 November, 2019

Source: 	Nokia, Nokia Shanghai Bell

Title: 					TP to TR 38.820: Conclusions on signal quality

Agenda item:			11.4.8.2

Document for:			Approval

# 1	Introduction

In this contribution conclusions for the signal quality requirements have been provided.

# 2	Discussion

Signal quality aspects were discussed in RAN4#92bis but text proposals to TR were not agreed. The proposed conclusions below are based on the contributions provided to previous meeting and content which has been already approved to the TR, only discussion on 256QAM has ben modified as in previous meeting it was agreed that 256QAM is feasible in FR2.

# 3	Conclusion

The text proposal below is proposed to be approved.

<Start of text proposal>

#### 7.4.1.1	Tx requirements overview

Summary of the conducted and radiated Tx requirements specified in Rel-15 for the NR BS is presented in this subclause. More detailed elaboration on the motivation on selected requirements is provided in dedicated subclauses below.

All the findings captured for the conducted requirements in table 7.4.1.1-1 and related subclauses below are considered to be applicable to the (sub)-range of the 7 – 24 GHz for which the conducted requirements will be found to be feasible during related WI.

While radiated requirements are considered to be applicable to the whole 7 – 24 GHz range, their definitions, values and levels may differ across the 7 – 24 GHz range.

Table 7.4.1.1-1: Overview of conducted Tx requirements for NR BS in 7 – 24 GHz range

| Tx requirement |  | Conclusions from SI | Items to be completed in related WI |
| --- | --- | --- | --- |
| Base station output power |  |  |  |
| Output power dynamics | RE power control dynamic range | If unwanted emission limits are not increased compared to FR1 limits, then the RE power control dynamic range requirements for QPSK and 16 QAM can be reused for the applicable 7-24 GHz (sub)-range. No requirement for 64QAM. | If unwanted emission limits are increased compared to FR1 limits, then the RE power control dynamic range may require to be revisited. Decide if requirement for 256QAM is needed. |
|  | Total power dynamic range | Same requirement derivation methodology as and FR1, based on the set of supported channel bandwidths and SCS. |  |
| Transmit ON/OFF power | Transmitter OFF power |  |  |
|  | Transmitter transient period |  |  |
| Transmitted signal quality | Frequency error | No difference to FR1 was observed. |  |
|  | Modulation quality | QPSK, 16QAM, 64QAM core requirements can be reused from FR1. | Define side conditions like EVM window length. Define 256QAM requirement in case FR1 and FR2 requirements don’t end up being the same. Otherwise also 256QAM requirement can be re-used. |
|  | Time alignment error | Requirements for MIMO transmission and for inter-band CA (with or without MIMO) can be reused from FR1 | Derive intra-band CA TAE requirements based on the set of supported SCS. |
| Occupied bandwidth |  | Single carrier and CA requirements can be reused from FR1/FR2. |  |
| ACLR |  |  |  |
| Operating band unwanted emissions |  |  |  |
| Transmitter spurious emissions | General transmitter spurious emissions |  |  |
|  | Protection of the BS receiver of own or different BS |  |  |
|  | Additional spurious emissions requirements |  |  |
|  | Co-location with other base stations |  |  |
| Transmitter intermodulation |  |  |  |

Table 7.4.1.1-2: Overview of radiated Tx requirements for NR BS in 7 – 24 GHz range

| OTA Tx requirement |  | Conclusions from SI | Items to be completed in related WI |
| --- | --- | --- | --- |
| Radiated transmit power |  |  |  |
| OTA base station output power |  |  |  |
| OTA output power dynamics | RE power control dynamic range | For “FR1-like” sub-range of the 7 – 24 GHz range, the requirement for QPSK and 16QAM is the same as conduced requirement for 7 – 24 GHz. For “FR2-like” sub-range of the 7 – 24 GHz range, there is no requirement defined. No requirement for 64QAM. | If unwanted emission limits are increased compared to FR1 limits, then the RE power control dynamic range may require to be revisited. Decide if requirement for 256QAM is needed. |
|  | Total power dynamic range | Same requirement derivation methodology as and FR1, based on the set of supported channel bandwidths and SCS. |  |
| OTA transmit ON/OFF power | Transmitter OFF power |  |  |
|  | Transmitter transient period |  |  |
| OTA transmitted signal quality | Frequency error | No difference to FR1 and FR2 was observed. |  |
|  | Modulation quality | QPSK, 16QAM, 64QAM core requirements can be reused from FR1/FR2. | Define side conditions like EVM window length. Define 256QAM requirement in case FR1 and FR2 requirements don’t end up being the same. Otherwise also 256QAM requirement can be re-used. |
|  | Time alignment error | Requirements for MIMO transmission and for inter-band CA (with or without MIMO) can be reused from FR1/FR2 | Derive intra-band CA TAE requirements based on the set of supported SCS. |
| OTA occupied bandwidth |  | Single carrier and CA requirements can be reused from FR1/FR2. |  |
| OTA ACLR |  |  |  |
| OTA operating band unwanted emissions |  |  |  |
| OTA transmitter spurious emissions | General transmitter spurious emissions |  |  |
|  | Protection of the BS receiver of own or different BS |  |  |
|  | Additional spurious emissions requirements |  |  |
|  | Co-location with other base stations |  |  |
| OTA transmitter intermodulation |  |  |  |

#### 7.4.1.2	Output power accuracy

<Unchanged sections omitted>

#### 7.4.1.5	Transmitted signal quality

##### 7.4.1.5.1	Frequency error

Both conducted and radiated NR BS frequency error core requirements are the same for FR1 and FR2. Therefore, it is expected that same requirements can be re-used for 7-24 GHz frequencies.

##### 7.4.1.5.2	EVM

Both conducted and radiated NR BS EVM core requirements with QPSK to 64QAM requirements have been defined for FR1 and FR2 with the same requirement values. Therefore, it is expected the same requirement values can be re-used for 7-24 GHz frequencies. Some aspects like EVM window length differ between FR1 and FR2 and need further consideration for 7-24 GHz range.

##### 256QAM requirement is defined for FR1 and current status (November 2019) is that 256QAM has been concluded to be feasible in FR2 but requirement work has not yet finished. Therefore, it can be expected that requirements for 256QAM will be specified also for 7-24 GHz frequencies while the re-use of actual requirement values depend on whether the FR2 requirement will end up being the same as for FR1.

##### 7.4.1.5.2	TAE

#### TAE requirements for MIMO and inter-band CA with or without MIMO are the same for FR1 and FR2 and therefore it is expected they can be re-used also for 7-24 GHz frequencies. For intra-band CA TAE differs between FR1 and FR2 due to difference in supported SCS. Intra-band CA TAE can be derived once the set of supported SCS is known for 7-24 GHz frequencies.

#### 7.4.1.6	ACLR

<End of text proposal>