3GPP TSG-RAN WG4 Meeting #92				  												R4-1914328

Reno, USA, 18th Nov. 2019 – 22nd Oct. 2019

Source: 	Nokia, AT&T

Title: 					TP for 37.716-21-11 to introduce DC_2-12_n2

Agenda Item: 			10.4.2 [DC_R16_2BLTE_1BNR_3DL2UL-Core]

Document for:	Approval

# 1	Introduction

This contribution is a TP for TR 37.716-21-11 (v0.7.0) [R4-1910871] to introduce DC_2-12_n2 .

# 2	Text Proposal

*********************** Start of the TP ***************************************************

## 5.1.x	DC_2-12_n2

### 5.1.x.1	Operating bands for DC

Table 5.1.x.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-12_n2 | CA_2-12 | n2 | No |

### 5.1.x.2	Configuration for DC

Table 5.1.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-12A_n2A | DC_12A_n2A | CA_2A-12A | n2A |

### 5.1.x.3	Coexistence studies

Based on co-existence studies of DC_12A_n2A in TR 37.716-01-01, there is no harmonic or IMD impact to any of own Rx bands.

### 5.1.x.4	∆TIB and ∆RIB values

For DC_2-12_n2, the TIB,c and RIB,c values are derived from CA_2-2-12 in TS 36.101.

Table 5.X.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-12_n2 | 2 | 0.3 |
|  | 12 | 0.3 |
|  | n2 | 0.3 |

Table 5.X.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-12_n2 | 2 | 0 |
|  | 12 | 0 |
|  | n2 | 0 |

### 5.1.x.5	REFSENS requirements

There is no additional requirement for this band combination.

*********************** End of the TP ***************************************************

# 3	References