3GPP TSG-RAN WG4 Meeting #93                 	R4-1914490

Reno, USA, 18th – 22nd November 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM requirements for IAB

Agenda item:	9.5.5.1

Document for:	Discussion

Introduction

RAN4#92bis discussed the need for IAB RRM requirements and some progress was made and captured in [3]. In this paper we recap the IAB behaviour during the integration (e.g. power on) and during the IAB active operation. We look at the FFS parts in the [3] and discuss which IAB specific requirements would need to be specified for Rel-16 IAB’s.

Discussion

IAB extensions for SSB TX and RX

RAN1 has agreed on SSB TX and RX configurations that support inter node discovery and measurements between nodes that are already serving UEs and/or providing backhaul connection to their child nodes. An SSB TX configuration, STC, includes SSB center frequency, SCS, STC periodicity (from 5 up to 640ms), timing offset, and indexes of SSBs to transmit. An SSB measurement timing configuration, SMTC, includes periodicity (from 5 up to 1280ms), timing offset, duration of the measurement window, and SSB indexes and PCIs to measure. Up to four STCs and four SMTCs can be configured per frequency. These numbers allow sufficiently many TX-RX patterns for performing detection and measurements under the half duplex constraint that DU TX cannot overlap in time with MT RX. With the maximum number of STCs and SMTCs there are 84=70 patterns allowing 1-4 measurement opportunities (per period) between nodes allocated with different patterns.

Figure 1 shows an example of six TX-RX patterns that are obtained by configuring two STCs and two SMTCs per node. In addition to the STCs and SMTCs configured for measurements between already serving IAB nodes, Figure 1 shows also STCs for access UEs and IAB-MT cell selection discussed in Section 2.2.

Figure 1: An example of SSB TX and RX patterns. The figure displays the half-frames with SSBs transmitted for access UEs (and IAB initial access) with 20ms periodicity and the half-frames corresponding to STCs and SMTCs configured with 80ms periodicity for measurements between IAB nodes.

IAB cell Change

In last meeting RAN4 had further discussions related to RRM requirements for IAB. It was agreed to introduce requirements for RRC Connection Mobility control while the need for handover requirements was left FFS.

Connected state RRM Measurements and ‘Mobility’

In general, Rel-16 IAB does not support mobile IAB’s and this significantly reduces the need to RRM requirements related to mobility control.

RRM Measurement Requirements

In Rel-16 the IAB is assumed fixed in location. Hence, it can be assumed that link between the DU and MT is considered very stable compared the general assumptions used when defining RAN4 requirements for mobile devices. As the IABs will not move there is no justification to define tight RRM requirements as known from the existing RAN4 requirements.

RAN4 does not define RRM measurement requirements (timing requirements for cell detection, Index reading, measurement period, number of cells etc.) for Rel-16 IAB.

As mobility is not supported, we do not see a need to define measurement reporting requirements for Rel-16 IAB. The need for connected state inter-IAB node measurements depends on the IAB network deployment (topology), resource allocation using TDM, SDM and/or FDM, SSB TX configuration and SMTC configuration(s) with configured RX/TX patterns for inter-IAB measurements, number of antenna panels, number of beams per panel, etc.

There are multiple deployment parameters, some of which IAB specific, affecting the need for IAB RRM measurements.

Even though IAB mobility is not supported in Rel-16, there may be changes in the deployment environment which may lead to the situation that there is a need to update the currently used BH link. E.g. the BH is no longer available or no longer the most optimal BH link to be used. One example could be in mmvw, in case the BH connection between IAB1 is blocked, there would be need for action to recover or change the BH link. Another example is a BH link change due to load balancing where the connection is changed from very loaded parent node to a node with unused capacity.

However, such changes would happen less frequent and would not be due to the IAB mobility but due to changes in the deployment environment. Hence, due to the slowness in the changes there would be much longer period available for performing any RRM measurements (in a stable environment) leading to long measurement periods. Therefore, we do not see a need to define RRM measurement accuracy.

RAN4 does not define RRM measurement accuracy requirements for Rel-16 IAB.

Additionally, it should be observed, that with the current RAN4 assumptions and requirements (for UE’s) RRM measurements is likely to impact the throughput in FR2 due to sweeping (Mpss/sss_sync_w/o_gaps and Mmeas_period_w/o_gaps). Hence, similar impact will likely be seen on the IAB BH.

IAB and Handover

IAB topology adaptation could use redirection or NR handover procedures as the baseline. It was agreed to introduce requirements for release with re-direction and next is to discuss whether IAB requirements for handover delay should also be introduced. Handover delay requirement include:

- Handover delay

- Interruption time

The handover delay Dhandover equals the maximum RRC procedure delay to be defined in clause 12 in TS 38.331 plus the interruption.

The delay from the RRC procedure is specified in 38.331. The rest of the delay depend on the interruption time, see below.

The definition of the NR HO interruption time is, [2]:

Tinterrupt = Tsearch + TIU + Tprocessing+ T∆ ms

where:

- Tsearch is the time required to search the target cell when the handover command is received

- TIU is the interruption uncertainty in acquiring the first available PRACH occasion

- Tprocessing is the time for UE processing

- T∆ is time for fine time tracking and acquiring full timing information of the target cell.

Considering the inter-IAB discovery and measurements, we may assume that the blind handover can be used in case candidate nodes are not already known at the time when HO command for topology adaptation is requested by DU and received MT. As Handovers can be needed to enable topology adaptation and it can be assumed that such adaptation is not frequent and most likely not time critical.

The IAB-MT handover interrupt time requirement could be introduced based on existing handover interrupt time requirements independently from support of RRM measurements and reporting requirements.

Tsearch for blind HO may not be zero, however the current conditions could re-used. I.e. if the HO is topology or load based with network knowledge about available target, the search time can be assumed being limited. Assuming normal access starting with RACH to the target node, TIU will depend on the configured PRACH occasions in the target cell. Tprocessing will be IAB MT specific possibly deviating the values specified for the UE and should therefore be discussed further. Similar to Tsearch, the fine time tracking could potentially be reduced as the IAB’s are assumed stationary in Rel-16. Therefore, T∆ could be assumed to be close to zero.

The HO interruption time from existing UE requirements could be re-used as baseline for defining HO interrupt time in IAB while the IAB specific latency values would need more discussion.

IAB and Link Control

RAN4 discussed the need for Link control in terms of RLM/RLF and BFD/LR in last RAN4#92bis meeting. Although we do not expect that Rel-16 IAB will need RRM measurement requirements and accuracy requirements for mobility support, including measurement reporting from MT to DU, we believe that there may be a need to have link monitoring and link control to handle possible link connection errors.

RLM and RLF for IAB

We do not expect that the connection between the MT and the serving DU in IAB will deteriorate such that the MT would experience RLF. However, to ensure that the MT will act in the situation where the BH connection does in fact ‘disappear’ e.g. due to an unexpected failure, it would likely be best to introduce RLF related actions in IAB.

RAN4 could introduce RLM requirements in IAB.

Hence, once the MT experience that the DL signal quality gets bad the MT will initiate search for another DU to serve the MT. It is expected that the existing RLF procedure could be re-used. I.e. once RLF is detected the MT will perform cell selection. The detailed requirements for the RLM would need to be discussed further. We would however expect that the requirements would be a simplified version of the current UE requirements. Additionally, RAN4 would need to discuss scheduling availability during RLM as this could impact the BH throughput.

When defining any RLM/RLF IAB requirements RAN4 need to consider the impact on the BH throughput due to related RLM requirements.

IAB and Link Recovery

Especially in mmvw it is expected that in certain deployment scenarios, there could be conditions which would e.g. lead to blockage. For this, we believe it is beneficial to define some minimum Link Recovery requirements. In principle it enough to define requirements for link failure detection and recovery.

Link Recovery procedure, in terms of link failure handling, should be supported in IAB Rel-16.

Possibly the current UE defined frame work could be used as baseline for defining the IAB requirements. However, the IAB requirements should consider the difference in the assumed environment when defining the requirements. Hence, the IAB link is assumed to be static and good under normal conditions. Additionally, the IAB nodes are not moving and the link is static and under normal conditions it would not change significantly in a short time period. RAN4 would need to consider these facts when defining the link recovery procedures (if agreed to be introduced).

IAB and TCI switching

As IAB Rel-16 does not consider IAB mobility it is expected that the link will remain stable for longer periods of time and there might not be a need to switch TCI state. This of course depends on the deployment scenario, but it must be expected that Rel-16 IAB is likely mostly suitable for deployments in stable conditions where the IAB link is expected not to change often if at all. Based on this we do not see a strong need to introduce TCI state switching requirements for IAB in Rel-16.

RAN4 does not define TCI state switching delay requirements for Rel-16 IAB.

To manage potential error cases where the BH link break for unforeseen reasons, we believe this should be handled through link recovery procedures.

Introduction of TCI state switching would in our view also lead to a need to define L1-RSRP measurement requirements, L1-RSRP measurement accuracy and L1-RSRP reporting requirements.

IAB Connection Control Requirements

In RAN4#92bis it was agreed to introduce RRC Connection Control requirements including RRC-re-establishment and RRC release with redirection. Additionally, it was agreed to introduce requirements for Random access. Following we look at how we see such requirements could be defined for IAB using the existing UE requirements as baseline.

IAB and RRC re-establishment

In [2] the re-establish delay for the UE was defined as the capability of sending RRCReestablishmentRequest message within Tre-establish_delay seconds from the moment it detects a loss in RRC connection. The total RRC connection delay (Tre-establish_delay) shall be less than:

Tre-establishdelay=TUEre-establishdelay+TULgrant

TUL_grant: It is the time required to acquire and process uplink grant from the target PCell. The uplink grant is required to transmit RRCReestablishmentRequest message.

Further, [2] says that the UE re-establishment delay (TUE_re-establish_delay) is the time between the moment of detecting condition requiring RRC re-establishment and the time when the PRACH is sent to the target cell. The UE re-establishment delay (TUE_re-establish_delay) requirement shall be less than:

TUEre-establishdelay=50+TidentifyintraNR+i=1Nfreq-1TidentifyinterNR,i+TSI-NR+TPRACH

where:

Tidentify_intra_NR is the time to identify the target intra-frequency NR cell

Tidentify_inter_NR,i is the time to identify the target inter-frequency NR cell on inter-frequency carrier

TSI-NR  is the time required for receiving all the relevant system information

TPRACH is the delay caused due to the random access procedure

Among these parameters, all could be directly re-used for static IAB deployment also without need for RRM measurements and reporting requirements. Provided that the inter-IAB node measurements are configured properly and carried out during the normal IAB operation the delays for identifying the target cell is considering both known and unknown target cells. The constant value (50) in the above equation is UE specific and it is FFS how to define that for the IAB node.

The delay due to RACH depends on the IAB configuration and RACH related parameters similarly as during the access to the target cell in the HO procedure.

Existing re-establishment delay requirements can be used as baseline for defining IAB re-establishment delay requirements. RAN4 would need to discuss assumptions and latencies further.

Release with redirection

Current requirements for release with direction are as follows:

The UE shall be capable of performing the RRC connection release with redirection to the target NR cell within Tconnection_release_redirect_NR.

The time delay (Tconnection_release_redirect_NR) is the time between the end of the last slot containing the RRC command, “RRCRelease” (TS 38.331 [2]) on the NR PDSCH and the time the UE starts to send random access to the target NR cell. The time delay (Tconnection_release_redirect_NR) shall be less than:

Tconnection_release_redirect_NR = TRRC_procedure_delay + Tidentify-NR + TSI-NR + TRACH

The target NR cell shall be considered detetable when for each relevant SSB, the side conditions should be met that,

- the conditions of SSB_RP and SSB Ês/Iot according to Annex B.2.5 for a corresponding NR Band are fulfilled.

TRRC_procedure_delay: It is the RRC procedure delay for processing the received message “RRCRelease” as defined in clause 6.2.2 of TS 38.331 [2].

Tidentify-NR: It is the time to identify the target NR cell and depends on the frequency range (FR) of the target NR cell. It is defined in Table 6.2.3.2.1-1. Note that Tidentify-NR = TPSS/SSS-sync + Tmeas, in which TPSS/SSS-sync is the cell search time and Tmeas is the measurement time due to cell selection criteria evaluation.

TSI-NR: It is the time required for acquiring all the relevant system information of the target NR cell. This time depends upon whether the UE is provided with the relevant system information of the target NR cell or not by the old NR cell before the RRC connection is released.

TRACH: It is the delay caused due to the random access procedure when sending random access to the target NR cell. This delay depends on the PRACH configuration defined in Table 6.3.3.2-2 [6] or Table 6.3.3.2-3 [6] for FR1 and in Table 6.3.3.2-4 [6] for FR2.

Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cell in the redirection command, otherwise Trs is the SMTC periodicity configured in the measObjectNR having the same SSB frequency and subcarrier spacing configured for the RRC connection release with redirection. If the UE is not provided with SMTC configuration or measurement object for the frequency which is also configured for the RRC connection release with redirection then:

- the requirement in this section is applied with Trs = 20 ms if the SSB transmission periodicity is not larger than 20 ms; otherwise,

- there is no requirement if the SSB transmission periodicity is larger than 20ms.

In general, it should be possible to re-use the frame work defined for the UE requirements also for IAB. RAN4 would need to discuss the assumptions and actual delay numbers further.

Existing release with redirection delay requirements can be used as frame work for defining release with redirection delay requirements for IAB. RAN4 would need to discuss assumptions and latencies further.

Random access

As the DU should be able to serve both MT and access UEs, the IAB requirements for Random access should be the same as existing random access requirements defined for UE’s.

Requirements for Random access for IAB re-use the same requirements as existing UE Random access requirements.

Handover

If it is decided to introduce handover delay requirements for IAB, section 2.3.2 shows how this could be done using the existing handover requirements. The current handover requirements are generic and covers both known and unknown target cell. Hence, also blind handover is by the requirements in the Tsearch delay by allowing device more time to search for target cell under certain conditions. Therefore, one approach is to re-use existing requirements.

RAN4 would need to discuss the actual latencies further.

Existing handover delay requirements can be used as baseline for defining IAB handover delay requirements. RAN4 would need to discuss assumptions and latencies further

Conclusion

RAN4#92 discussed the need for IAB SSB based RRM requirements with agreements and open items listed in the agreed WF [3]. In this paper we analysed the IAB behaviour during the integration and during the IAB active operation, and at the FFS parts in the [3] and discuss which IAB specific requirements would need to be specified for Rel-16 IAB’s:

Proposal 1: RAN4 does not define RRM measurement requirements (timing requirements for cell detection, Index reading, measurement period, number of cells etc.) for Rel-16 IAB.

Proposal 2: RAN4 does not define RRM measurement accuracy requirements for Rel-16 IAB.

Proposal 3: The IAB-MT handover interrupt time requirement could be introduced based on existing handover interrupt time requirements independently from support of RRM measurements and reporting requirements.

Proposal 4: The HO interruption time from existing UE requirements could be re-used as baseline for defining HO interrupt time in IAB while the IAB specific latency values would need more discussion.

Proposal 5: RAN4 could introduce RLM requirements in IAB.

Proposal 6: Link Recovery procedure, in terms of link failure handling, should be supported in IAB Rel-16.

- Proposal 7: RAN4 does not define TCI state switching delay requirements for Rel-16 IAB.

Proposal 8: Existing re-establishment delay requirements can be used as baseline for defining IAB re-establishment delay requirements. RAN4 would need to discuss assumptions and latencies further.

Proposal 9: Existing release with redirection delay requirements can be used as frame work for defining release with redirection delay requirements for IAB. RAN4 would need to discuss assumptions and latencies further.

- Requirements for Random access for IAB re-use the same requirements as existing UE Random access requirements.

- Existing handover delay requirements can be used as baseline for defining IAB handover delay requirements. RAN4 would need to discuss assumptions and latencies further

- References

- Draft RAN4 #92 Meeting report

- TS 38.133 NR; Requirements for support of radio resource management

- R4-1912829, Updated WF on IAB RRM Requirements, Qualcomm

- R4-1911794, SSB based RRM requirements for IAB, Nokia, Nokia Shanghai Bell