3GPP TSG-RAN4 Meeting #93	R4-1914492

# Reno, USA, 18th – 22nd November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0212 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to clarify and update QCL applicability (section 3.6) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In last RAN3 meeting 92bis the draft CR in R4-1912769 relsted to adding QCL applicability in 38.133 was endorsed. The CR introduced the term TCI chain which is not clearly defined in the new section and the concept of TCI chain is not defined in RAN1 either. This leads to unclear specification and risk of misaligned specifications between RAN4 and RAN1. This CR corrects the unclarity introduced by introducing the use of TCI chain wording in 38.133. Clarification is done by making referring to QCL chain instead of TCI chain and refer to the already defined QCL rules (and QCL chains) as defined in the RAN1 specification 38.214 section 5.1.5. This also ensures the basic principle that RAN4 specification shall not define or specify what is already defined or specified in antoher specification (here RAN1 38.214) Additionally, last line is removed as this is already specified in RAN1 and shall not be dublicated in RAN4 specification due to maintanence reasons. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification and update to applicability of QCL. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear how to determine two RS-es or one channel and one RS are QCL-ed with each other. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Next Modified Subclause

3.6.7	Applicability of QCL

For the requirements specified in this version of the specification, the total number of associated TCI states for the reception of a reference signal can be at most 4. This applies to reference signals as specificed in TS 38.214, sect 5.1.5, e.g. DMRS of PDCCH, DMRS of PDSCH, CSI-RS for tracking.

Next Modified Subclause