3GPP TSG RAN WG4 Meeting #93	R4-1914670

Reno, US, November 18-22, 2019

Agenda Item:	9.1.1.5

Source:	Futurewei

Title:	Sync raster design for NR-U

Document for:	Approval

# Introduction

In RAN4#92Bis, WF [1] suggested several synchronization raster locations based on SSB being located close to the edge of the sub-band. Suggested raster locations are provided while considering the sub-bands as well as two draft CRs [4][5].

# Discussion

## Background

In RAN4#92bis, the agreement in [1] captured

Considering a frequency offset of minimum 3 full RBs at 30kHz SCS to account for intra-carrier guardbands for each sub-band, define the following GSCN points:

GSCN = {[8996], 9010, 9024, [9037/9038], 9051, 9065, 9079, 9093, 9107, [9121], [9218], 9232, 9246, 9260, 9274, 9287, 9301, 9315, 9329, 9343, 9357, [9371], 9385, 9402, 9416, 9430, 9444, 9458, [9471], [9485], [9499], [9513]}

Proposed values are conditioned on NR minimum channel guard bands

If it is later found that larger guard bands are needed, it is common understanding that some of the SSB points may need to be changed

## Sync raster background

As captured in subclause 4.3.1.5 of [2], the first possible location of the SSB in a band is given by

|  | fmin=flow+G+∆fNRBSSNRERB+12 | (1) |
| --- | --- | --- |

where G is the guard band for that bandwidth and subcarrier spacing, ∆f is the subcarrier spacing, NRERB=12 is the number of REs per RB, NRBSS=20 is the number of RBs in the SS block, and flow is the lowest frequency in the band / channel. For discussion purposes, table 5.3.3-1 from TS 38.104 is presented here for indicating possible values of G.

Table 5.3.3-1: Minimum guardband (kHz) (FR1)

| SCS (kHz) | 10 MHz | 20 MHz | 40 MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 30 | 665 | 805 | 905 | 825 | 925 | 845 |

The GSCN for frequencies above 3 GHz is given by

|  | N=fmin-Flow∆Fraster+7499 | (2) |
| --- | --- | --- |

where ∆Fraster=1.44 MHz, fmin is in MHz, and Flow=3000 MHz.

Using the guard band for 80 MHz (925 kHz) for 30 kHz SCS, the raster locations are shown in Table 1 where the column difference is obtained by

|  | N-7499∆Fraster+3000-∆fNRBSSNRERB2-flow | (3) |
| --- | --- | --- |

Note because the sync raster points to the center of SSB, the removal of the term 12∆fNRBSSNRERB is needed to locate the first RE of the SSB.

Table 1. Sync raster locations based on minimum guard band of 925 kHz using 30 kHz SCS. The green highlight shows agreement to WF. The yellow highlighting identifies SSBs whose first RE is less than 1.2 MHz from sub-band edge.

| Sub-band edge, MHz | GSCN | Difference, kHz | Sub-band edge, MHz | GSCN | Difference, kHz |
| --- | --- | --- | --- | --- | --- |
| 5150 | 8996 | 2080 | 5590 | 9301 | 1280 |
| 5170 | 9010 | 2240 | 5610 | 9315 | 1440 |
| 5190 | 9023 | 960 | 5630 | 9329 | 1600 |
| 5210 | 9037 | 1120 | 5650 | 9343 | 1760 |
| 5230 | 9051 | 1280 | 5670 | 9357 | 1920 |
| 5250 | 9065 | 1440 | 5690 | 9371 | 2080 |
| 5270 | 9079 | 1600 | 5710 | 9385 | 2240 |
| 5290 | 9093 | 1760 | 5735 | 9402 | 1720 |
| 5310 | 9107 | 1920 | 5755 | 9416 | 1880 |
| 5330 | 9121 | 2080 | 5775 | 9430 | 2040 |
| 5470 | 9218 | 1760 | 5795 | 9444 | 2200 |
| 5490 | 9232 | 1920 | 5815 | 9458 | 2360 |
| 5510 | 9246 | 2080 | 5835 | 9471 | 1080 |
| 5530 | 9260 | 2240 | 5855 | 9485 | 1240 |
| 5550 | 9273 | 960 | 5875 | 9499 | 1400 |
| 5570 | 9287 | 1120 | 5895 | 9513 | 1560 |

3 RBs corresponds to 3∆fNRERB=1080 kHz. There are 2 sub-bands whose first RE of the SSB that is less than 1000 kHz, 1 sub-band between 1000 and 1100 kHz, 2 sub-bands between 1.1 and 1.2 MHz from sub-band edge. However only three are in disagreement from the WF.

## 3 RB minimum

Eq. (1) can be generalized to maximum of 3 RB and guard

|  | fmin=flow+maxG,3∆fNRERB+∆fNRBSSNRERB+12 | (4) |
| --- | --- | --- |

It is expected that only 2 values (those whose difference is less than 1080 kHz in Table 1) would be affected by the generalization, and it is confirmed in Table 2.

Table 2. Sync raster locations based on minimum guard band of 3 RBs. The pink highlighting indicates the changed GSCN value from Table 2.

| Sub-band, MHz | GSCN | Difference, kHz | Sub-band, MHz | GSCN | Difference, kHz |
| --- | --- | --- | --- | --- | --- |
| 5150 | 8996 | 2080 | 5590 | 9301 | 1280 |
| 5170 | 9010 | 2240 | 5610 | 9315 | 1440 |
| 5190 | 9024 | 2400 | 5630 | 9329 | 1600 |
| 5210 | 9037 | 1120 | 5650 | 9343 | 1760 |
| 5230 | 9051 | 1280 | 5670 | 9357 | 1920 |
| 5250 | 9065 | 1440 | 5690 | 9371 | 2080 |
| 5270 | 9079 | 1600 | 5710 | 9385 | 2240 |
| 5290 | 9093 | 1760 | 5735 | 9402 | 1720 |
| 5310 | 9107 | 1920 | 5755 | 9416 | 1880 |
| 5330 | 9121 | 2080 | 5775 | 9430 | 2040 |
| 5470 | 9218 | 1760 | 5795 | 9444 | 2200 |
| 5490 | 9232 | 1920 | 5815 | 9458 | 2360 |
| 5510 | 9246 | 2080 | 5835 | 9471 | 1080 |
| 5530 | 9260 | 2240 | 5855 | 9485 | 1240 |
| 5550 | 9274 | 2400 | 5875 | 9499 | 1400 |
| 5570 | 9287 | 1120 | 5895 | 9513 | 1560 |

Proposal 1: Sync raster location based on 3 RBs for guard can be used for the 20 MHz sub-bands.

The values to use are

{8996, 9010, 9024, 9037, 9051, 9065, 9079, 9093, 9107, 9121, 9218, 9232, 9246, 9260, 9274, 9287, 9301, 9315, 9329, 9343, 9357, 9371, 9385, 9402, 9416, 9430, 9444, 9458, 9471, 9485, 9499, 9513}

## 10 MHz sub-band

In WF [3], there are a proposal to introduce 10 MHz sub-bands for certain regions (group 1). However, the applicability of the 30 kHz SCS for SSB agreement may need to be re-examined for the 10 MHz sub-band because only 24 RBs are available. Using the spreadsheet in [2], the sync raster locations are presented in Table 3.

Table 3. GSCN for 10 MHz sub-bands

| Sub-band center freq., MHz | Sub-band lower freq., MHz | Sub-band upper freq., MHz | GSCN | Location of first RB SSB, kHz | Location of last RB SSB, kHz |
| --- | --- | --- | --- | --- | --- |
| 5730 | 5725 | 5735 | 9395 | 1640 | 1160 |
| 5830 | 5825 | 5835 | 9464 | 1000 | 1800 |

Because there is only one sync raster location possible in the 10 MHz sub-band, the minimum 3 RB does not apply to the sub-band whose center frequency is located at 5830 MHz. Note that the 925 kHz guard-band can be supported.

Proposal 2: The GSCN for the 10 MHz sub-band should be captured in the standards.

The values to use are {9395, 9464}.

# Conclusions

The contribution confirms the values in the WF [1].

Proposal 1: Sync raster location based on 3 RBs for guard can be used for the 20 MHz sub-bands.

The values to use are

{8996, 9010, 9024, 9037, 9051, 9065, 9079, 9093, 9107, 9121, 9218, 9232, 9246, 9260, 9274, 9287, 9301, 9315, 9329, 9343, 9357, 9371, 9385, 9402, 9416, 9430, 9444, 9458, 9471, 9485, 9499, 9513}

Proposal 2: The GSCN for the 10 MHz sub-band should be captured in the standards.

The values to use are {9395, 9464}.

# References

R4-1912982, “WF for sync raster for NR-U”, Qualcomm, RAN4#92B, Oct. 14-18 2019

TR 38.817-01

R4-1912870, “WF on the NR-U channel raster on 5GHz band”, Ericsson, RAN4#92B, Oct. 14-18, 2019

R4-1914671, “Draft CR for Sync raster design for NR-U in 38.104”, Futurewei, RAN4#93, Nov. 18-22, 2019.

R4-1914672, “Draft CR for Sync raster design for NR-U in 38.101-1”, Futurewei, RAN4#93, Nov. 18-22, 2019.