3GPP TSG-RAN4 Meeting #93	R4-1914951

# Reno, US, 18th – 22nd November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0352 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on direct SCell activation delay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  | Date: |  |  | 2019-10-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN2 has agreed to define direct SCell activation in NR. Corresponding delay requirements should be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define delay requirements for direct SCell activation at SCell addition and at HO. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No delay requriement for direct SCell activation in NR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 8.3.x and 8.3.y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

### 8.3.x	Direct SCell Activation at SCell addition

The requirements in this section apply for UE being configured in the RRC reconfiguration message [2] with one SCell for which the parameter sCellState is set to activated.

Editor’s Note: FFS for the direct activation requriements for multiple SCells.

The UE shall configure the SCell in activated state upon successful completion of the RRC reconfiguration procedure within the specified delay. Upon receiving the RRC reconfiguration message in subframe n, the UE shall be capable to transmit valid CSI report and apply actions for the directly activated SCell no later than in slot +NdirectNR slot length ,

Where:

Ndirect = TRRC_Process + T1 + Tactivation_time + TCSI_Reporting

TRRC_Process: RRC procedure delay defined in section 12 of TS 38.331 [2],

T1: Delay from slot n+TRRC_ProcessNR slot length until the transmission of RRCConnectionReconfigurationComplete message,

Note: T1 is UE implementation dependent.

Tactivation_time and TCSI_Reporting are specified in section 8.3.2.

In addition to CSI reporting defined above, UE shall also apply other actions related to the activation command specified in [7] for an SCell at the first opportunities for the corresponding actions once the SCell is activated.

During the direct SCell activation, interruption on PCell, PSCell and/or any activated SCell shall not occur before slot n+TRRC_Process + T1NR slot length and not after slot n+TRRC_Process+ T1+3+TSMTC_MAX+TSMTC_durationNR slot length, where TSMTC_MAX and TSMTC_duration are specified in section 8.3.2.

Starting from the slot n+TRRC_Process + T1NR slot length until the UE has completed the direct SCell activation, the UE shall report CQI index = 0 (out of range) if the UE has available uplink resources to report CQI for the SCell.

### 8.3.y	Direct SCell Activation at Handover

The requirements in this section apply for UE being configured in the RRC reconfiguration message [2] for handover with one SCell for which the parameter sCellState is set to activated.

Editor’s Note: FFS for the direct activation requriements for multiple SCells.

The UE shall configure the SCell in activated state upon successful completion of the RRC reconfiguration procedure within the specified delay. Upon receiving the RRC reconfiguration message in subframe n, the UE shall be capable to transmit valid CSI report and apply actions for the directly activated SCell no later than in slot +NdirectNR slot length ,

Where:

Ndirect = TRRC_process + Tinterrupt + T2 + T3 + Tactivation_time + TCSI_Reporting

TRRC_Process: RRC procedure delay defined in section 12 of TS 38.331 [2],

Tinterrupt: Interruption time during hanover as specified in section 6.1.1,

T2: Delay from slot n+TRRC_Process+TinterruptNR slot length until UE has obtained a valid TA command for the target PCell,

T3: Delay for applying the received TA for upling transmission in the target PCell, and greater than or equal to k+1 slot, where k is defined in clause 4.2 in TS 38.213,

Tactivation_time and TCSI_Reporting are specified in section 8.3.2.

In addition to CSI reporting defined above, UE shall also apply other actions related to the activation command specified in [7] for an SCell at the first opportunities for the corresponding actions once the SCell is activated.

During the direct SCell activation, interruption on PCell shall not occur before slot n+TRRC_Process + Tinterrupt+T2+ T3NR slot length and not after slot n+TRRC_Process + Tinterrupt+T2+ T3+3+TSMTC_MAX+TSMTC_durationNR slot length, where TSMTC_MAX and TSMTC_duration are specified in section 8.3.2.

Starting from the slot n+TRRC_Process + Tinterrupt+T2+ T3NR slot length and until the UE has completed the direct SCell activation, the UE shall report CQI index = 0 (out of range) if the UE has available uplink resources to report CQI for the SCell.

<End of Change 1>