3GPP TSG-RAN WG4 Meeting #93		 	R4-1914981

Reno, Nevada, 18th – 22nd Nov 2019

Title:	Reply LS on Applicable Timing for Pathloss RS Activated/Updated by MAC-CE

Response to:	R1-1911616 (R4-1913104)

Release:	Rel-16

Work Item:	NR_eMIMO-Core

Source:	RAN WG4

To:	RAN WG1

Cc:	-

Contact Person:

#### Name:	Nazmul Islam

E-mail Address:	mislam@qti.qualcomm.com

Attachments:	N/A

1. Overall Description:

RAN4 would like to thank RAN1 for the LS on applicable timing for path loss RS (PL RS) activated/updated by MAC-CE.

RAN4 has made following agreements in this regard:

- If the TCI state of the newly activated/updated PL RS is known, RAN4 agrees that filtered RSRP value for previous pathloss RS will be used before the application time, which is the next slot after the 5th measurement sample, where the 1st measurement sample corresponds to be the 1st instance, 3ms after sending ACK for the MAC CE.

- If the TCI state of the newly activated/updated PL RS is unknown, filtered RSRP value for previous pathloss RS will be used before the application time, which is the next slot after n+ THARQ +3 ms + TL1-RSRP + Tfifth-SSB.

Where T L1-RSRP  is the time for L1-RSRP measurement for Rx beam refinement, defined as

  - TL1-RSPR_Measurement_Period_SSB for SSB as specified in clause 9.5.4.1 of 38.133,

  - with the assumption of M=1

  - with TReport = 0

TL1-RSRP_Measurement_Period_CSI-RS for CSI-RS as specified in clause 9.5.4.2 of 38.133

- with the assumption of M=1 for periodic CSI-RS

- for aperiodic CSI-RS if number of resources in resource set at least equal to MaxNumberRxBeam

- with TReport = 0

Tfifth-SSB is the time to the fifth measurement sample after n+ THARQ +3 ms + TL1-RSRP.

RAN4 would like to inform RAN1 that section 8.3.2 of 38.133 describes the conditions for a TCI state to be known.

RAN4 has also agreed not to identify any RAN4 impact due to the mechanism of updating PL RS for PUSCH/SRS via MAC-CE. Hence, the applicable timing for activated/updated PL RS should be captured in RAN1 spec.

2. Actions:

To RAN WG1.

ACTION: 	RAN4 respectfully asks RAN1 to take the above information into account in the future work.

3. References:

[1] R1-1911616, “LS on Applicable Timing for Pathloss RS Activated/Updated by MAC-CE”, RAN1#98bis.

4. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #94		Feb. 24	– 28, 2020			Athens, Greece

TSG RAN WG4 Meeting #94-Bis		April 20 – 24, 2020			China