3GPP TSG-RAN WG4 Meeting #93	R4-1915195

Reno, NV, USA, 18 – 22 November, 2019

Agenda item:	8.11.3.3

Source: 	Qualcomm Incorporated

Title: 	Remaining issues on RRM measurements in non-anchor carrier for NB-IoT

Document for:	Discussion

# Introduction

In RAN4#92-Bis meeting, the following agreements were reached for RRM measurements in non-anchor carrier [1]:

- If the following conditions are met:

  - The relaxed cell monitoring criteria defined in TS 36.304 clause 5.2.4.12, and

  - Transmit power difference of NRS between anchor carrier and non-anchor paging carrier is signalled to the UE via the existing parameter nrs-PowerOffsetNonAnchor, and

  - UE is not configured with any positioning measurements,

- Then UE:

  - Stops monitoring neighbor cells

  - May stop performing NRSRP measurements on anchor carrier

  - May continue to perform NRSRP measurements only on the non-anchor carrier

  - May continue to evaluate relaxed monitoring conditions using non-anchor carrier NRSRP measurements and nrs-PowerOffsetNonAnchor for translation to their equivalent NRSRP in anchor carrier

  - Exiting this state will be based on not fulfilling the relaxed conditions in TS 36.304 clause 5.2.4.12.

The remaining issues on this topic were also outlined as follows:

- Issue 1:

- For the sparse PO case, there are sufficient number of NRS subframes available for performing NRSRP measurement on the non-anchor carrier.

- In the non-sparse PO case, the feasibility of conducting serving cell NRSRP measurement is FFS once further agreement has been reached with regard to N and M configurations in RAN1,

where N= number of NRS subframes starting from the subframe containing the PO, i.e., within the   NPDCCH search space, and M= number of NRS subframe before the subframe containing the PO.

- Issue 2:

- Filtering the samples from anchor and non-anchor carriers is FFS

In this paper, we discuss the remaining issues on this topic.

# Discussion

## Non-sparse paging occasions

In the previous RAN1 meeting, that pairs of (M,N) were agreed for non-sparse paging occasions as in Table 1:

Table 1 Values of (M,N) for non-sparse paging occasions

| nB | M | N |
| --- | --- | --- |
| 4T | 1 | 0 |
| 2T | 2 | 0 |
| T | 5 | 0 |
| T/2 | 10 | 0 |

Table 2 shows the arrangement of NRS subframes with respect to non-sparse paging occasions where yellow cells indicate the subframes with NRS and ‘x’ indicate a paging occasion. In non-sparse PO, M refers to the first M subframes out of 10 subframes before a paging occasion. As an example, with nB=T and M=5, the subframes# 9-13 are NRS subframes corresponding to the paging occasion in subframe#19.

Table 2 Arrangement of NRS subframes with respect to non-sparse paging occasions

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4T | x |  |  |  | x | x |  |  |  | x | x |  |  |  | x | x |  |  |  | x | x |  |  |  | x | x |  |  |  | x |
| 2T |  |  |  |  | x |  |  |  |  | x |  |  |  |  | x |  |  |  |  | x |  |  |  |  | x |  |  |  |  | x |
| T |  |  |  |  |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  | x |
| T/2 |  |  |  |  |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |

It is evident that in the case of nB=T/2, there are enough number of NRS subframes to enable RRM measurements on the non-anchor carrier. However, every other PO has NRS subframes. In our view, UE should still be allowed to perform RRM measurements on the non-anchor carrier if in DRX cycles corresponding to PO with NRS subframes preceding it as UE has this knowledge.

Observation 1. In the case of nB=T/2, there are enough number of NRS subframes to enable RRM measurements on the non-anchor carrier. However, every other PO has NRS subframes.

Proposal 1. UE to be allowed to perform RRM measurements on the non-anchor carrier if in DRX cycles corresponding to PO with NRS subframes preceding it as UE has this knowledge.

In denser scenarios, there are less than 10 subframes with NRS. However, these scenarios are very unlikely to occur in real-life. With such dense POs, the number of subframes with NRS are close to each other so UE can potentially stay awake longer and capture enough NRS subframes for a measurement. We have no strong view for these scenarios due to their unlikely occurrence and can think of two options for moving forward: 1) disallow RRM measurements in non-anchor carrier for nB<T/2, 2) allow it but leave to UE implementation and not specify requirements for them.

Proposal 2. RAN4 to discuss RRM measurements on non-anchor carrier for nB<T/2:

- Option 1: disallow RRM measurements in non-anchor carrier

- Option 2: allow it but leave it to UE implementation and not specify requirements for them

## Filtering measurements of anchor and non-anchor

Per previous agreements, only NRSRP can be measured in non-anchor carrier. The issue that has been raised is how the measurements from anchor and non-anchor carrier can be filtered together. This situation can occur if UE decides to drop back to anchor carrier for resynchronization or other purposes but still has to perform filtering of NRSRP measurements across DRX cycles (e.g., for serving cell evaluation).

As agreed in last RAN4 meeting, the NRSRP measurement on the non-anchor carrier can be translated to its equivalent in anchor carrier via the parameter nrs-PowerOffsetNonAnchor. After this translation, both metrics (NRSRP from anchor and NRSRP from non-anchor) are in common domain and can be combined or filtered together. For example, assuming that filtering operation is simply an averaging function, then:

NRSRP'=NRSRPnon-anchor.nrsPowerOffsetNonAnchor

NRSRP=0.5NRSRP'+NRSRPanchor

Proposal 3. NRSRP measurements on non-anchor carrier can be filtered or combined with NRSRP measurement on anchor carrier after translating the non-anchor carrier measurement with parameter nrs-PowerOffsetNonAnchor.

# Conclusions

Observation 1. In the case of nB=T/2, there are enough number of NRS subframes to enable RRM measurements on the non-anchor carrier. However, every other PO has NRS subframes.

Proposal 1. UE to be allowed to perform RRM measurements on the non-anchor carrier if in DRX cycles corresponding to PO with NRS subframes preceding it as UE has this knowledge.

Proposal 2. RAN4 to discuss RRM measurements on non-anchor carrier for nB<T/2:

- Option 1: disallow RRM measurements in non-anchor carrier

- Option 2: allow it but leave it to UE implementation and not specify requirements for them

Proposal 3. NRSRP measurements on non-anchor carrier can be filtered or combined with NRSRP measurement on anchor carrier after translating the non-anchor carrier measurement with parameter nrs-PowerOffsetNonAnchor.

# References

[1] R4-1912783