3GPP TSG-RAN4 Meeting #93	R4-1915623

# RENO, USA, November 18 – 22, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0302 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for introduction of Intra-frequency measurements with measurement gaps for NR-U |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Core |  |  |  |  |  | Date: |  |  | 2019-10-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introdction of Intra-frequency measurements with measurement gaps for NR-U |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introdction of Intra-frequency measurements with measurement gaps for NR-U based on the discussion of previous meetings. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The corresponding requirements of NR-U is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2A.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

9.2A.6	Intra-frequency measurements with measurement gaps

Editor notes: FFS whether SMTC or DMTC applies in this section.

#### 9.2A.6.1	Intra-frequency cell identification

The UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_without_index_CCA if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE has been indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index_CCA. The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index CCA. It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD.

Tidentify_intra_without_index CCA = TPSS/SSS_sync_intra_CCA + T SSB_measurement_period_intra_CCA  ms

Tidentify_intra_with_index_CCA = TPSS/SSS_sync_intra_CCA + T SSB_measurement_period_intra_CCA + TSSB_time_index_intra_CCA

Where:

TPSS/SSS_sync_intra_CCA: it is the time period used in PSS/SSS detection given in table 9.2A.6.1-1.

TSSB_time_index_intra_CCA: it is the time period used to acquire the index of the SSB being measured given in table 9.2A.6.1-2.

T SSB_measurement_period_intra_CCA: equal to a measurement period of SSB based measurement given in table 9.2A.6.2-1 or 9.2A.6.1-3.

CSSFintra: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in clause 9.1.5.2 for measurement conducted within measurement gaps.

If the higher layer signaling in TS 38.331 [2] signaling of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, requirements are not specified for Tidentify_intra_without_index CCA or Tidentify_intra_with_index_CCA.

If SCG DRX is in use, intra-frequency cell identification requirements specified in Table 9.2A.6.1-1 and Table 9.2A.6.1-2 shall depend on the SCG DRX cycle. Otherwise, the requirements for when DRX is not in use shall apply.

Table 9.2A.6.1-1: Time period for PSS/SSS detection

| Condition | TPSS/SSS_sync_intra_CCA |
| --- | --- |
| No DRX | max(600ms, (5+LPSS/SSS,gaps) x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(600ms, ceil(1.5x (5+LPSS/SSS,gaps)) x max(DRX cycle, MGRP, SMTC period)) x CSSFintra |
| DRX cycle>320ms | (5+LPSS/SSS,gaps) x (MGRP, DRX cycle) x CSSFintra |
| NOTE 1: LPSS/SSS,gaps is the number of SMTC periods not available at the UE during TPSS/SSS_sync_intra_CCA, where LPSS/SSS,gaps <LPSS/SSS,gaps,max.. NOTE 2: LPSS/SSS,gaps,max, =TBD for Max(DRX cycle, SMTC period, MGRP)≤40ms where DRX cycle is 0 for non-DRX, LPSS/SSS,gaps,max =TBD for 40ms<Max(DRX cycle, SMTC period, MGRP)≤320ms, LPSS/SSS,gaps,max =TBD for DRX cycle>320ms. |  |

Editor notes: The UE behaviour when LPSS/SSS exceeds LPSS/SSS,max is FFS.

Table 9.2A.6.1-2: Time period for time index detection

| Condition | TSSB_time_index_intra_CCA |
| --- | --- |
| No DRX | max(120ms, (3+Lind,gaps) x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil(1.5x (3+Lind,gaps)) x max(MGRP, SMTC period,DRX cycle) x CSSFintra) |
| DRX cycle>320ms | (3+Lind,gaps) x (MGRP, DRX cycle) x CSSFintra |
| NOTE 1: Lind,gaps is the number of SMTC periods not available at the UE during TSSB_time_index_intra_CCA where Lind,gaps < Lind,gaps,max.. NOTE 2: Lind,gaps,max, =TBD for Max(DRX cycle, SMTC period, MGRP)≤40ms where DRX cycle is 0 for non-DRX, Lind,gaps,max =TBD for 40ms<Max(DRX cycle, SMTC period, MGRP)≤320ms, Lind,gaps,max =TBD for DRX cycle>320ms. NOTE 2: Upon exceeding Lind,gaps,max over the TSSB_time_index_intra_CCA period of time, the UE has to restart the time index detection procedure. |  |

#### 9.2A.6.2	Intra-frequency Measurement Period

The measurement period for FR1 intra-frequency measurements with gaps is as shown in table 9.2A.6.2-1.

If SCG DRX is in use, intra-frequency measurement period requirements specified in Table 9.2A.6.2-1 shall depend on the SCG DRX cycle. Otherwise, the requirements for when DRX is not in use shall apply.

Table 9.2A.6.2-1: Measurement period for intra-frequency measurements with gaps

| Condition | T SSB_measurement_period_intra_CCA |
| --- | --- |
| No DRX | max(200ms, (5+Lmeas,gaps) x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x (5+Lmeas,gaps)) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | (5+Lmeas,gaps) x (MGRP, DRX cycle) x CSSFintra |
| NOTE 1: Lmeas,gaps is the number of SMTC periods not available at the UE during TSSB_time_index_intra_CCA where Lmeas,gaps < Lmeas,gaps,max. NOTE 2: Lmeas,gaps,max =TBD for Max(DRX cycle, SMTC period, MGRP)≤40ms where DRX cycle is 0 for non-DRX, Lmeas,gaps,max =TBD for 40ms<Max(DRX cycle, SMTC period, MGRP)≤320ms, Lmeas,gaps,max. =TBD for DRX cycle>320ms. NOTE 3: Upon exceeding Lmeas,gaps,max over the T SSB_measurement_period_intra_CCA period of time, the UE has to restart the measurement procedure. |  |

<End of Change 1>