3GPP TSG-RAN WG4 Meeting #93	R4-1915640

Reno, Nevada, USA, 18 November – 22 November 2019

Source:	Ericsson, Telstra

Title:	TP for TR 37.716-21-11: Including band combinations 1-28_n7

Agenda item:			10.4.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.716-21-11 to add 4 additional band combinations using 1-28_n7. The following to be added: DC_1A-28A_n7A, DC_1A-1A-28A_n7A, DC_1A-28A_n7B and DC_1A-1A-28A_n7B.

# 2. Text Proposal

---Start of changes---

## 5.1.X	DC_1-28_n7

### 5.1.X.1	Operating bands for EN-DC

Table 5.1.X.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 1-28_n7 | CA_1-28 | n7 | No |

### 5.1.X.2	Configuration for DC

Table 5.1.X.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-28A_n7A DC_1A-28A_n7B | DC_1A_n7A DC_28A_n7A DC_1A_n7B DC_28A_n7B | CA_1A-28A | n7 CA_n7B |
| DC_1A-1A-28A_n7A DC_1A-1A-28A_n7B | DC_1A_n7A DC_28A_n7A DC_1A_n7B DC_28A_n7B | CA_1A-1A-28A | n7 CA_n7B |

### 5.1.X.3		∆TIB and ∆RIB values

For DC_1-28_n7, the TIB,c and RIB,c values are reused from LTE combination CA_1-7-28 and DC_1-7_n28, and are given in the tables below.

Table 5.1.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-28_n7 | 1 | 0.5 |
|  | 28 | 0.6 |
|  | n7 | 0.6 |

Table 5.1.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-28_n7 | 1 | 0 |
|  | 28 | 0.2 |
|  | n7 | 0 |

### 5.1.x.4	REFSENS requirements

Co-existence analysis from DC_28_n7 shows that there is no impact from DC_28_n7 UL to Band 1 DL.

Co-existence analysis from DC_1_n7 shows that there is IMD5 impact from DC_1_n7 UL to Band 28 DL. MSD values are reused from CA_1A-7A-28A with CA_1A-7A in uplink.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-28A_n7A DC_1A-1A-28A_n7A DC_1A-28A_n7B DC_1A-1A-28A_n7B | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | 28 | 730 | 10 | 50 | 785 | 4.5 | IMD5 |
|  | n7 | 2510 | 10 | 50 | 2630 | N/A | N/A |

---End of changes---

# Reference

[1]		RP-192087, “Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)” Huawei, HiSilicon