3GPP TSG-RAN WG4 Meeting #92				  												R4-1915652

Reno, USA, 18th Nov. 2019 – 22nd Oct. 2019

Source: 	Nokia, BT

Title: 					TP for 37.716-21-21 to introduce DC_3-20A_n1A-n28A

Agenda Item: 			10.8.2 [DC_R16_xBLTE_2BNR_yDL2UL-Core]

Document for:	Approval

# 1	Introduction

This contribution is a TP for TR 37.716-21-21 (v0.7.0) [R4-1911443] to introduce DC_2A-66A_n41A-n261 which is listed in WID [R4-1911444]

### EN-DC of LTE inter band CA for x bands DL with 1 band UL(x=1, 2, 3, 4) + NR inter-band CA for 2 bands DL with 1 band  UL

| EN-DC configuration | Uplink EN-DC Configuration | contact name, company | contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) | supported next level fallback modes (in DL and UL) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_3A-20A_n1A-n28A | DC_3A_n1A DC_3A_n28A DC_20A_n1A DC_20A_n28A | S. Truelove, BT plc. | stephen.truelove@bt.com | Ericsson, Huawei, Rogers Comms. Canada | Ongoing | DL_3A-20A_n1A_UL_3A_n1A, DL_3A_n1A-n28A_UL_3A_n1A DL_3A_n1A-n28A_UL_3A_n28A, DL_3A-20A_n28A_UL_3A_n28A DL_3A-20A_n1A_UL_20A_n1A, DL_20A_n1A-n28A_UL_20A_n1A DL_20A_n1A-n28A_UL_20A_n28A, DL_3A-20A_n28A_UL_20A_n28A |
| DC_3C-20A_n1A-n28A | DC_3A_n1A DC_3A_n28A DC_20A_n1A DC_3C_n1A DC_3C_n28A DC_20A_n28A | S. Truelove, BT plc. | stephen.truelove@bt.com | Ericsson, Huawei, Rogers Comms. Canada | Ongoing | DL_3C-20A_n1A_UL_3A_n1A DL_3C_n1A-n28A_UL_3A_n1A DL_3A-20A_n1A-n28A_UL_3A_n1A DL_3C_n1A-n28A_UL_3A_n28A DL_3A-20A_n1A-n28A_UL_3A_n28A DL_3C-20A_n28A_UL_3A_n28A DL_3C-20A_n1A_UL_20A_n1A DL_3A-20A_n1A-n28A_UL_20A_n1A DL_20A_n1A-n28A_UL_20A_n1A DL_3C-20A_n1A_UL_3C_n1A DL_3C_n1A-n28A_UL_3C_n1A DL_3C-20A_n1A-n28A_UL_3A_n1A DL_3A-20A_n1A-n28A_UL_3A_n1A DL_3C_n1A-n28A_UL_3C_n28A DL_3C-20A_n1A-n28A_UL_3A_n28A DL_3A-20A_n1A-n28A_UL_3A_n28A DL_3C-20A_n28A_UL_3C_n28A DL_3A-20A_n1A-n28A_UL_20A_n28A DL_20A_n1A-n28A_UL_20A_n28A DL_3C-20A_n28A_UL_20A_n28A |

# 2	Text Proposal

*********************** Start of the TP ***************************************************

## 7.x	DC_3-20_n1-n28

### 7.x.1	Operating bands for DC

Table 7.x.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-20_n1-n28 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | TDD |
| NOTE: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. |  |  |  |  |  |  |  |  |

### 7.x.2	Channel bandwidths per operating band for DC

Table 7.x.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 30 | 40 | 50 | 60 | 80 | 100 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-20A_n1A-n28A | DC_3A_n1A DC_3A_n28A DC_20A_n1A DC_20A_n28A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 80 |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes7 |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes7 |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| DC_3C-20A_n1A-n28A | DC_3A_n1A DC_3A_n28A DC_20A_n1A DC_3C_n1A DC_3C_n28A DC_20A_n28A | 3 | See CA_3C in TS 38.101-1 or Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 100 |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes7 |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes7 |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 7: For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.x.3	Co-existence studies

Co-existence studies of DC_3_n1A, DC_3_n28, DC_3_n20, DC_20_n1, DC_20_n28 and DC_1_n28 are captured in TR 37.716-11-11 as well as for lower order combinations in TR 37.716-21-21.

### 7.x.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values are given in the tables below based on lower order combinations including the same bands.

Table 7.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-20_n1-n28 | 3 | 0.3 |
|  | 20 | 0.3 |
|  | n1 | 0.6 |
|  | n28 | 0.6 |

Table 7.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-20_n1-n28 | 3 | 0 |
|  | 20 | 0 |
|  | n1 | 0.2 |
|  | n28 | 0.2 |

### 7.x.5	MSD

MSD was studied in lower order band combinations for fallback modes. No need for additional MSD analysis.

*********************** End of the TP ***************************************************

# 3	References