3GPP TSG-RAN WG4 Meeting #94-e	R4-2002506

Electronic Meeting, Feb.24th – Mar.6th 2020

Agenda item:			6.8.3, 6.8.4,6.8.5

Source:	Moderator (ZTE Corporation)

Title:	Email discussion for RAN4#94e_#77_NR_NewRAT_Conformance_BS_Part_2

Document for:	Information

Introduction

Scope of this email discussion is listed in Table 1.

In this meeting following open issues will be discussed

Topic 1: TC updates for TS38.141-1/38.141-2

Topic 2: PHY Data generation for test model and

Topic 3: OSTP calculation

Topic 4: OBUE Cat B option 2 for n7 and n38 and removal of n65 in R15

Topic 5: Correlation between wanted signal and in-band emission

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: Topics listed above with numbers

- 2nd round: TBA

| # | Email title | WI | Topic areas | AI |
| --- | --- | --- | --- | --- |
| 77 | RAN4#94e_#77_NR_NewRAT_Conformance_BS_Part_2 | NR_newRAT-Perf | - TC updates for TS38.141-1/38.141-2 - PHY Data generatioin for test model - OSTP calculation - OBUE Cat B option 2 for n7 and n38 and removal of n65 in R15 - Correlation between wanted signal and in-band emission | 6.8.3, 6.8.4, 6.8.5 |

Topic #1: TC updates for TS38.141-1 and TS38.141-2

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2000666 | Nokia, Nokia Shanghai Bell | Title:CR to TR 38.141-1: Corrections on generation of test configurations Proposal 1: 1) For NRTC1 power allocation, set the power spectral density of each carrier to the same level only be used for testing BS supporting CA only operation (D.15), and set the power of each carrier to the same level for testing BS supporting multiple carriers (D.16), as in E-UTRA ETC1. 2) For NRTC4 generation, use Maximum number of supported carriers in multi-band operation (D.18) for carrier placement in each supported operating band (2nd bullet). 3) For NRTC4 generation, use Total maximum number of supported carriers (D.19) to compare to the calculated sum of the maximum number of supported carriers of each supported operating band (last bullet). |
| R4-2000667 | Nokia, Nokia Shanghai Bell | Title:CR to TR 38.141-1: Corrections on generation of test configurations |
| R4-2000668 | Nokia, Nokia Shanghai Bell | Title :CR to TR 38.141-2: Corrections on generation of test configurations Proposal : ) For power allocation for all test configurations except NRTC2, set the power of each carrier to the same level, and use rated transmitter TRP,Prated,t,TRP (D.38) instead of rated carrier TRP,PRated,c,TRP (D.37) for the total radiated power. 2) For NRTC1 generation, points to (D.60) instead of (D.59) for inter-band CA bands declared to be supported by the beam. 3) For NRTC2 power allocation, remove the condition of CA-only operation (D.20). 4) For NRTC4, change the term ‘active electronic components(s)’ to ‘active RF components’ to match the definition of ‘multi-band RIB’. 5) For NRTC5 power allocation, change the ‘EIPR’ to ‘TRP’, and clarify the declared rated TRP as the rated carrier OTA BS power, PRated,c,TRP (D.37). |
| R4-2000669 | Nokia, Nokia Shanghai Bell | CR to TR 38.141-2: Corrections on generation of test configurations |
| R4-2000679 | Nokia, Nokia Shanghai Bell | CR to TR 38.141-1: Corrections on generation of test configurations |
| R4-2000680 | Nokia, Nokia Shanghai Bell | CR to TR 38.141-1: Corrections on generation of test configurations |
| R4-2000681 | Nokia, Nokia Shanghai Bell | CR to TR 38.141-2: Corrections on generation of test configurations |
| R4-2000682 | Nokia, Nokia Shanghai Bell | CR to TR 38.141-2: Corrections on generation of test configurations |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1-1: TC updates for TS38.141-1

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 1-1: TC  updates for TS38.141-1

- Proposals

1) For NRTC1 power allocation, set the power spectral density of each carrier to the same level only be used for testing BS supporting CA only operation (D.15), and set the power of each carrier to the same level for testing BS supporting multiple carriers (D.16), as in E-UTRA ETC1.

2) For NRTC4 generation, use Maximum number of supported carriers in multi-band operation (D.18) for carrier placement in each supported operating band (2nd bullet).

3) For NRTC4 generation, use Total maximum number of supported carriers (D.19) to compare to the calculated sum of the maximum number of supported carriers of each supported operating band (last bullet).

Sub-topic 1-2: TC updates for TS38.141-2

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-2:

- Proposals

- For power allocation for all test configurations except NRTC2, set the power of each carrier to the same level, and use rated transmitter TRP,Prated,t,TRP (D.38) instead of rated carrier TRP,PRated,c,TRP (D.37) for the total radiated power.

2) For NRTC1 generation, points to (D.60) instead of (D.59) for inter-band CA bands declared to be supported by the beam.

3) For NRTC2 power allocation, remove the condition of CA-only operation (D.20).

4) For NRTC4, change the term ‘active electronic components(s)’ to ‘active RF components’ to match the definition of ‘multi-band RIB’.

5) For NRTC5 power allocation, change the ‘EIPR’ to ‘TRP’, and clarify the declared rated TRP as the rated carrier OTA BS power, PRated,c,TRP (D.37).

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| ZTE | Sub topic 1-1: The number of carriers of each supported operating band shall be the declared maximum number of supported carriers in multi-band operation (D.18) I think it should be D.17 for each operating band I think instead of multiple band , otherwise the maximum supported carrier per band is the same as maximum supported carrier in multi-band. If the sum of the maximum number of supported carriers in multi-band operation (D.18) is larger than the declared total maximum number of supported carriers of the BS (D.19), repeat the steps above for test configurations where in each test configuration the number of carriers of one of the operating band shall be reduced so that the total number of supported carriers is not exceeded and vice versa. I think the original one is also correct, why we need to remove band combination or each supported band for multi-band operation. Sub topic 1-2: it’s fine for all updates. |
| Nokia | Sub topic 1-1 (reply to ZTE comments): For LTE, we have in TS 36.141 clause 4.6.8. For all BS: - Maximum number of supported carriers within each band; - for contiguous spectrum operation - for non-contiguous spectrum operation For multi-band BS: - Total number of supported carriers for the declared band combinations of the BS - Maximum number of supported carriers per band in multi-band operation So there are two declarations for multi-band operation, one for CA and one for multi-band. Hence if we apply the same concept for NR, then D.17 is for all BS, while D.18 and D.28 are for multi-band BS, hope this clarifies. ZTE: clarification for D.18 of per band is needed, other part is fine for me :) |
| Ericsson | Sub-topic 1-1: CR is not correct: - Power: As it would anyway be same 5 MHz (or 20 MHz) signals used to build the TC, equal PSD or equal power is the same result. This would be usefuel when considering LTE 1.4Mhz or GSM, but not with NR. - Multi-band: D.17 should be replaced by D.18, ok. But following changes are not correct. First, the "the sum of the maximum number of supported carriers of each supported operating bands in multi-band operation ", is not meaningful. Second, D.18 is always lower or equal to D.19, so it could never be larger.... Nokia: Power, we are adding NB-IoT operation in NR in-band in TS 38.141-1 now, so it is time to align TS 38.141-1 with 36.141 and 37.141. Also my understanding is D.18 is for eachoperating band, so the sum of D.18 for multi-band can be larger than D.19. Sub-topic 1-2: This CR is not correct: - D.59 should be replaced with D.60, that's ok - But other changes are not ok: the power set on each carrier should be indeed according to the declared one (D.37) and not shared euqally in between all carriers as it is proposed. This is also aligned with 37.145-2. Nokia: The existing statement says total power (not on each carrier) is set to D.37, ‘For all other requirements ensure the total radiated power is set to rated carrier TRP PRated,c,TRP(D.37)’. Also BS manufacturers should have the flexibility to declare lower output power per carrier for multi-carrier operation, right?.. |

CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2000666 | Company A: |
|  | Company B: |
| R4-2000668 | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Sub-topic 1-1: R4-2000666 need revision Agreement: D.18 needs to be updated with per band Sub-topic 1-2: R4-2000668 return to Recommendations for 2nd round: Sub-topic 1-1:R4-2000666 revised to R4-2002453 Sub-topic 1-2:R4-20006668 revised to R4-2002532 |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
|  | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Nokia: R4-2002453, R4-2000667, R4-2000668 and R4-2000669 are revised to correct title error: ‘TR’ should be ‘TS’.

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2000666 | Revised to R4-2002453, Noted. |
| R4-2000667 | Cat A CR Withdraw |
| R4-2000668 | Revised to R4-2002532,agreed |
| R4-2000669 | Cat A CR, agreed |

Topic #2: PHY data generation for test model

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2001171 | CATT | Discussion on random data content of physical channels for NR test models |
| R4-2001676 | Nokia, Nokia Shanghai Bell | Discussion on data content for NR test models Observation 1: Generation for NR test models should match physical layer design in RAN1 specifications. Observation 2: Using “random” data instead of “all 0” may be beneficial for some of the test models, but it also increases test time and complexity for implementing PN. |
| R4-2001722 | Ericsson | Random data content of physical channels for NR test modes Proposal 1: random data has been proposed as a means to bring the NR TM for a more realistic waveform but also to provide amplitude statistics of the NR TM to be Rayleigh distributed. Proposal: PN 23 for random data generation |
| R4-2001723 | Ericsson | CR to TS 38.141-1: Random data content for NR BS Test Models |
| R4-2001724 | Ericsson | CR to TS 38.141-1: Random data content for NR BS Test Models |
| R4-2001725 | Ericsson | CR to TS 38.141-2: Random data content for NR BS Test Models |
| R4-2001726 | Ericsson | CR to TS 38.141-2: Random data content for NR BS Test Models |
| R4-2001730 | Futurewei | Scrambling and initialization for test models Observation 1: it is possible to generate independent realizations over a measurement interval if an LFSR has length of at least 23 bits. Observation 2: Cascading current scramblers initialized with different seeds may not improve the randomness of bits. Observation 3: How the PN generator operates should be clarified (e.g., continuous, reinitialized with different seeds). Proposal 1: If more realizations are needed, consider augmenting the RNTI with the slot number in the calculation of the initialization of the scrambling seed for PDSCH. |
| R4-2001805 | Keysight Technologies UK Ltd | Study on NR Test Model signal characteristic by data content choice Proposal1. Use CCDF curve as tool to evaluate data content of Test model. Proposal2. Use random data rather All zero for test model data content. Proposal3. Between PN23 and PN31, either PN sequence is good enough. And no further randomization study is not necessary. Proposal4. No further study necessary for multiple CC case and cell ID. Proposal5. Further study may be needed on TM2 if RAN4 sees concern on CCDF plot of TM2 |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 2-1: PHY data generation for test model

Issue 2-1: data generationt to solve the correlation between symbols

- Proposals

  - Option 1: random data with PN23

  - Option 2: augmenting the RNTI with the slot number in the calculation of the initialization of the scrambling seed for PDSCH

- Recommended WF

  - TBA

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| ZTE | Sub topic 2-1: support option 1 with random data with PN23 as option 2 is not aligned with PHYspec. |
| Futurewei | Sub topic 2-1 To understand the implementation impact of option 1 - Can the polynomial for PN23 be provided? - What is the initialization seed? When is the PN generator initialized? Is the output of generator continuous across slots or reset each slot? Is there advancing such as by 1600 in NR? (How is synchronization of the generator ensured) For option 2: - The comment about alignment to Phy spec: The use of RNTI to classify the types of PDSCH is arbitrary. We chose to use 0, 1, 2 for all slots to keep the test model specification simple. We could have used RNTI=(0,1,2) for the first slot, (4, 5, 6) for the second slot, (8, 9, 10) for the third slot, etc. If we had done that, then this use of the RNTI is compliant with the PHY spec. The proposal is just a logical change of the RNTI used (basically add slot number to the RNTI with no other changes). There can be other ways to capture that statement. - If the decision is to change the number of test realizations, we should evaluate the implementation impacts. For example, implementing a PN generator is also non-compliant. |
| Nokia | Sub topic 2-1: Option 2 seems to be against RAN1 design, we think test models should be design according RAN1 specification even if this is for testing purpose only, also it is not sure what is benefit of this solution i.e. what is PAPR difference compare to “all zero’s” current design. For option 1 there are some benefits (PAPR limitation) of usage of PN for some test models, however there would be a trade-off additional test time and complexity using the PN compared to current “all zeros” data. If this option would be specified it is not enough to add PN23, also the PN polynomial need to be specified in specification. |
| Futurewei | Subtopic 2.1: Followup comment to Nokia: The contribution R4-2001730 shows a performance difference to the “all zeros”. In addition a comparison to the PN23 is provided. The performance of PN23 and making the RNTI vary by slot number is quite similar. The comment about RAN1 design is not correct. What is provided to RAN1 is an RNTI value, which is needed by the scrambler initialization. The RNTI value just changes each slot; thereby causing the realizations to be different each slot. |
| Nokia | Reply to Futurewei comment: Our understanding is that RNTI is unique for an UE and cannot be changed slot-by-slot in real implementation for the UE. |
| ZTE | PN23 could generate 2^23-1 bit in total is that right, assuming maximum channel bandwidth 273PRB for 100MHz, 30KHz and 10 slots, then maixmum REs could be 273*12*14*10*2=917280 which is still less than 2^23-1. |
| Keysight | For PN23 definition question, what we refer to is, ITU-T O.151 it’s available from ITU web. (free to download!!), please see section 2.2 for PN23. By the way, this definition itself doesn’t provide beginning or end of sequence because original intention is to be used repeatedly. But for those examples I’m providing uses sequence starting with 23 consecutive zero (note, this is inverted generator so shift register values are one) |
| Ericsson | Sub topic 2-1: The polynomial is taken from ITU guidance: https://www.itu.int/rec/dologin_pub.asp?lang=e&id=T-REC-O.150-199605-I!!PDF-E&type=items Seed is all ones. Which is also taken from guidance in ITU for other PN sequences. It should be continuous, as if it restarts at the beginning of every slot it does not solve the issue we saw in the scrambling code design raised in previous Nokia paper. The proposed change of “c_init” by using slot number (and RNTI) could work fine, but then we don’t use the standard (RAN1) scrambler and if one wants to retrieve the real bits transmitted in DL you must keep track of which scrambler was used. |
| ZTE | Sub topic 2-1: just purely technical clarification from my simulation experience in the past Pros and Cons for option 1 and option 2 Option 1: Pros : align with RAN1 spec and RNTI is unique Cons: when calculating EVM per RE and average the whole frame, it maybe a bit more complicated as EVM =Soft information after LDPC decoding before modulator hard decision -all REs QPSK information (different among REs due to random data generation) ; Option 2: Pros :when calculating EVM per RE and average the whole frame, it maybe easier as EVM=Soft information after LDPC decoding before modulator hard decision -all REs QPSK information (unique with 1/sqrt(2)(1+1i)) ; TE don’t need to align with BS for data generation, but for RNTI generation. Cons: not align with RAN1 spec i think. |
| Keysight | Regarding with Implementation complexity for TE, data payload content doesn’t matter at all for EVM measurement nor power measurement. Though if RNTI value is different per slot, it needs to be specified as defined. (this small difference though). Test time also shouldn’t change because of data payload difference either. |
| ZTE | To Keysight, if TE are fine with data generation at TE side, then we are also fine at BS side. |
| Futurewei | We are just providing more options for implementation considerations. With either option 1 or 2, it is important to capture the exact procedure so that all implementation experts can understand the ramifications of the options. To Keysight, thank you for providing the reference. Because there are so many different ways to describe PN generators, it is important to specify the process. To avoid confusion, references or perhaps a model provided in the specification could be considered? The comment about continuous / reset is critical. How will the test equipment know how to synchronize to the state of the PN generator? For example, the PN generator could be reset at the beginning of every frame. |
| Keysight | For Futurewei, thank you for question. Description in the ITU-T standard document is enough to have the sequence generator. Also, TE doesn’t make any synchronization with data content in payload. We just decode and get result. By the way, this is just additional note. Those PN sequences can be regenerated with, for example with PN23, taking any part of 23 bits of sequence in generator then rest of sequence can be reproduced. It is designed this way. |
| Ericsson | To Futurewei, please check in detail the document, which I provide the link in previous comment; we feel this document is clear enough for PN generation. To ZTE, regarding your comment number of bits. Yes, in our contribution we have calculated this R4-2001722 Figure 5. Please let me know if this is not clear for you. |
| ZTE | To Ericsson, it’s clear for me and aligned with our analysis. Anyway PN23 sequence is enough |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2001723 | ZTE: is that necessary to use the uniform distribution to describe PN23? |
|  | Futurewei: the implementation details are incomplete. |
|  | Nokia: More details is needed i.e. the PN polynomial need to be specified in specification. |
|  | Ericsson: We are happy to revise the CR with reference to ITU-T standards document if this would help. However, the we would prefer not to copy ITU-T document/text into the RAN4 spec. |
| R4-2001725 | ZTE: the same comment as before. |
|  | Nokia: Same comment as above for 1723. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: - Option 1: random data with PN23 - Reference fro PN23 and exact procedure for PN generator could be FFS - R4-2001723 and R4-2001725 need revision. Recommendations for 2nd round: |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

| Company | Comments |
| --- | --- |
| Futurewei | There are minor issues with the CR and some opportunities for misunderstanding the design of the PN generator. - Reference to ITU missing. (an update to the reference section is needed too) - Subclause 4.9.2.3 needs revision Randomisation of the data content is obtained by utilizing the length-31 Gold sequence scrambling of TS 38.211 [17], clause 5.2.1 which is invoked by all physical channels prior to modulation and mapping to the RE grid. An appropriate number of '0' bits shall be generated prior to the scrambling. A wording suggestion is “An appropriate number of bits shall be generated by a PN generator prior to the scrambling” - The wording in the CR needs clarification Generate this amount of bits according to the output of the PN23 generator sequence starting seed of all ones. - The wording “PN23 generator sequence” is unclear whereas “PN23 generator” or “PN23 sequence generator” may be intended. A suggestion is to remove the word “sequence” - “starting seed of all ones” can imply all ones initialization is used each slot (a suggestion is proposed later) From the discussions, consensus can be reached on - All physical channels (PDCCH and PDSCH) use a PN generator - The PN generator has a one-time initialization of all ones - The PN generator output is continuous. However, it is unclear whether - Up to 4 separate PN generators are used. (1 for PDCCH, 1 for PDSCH RNTI=0, 1 for PDSCH RNTI=1, and 1 for PDSCH RNTI=2). See approach 1. From the discussions, it seems to be the accepted configuration. But can it be agreed? PN generator 0 PDCCH PDSCH RNTI=0 PDSCH RNTI=1 PDSCH RNTI=2 PN generator 1 PN generator 2 PN generator 3 Approach 1: one PN generator per channel. Each generator is independent. The current wording is unclear with approach 1. One possible interpretation is approach 2. PN generator PDCCH PDSCH RNTI=0 PDSCH RNTI=1 PDSCH RNTI=2 Approach 2: One PN generator for all channels. It may be an implementation choice to use a single PN generator module/routine for all 4 channels but the output of approach 2 is different than approach 1. It should be clear what approach is used. The continuous generation of bits on a slot-by-slot basis: the state of the PN generator should be preserved at the end of the sequence generations in a slot and then the preserved value should initialize the PN generator in the next slot. It may not be clear how to produce this continuous generation with the current CR. If approach 1 is considered: perhaps some wording like this can be used Randomisation of the data content is obtained by utilizing a PN generator and the length-31 Gold sequence scrambling of TS 38.211 [17], clause 5.2.1 which is invoked by all physical channels prior to modulation and mapping to the RE grid. An appropriate number of bits shall be generated by a PN generator prior to the scrambling. - Each physical channel uses an independent PN generator. - Each PN generator has a one-time initialization with a starting seed of all ones - For the current slot, the PN generator is initialized using the final state of the same PN generator from the previous slot. … Generate this amount of bits according to the output of the PN23 generator associated with the physical channel. … |
| Keysight | Thank you Futurewei for summarizing points of discussion. I can provide some insight from TE vender and measurement point of view. Here is excerpt from your comment; From the discussions, consensus can be reached on - All physical channels (PDCCH and PDSCH) use a PN generator - The PN generator has a one-time initialization of all ones - The PN generator output is continuous. For a), it has very small impact on overall characteristic by PDCCH data payload because it’s relatively small portion compare with entire waveform based on our analysis. It doesn’t matter much but good to have the same thing as PDSCH so that use of PN23 would be better. For b) this would be wording issue, while everyone seems seeing all one in initial value in seed. current CR text is fine with me too. But again, this doesn’t change much (see following) For c), this also doesn’t matter, as you wrote, it’s implementation choice. For measurement, TE doesn’t care about content of data payload. This is for both EVM and power measurement. Key thing here is to have the same characteristic. Choosing data payload content is enough. Trying to define this level of details doesn’t change overall characteristic but requires a lot of discussion and effort, then loses time. I guess, when you run some simulation on changing data payload content, change might make one specific combination (ex. TM1.1, 20MHz BW, 30kHz SCS) result preferable but which might cause less preferred result on other combination (TM type, BW, SCS). Similarly, Waveform CCDF of each result changes by changing this level of detail you wrote, which somewhat unpredictable way, however, those are in certain range of variation. And doesn’t change overall characteristics. (and overall result better than all zero cases because bit sequence is random) When we start looking into each individual CCDF curve and measured result, we might want to specify detail, because one change might cause better or poor result, but when we look at family of result, there many combination of TM type, BW, SCS, overall characteristic is the thing to care about and doesn’t change with your choice. I proposes to leave it for implementation and not to be too specific on this level of detail. |
| Nokia | It would be beneficial to include more details on PN from ITU O.150 to specification to have details in our spec. Having these details in our spec would be useful for readers and people using it. For example: - Generate this amount of bits according to the output of the PN23 [23] generator sequence starting seed of all ones according [23]:. The sequence may be generated in a twenty-three-stage shift register whose 18th and 23rd stage outputs are added in a modulo-two addition stage, and the result is fed back to the input of the first stage. – Number of shift register stages 23 – Length of pseudo-random sequence 223 – 1  8 388 607 bits – Longest sequence of ZEROs 23 (inverted signal) |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2001723 | Revised toR4-2002454, Agreed |
| R4-2001724 | Cat A CR, agreed |
| R4-2001725 | Revised toR4-2002455,agreed |
| R4-2001726 | Cat A CR, agreed |
| Other Tdoc | Noted |

Topic #3: OSTP calculation

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2001677 | Nokia, Nokia Shanghai Bell | CR to 38.141-1 updates for OSTP calculations Proposal: For OSTP formula Nsym as all OFDM symbols that carry PDSCH and not contain PDCCH, RS or SSB is included to formula: OSTP=RETP / Nsym |
| R4-2001678 | Nokia, Nokia Shanghai Bell | CR to 38.141-1 updates for OSTP calculations |
| R4-2001679 | Nokia, Nokia Shanghai Bell | CR to 38.141-2 updates for OSTP calculations Proposal : For OSTP formula Nsym as all OFDM symbols that carry PDSCH and not contain PDCCH, RS or SSB is included to formula: OSTP=RETP / Nsym |
| R4-2001680 | Nokia, Nokia Shanghai Bell | CR to 38.141-2 updates for OSTP calculations |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 3-1: OSTP calculation

Issue 2-1: OSTP calculation

- Proposals

For OSTP formula Nsym as all OFDM symbols that carry PDSCH and not contain PDCCH, RS or SSB is included to formula

OSTP=RETP / Nsym

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| ZTE | Sub topic 3-1: fine to have the updates. |
| Futurewei | Sub-topic 3-1: the change is fine. This is just an editorial suggestion: use OSTP=1NsymRETP Or put the change within the equation object (to keep the font sizes consistent) |
| Nokia | To Futurewei: Thanks for comment, revision with editorial edition will be in folder. |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
|  | Company A |
|  | Company B |
|  | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: R4-2001677 and R4-2001679 need revision. Recommendations for 2nd round: |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2001677 | Revised to R4-2002456, agreed |
| R4-2001678 | Cat A CR, agreed, |
| R4-2001679 | Revised to R4-2002457,agreed |
| R4-2001680 | Cat A CR, agreed |

Topic #4: OBUE Cat B option 2 for n7 and n38 and removal of n65 in R15

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2001824 | Huawei | Title:CR to TS 38.141-1: OBUE Cat. B Option 2 correction for n7, Rel-15 Proposal 1: addding n7 and n38 for OBUE Cat B option 2 for AAS BS according to ECC decision; Proposal 2: Removal of n65 in R15 spec and capture n65 in R16 spec |
| R4-2001825 | Huawei | Title:CR to TS 38.141-1: OBUE Cat. B Option 2 correction for n7, Rel-16 |
| R4-2001826 | Huawei | Title: CR to TS 38.141-2: OBUE Cat. B Option 2 correction for n7 and n38, Rel-15, Cat F Proposal 1: addding n7 and n38 for OBUE Cat B option 2 for AAS BS according to ECC decision; Proposal 2: Removal of n65 in R15 spec and capture n65 in R16 spec |
| R4-2001827 | Huawei | Title: CR to TS 38.141-2: OBUE Cat. B Option 2 correction for n7 and n38, Rel-16, Cat A |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 4-1 OBUE Cat B option 2 for n7 and n38 and removal of n65 in R15

Issue 2-1: TBA

- Proposals

  - addding n7 and n38 for OBUE Cat B option 2 for AAS BS according to ECC decision

  - Removal of  n65 in R15 spec and capture n65 in R16 spec

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| ZTE | Sub topic 4-1: it’s fine to remove that n65 and add n7 and n38 into R15 spec. |
| Nokia | It is ok to align specs and correct. |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2001824 | ZTE: okay |
|  | NEC: Fine to remove n65 in R-15 and to add in Rel-16. It means R4-2001825 shall not be cat-A. Typo, add space between “n8” and “or”. |
|  | Huawei: This is well spotted by NEC. Band n65 shall be removed from Rel-15, but shall be kept in Rel-16 (that’s why R4-2001825 shall not be Cat A, but Cat F). So, the content of R4-2001824 is correct and can be agreed, while R4-2001825 needs to be changed from Cat A to Cat F and provided separately during 2nd round (R4-2001825 was not uploaded, this needs coordination with MCC). |
| R4-2001826 | ZTE: okay |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: R4-2001824 need revision R4-2001825 should be Cat F instead of Cat A R4-2001826 is approved please upload R4-2001827 Recommendations for 2nd round: |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

| CR/TP number | Comments collection |
| --- | --- |
| R4-2001824 | Nokia: We think that for bands n7 and n38 only Category B Option 1 shall apply. Cat B Option 2 applies only for Type 1-C in 38.141-1, thus this is correct in specification already. |
|  | Ericsson: we need more time to check again if the interpretation of ECC decision is correct in this case. |
| R4-2001826 | Ericsson: we need more time to check again if the interpretation of ECC decision is correct in this case. |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2001824 | Revised to R4-2002458 CR to TS 38.141-1: n65 removal from the OBUE Cat. B Option 2, Rel-15,uploaded, agreed |
| R4-2001825 | Cat A, CR, withdraw |
| R4-2001826 | It’s approved in the 1st round, after further discussion, it should be Noted in 2nd round |
| R4-2001827 | Cat A CR, withdraw |

Topic #5: Correlation between wanted signal and in-band emission

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2002042 | Nokia, Nokia Shanghai Bell | Observation 1: xn and yn in Equation (3) are independent Gaussian random values, while xn, yn and zn in Equation (2) provides the position of each array element in the Cartesian coordinate system. To avoid confusion, a different symbol could be used to represent the Gaussian random value. Question 1: Is the correlation model assumed by Equation (3) realistic or only valid for some AAS BS implementation variants? Observation 2: wn has only a real part which is constant. Observation 3: When ρ=1, the radiation pattern of the unwanted signal is identical to the wanted signal. As a result, the beam-based directions procedure can be applied without causing TRP estimation errors. Observation 4: wn is a random complex number. Observation 5: For ρ = 0, it can be concluded that criteria (a)-(c) are not met based on the above analysis, which implies criterion (d) is also not met. Observation 6: wn is a random complex number but the real part is composed of a constant 310 and a random number. Observation 7: For ρ = 0.9, it can be concluded that criteria (b)-(c) are not met based on the above analysis, which implies criterion (d) is also not met. Consequently, the beam-based directions procedure cannot be used for computing TRP estimate. Observation 8: wn is a random complex number but the real part is composed of constant 210 and a random number similar to the case ρ = 0.9. However, the constant is smaller as ρ decreases. Observation 9: For ρ = 0.4, it can be concluded that criteria (b)-(c) are not met based on the above analysis, which implies criterion (d) is also not met. Consequently, the beam-based directions procedure cannot be used for computing TRP estimate. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 5-1

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 2-1: correlation between wanted signal and in-band emission

- Observation: general relationship between the HPBW and signal correlation exist

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| ZTE | Sub topic 5-1: no strong opinions. |
| Ericsson | Subtopic 5-1: Issue 2-1: Since the correlation is generally not known for emission, its better to be safe than sorry. Hence, RC or dense grids are preferred for emission. If you know the beam characteristics, you can select a sampling grid accordingly. Maybe the hidden message here is that if we identify the first nulls of the OBUE emission in the same positions as for the main beam, then we can assume the OBUE has same pattern as main beam? Nulls are easy to identify in the lab, so it might be interesting to look at this approach. |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2002042 | Ericsson: Subtopic 5-1: Issue 2-1: Since the correlation is generally not known for emission, its better to be safe than sorry. Hence, RC or dense grids are preferred for emission. If you know the beam characteristics, you can select a sampling grid accordingly. Maybe the hidden message here is that if we identify the first nulls of the OBUE emission in the same positions as for the main beam, then we can assume the OBUE has same pattern as main beam? Nulls are easy to identify in the lab, so it might be interesting to look at this approach. |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: R4-2002042 Noted Recommendations for 2nd round: |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |