3GPP TSG-RAN4 Meeting #98-bis-e		R4-2104446

Electronic Meeting, 12 April – 20 April, 2021

3GPP TSG RAN WG1 Meeting #104e 		R1-2102034

e-Meeting, January 25th – February 5th, 2021

Title:	Reply LS on SL switching priority

Response to:	R1-2100009(R4-2017839) LS on SL switching priority from RAN4

Release:	Release 16

Work Item:	5G_V2X_NRSL-Core

Source:	RAN1

To:	RAN4

Cc:	RAN2

Contact person:	Qun Zhao

Email:		         zhaoqun1@xiaomi.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN1 would like to thank RAN4 for the LS (R1-2100009/R4-2017839) on SL switching priority.

RAN1 has discussed questions listed in the LS and would like to provide the following feedback.

Question 1: Is there priority defined for LTE SL and NR SL?

[RAN1 reply]  Yes, priority is defined for the LTE SL in clause 4.4.5.1 of TS 23.285 and for NR SL in clause 5.4.3.3 of TS 23.287. On SL priority in RAN1, please refer to answers of Question 2 and 3.

Question 2: How does RAN WG1 define the priority of LTE SL and NR SL? For example, which parameter is used and how to determine the priority?

[RAN1 reply] In RAN1, priority of a PSCCH/PSSCH transmission is indicated in the “Priority” field of an SCI format 1 in LTE, or SCI format 1-A in NR, after being set for the transport block by higher layers. The priority of NR S-SSB or LTE PSBCH/SSSS/PSSS is determined/provided by higher layers. The priority of NR PSFCH is the same as the corresponding PSSCH.

Question 3: Is it the case that there is no higher priority for LTE SL than NR SL?

[RAN1 reply] No. In RAN1#98 meeting, the following agreement was made, where PPPP is represented in the “Priority” field of an SCI format 1 in LTE:

Agreements:

- RAN1 understand that NR V2X priority field and PPPP are directly comparable i.e. the same numerical value has the same meaning in both the RATs.

# 2	Actions

To RAN4 group

ACTION: 	RAN1 respectfully asks RAN4 to take the above information into account.

# 3	Dates of next TSG RAN WG 1 meetings

TSG-WG1 Meeting #104-bis-e	12th – 20th April 2021			Online Meeting

TSG-WG1 Meeting #105-e	19th – 27th May 2021			Online Meeting