3GPP TSG-RAN WG4 Meeting # 98-bis-e 	                                         R4-2106449

Electronic Meeting, Apr. 12-20, 2021

Agenda Item:	5.5.2.2.2

Source:	Intel Corporation

Title:	Discussion on NR Positioning test cases configuration

Document for:		Discussion

# Introduction

In the last RAN4 meeting, the test case for NR positioning requirements was discussed. And the test case list and the remaining issues was summarized in [1].

In this paper we provide more considerations on the basic test configurations for PRS based measurement and other open issues for the test case list.

# Configuration Patterns for PRS

Similar as SSB configurations for RRM test cases in TS38.133, it is more efficient to specify the common PRS configurations before the detailed test cases definition. Then each test case for NR positioning defined in TS38.133 can also refer to these PRS configuration patterns more clearly.

NR-DL-PRS-Info-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceSetList-r16		SEQUENCE (SIZE (1..nrMaxSetsPerTrp-r16)) OF

NR-DL-PRS-ResourceSet-r16,

...

}

NR-DL-PRS-ResourceSet-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceSetID-r16			NR-DL-PRS-ResourceSetID-r16,

dl-PRS-Periodicity-and-ResourceSetSlotOffset-r16

NR-DL-PRS-Periodicity-and-ResourceSetSlotOffset-r16,

dl-PRS-ResourceRepetitionFactor-r16	ENUMERATED {n2, n4, n6, n8, n16, n32, ...}

OPTIONAL,	-- Need OP

dl-PRS-ResourceTimeGap-r16			ENUMERATED {s1, s2, s4, s8, s16, s32, ...}

OPTIONAL, 	-- Cond Rep

dl-PRS-NumSymbols-r16				ENUMERATED {n2, n4, n6, n12, ...},

dl-PRS-MutingOption1-r16			DL-PRS-MutingOption1-r16			OPTIONAL,	-- Need OP

dl-PRS-MutingOption2-r16			DL-PRS-MutingOption2-r16			OPTIONAL,	-- Need OP

dl-PRS-ResourcePower-r16			INTEGER (-60..50),

dl-PRS-ResourceList-r16				SEQUENCE (SIZE (1..nrMaxResourcesPerSet-r16)) OF

NR-DL-PRS-Resource-r16,

...

}

Particularly, the above PRS parameters above which are configured by LMF for the different types of positioning method (e.g. RSTD, UE Rx-Tx time difference) can be included in these PRS patterns.

- PRS periodicity: Periodicity of DL PRS allocation in slots configured per DL PRS Resource Set (i.e. all DL PRS Resources of a given Set have the same periodicity). And in order to minimize the testing efforts 2^u *160ms periodicity can be taken consideration into the test case as the most typical deployment scenarios.

- PRS SCS and transmission BW: In our simulations [3], the impacts on the PRS RSTD and RSRP measurement accuracy due to the different BW can be observed. Thus, if the more than accuracy requirements regarding to the PRS BW is introduced in TS38.133, the multiple PRS BW is necessary to be defined in these PRS patterns.

- Number of PRS symbol and Comb size: To simplify the test case, the number of PRS symbol can be same as comb size. In the previous meeting the accuracy requirements agonistic to comb size is under debating [1]. As a start point, the one comb size can be assumed up to further discussions.

- Repetition factor: This parameter controls how many times each DL-PRS Resource is repeated for a single instance of the DL-PRS Resource Set. It is applied to all resources of DL PRS Resource Set. In [simulation assumptions], the possible candidates of this parameter can be {1, 2, 4, 8, 16, 32}. But the very high repetition is rarely happened in our view and introduce extra UE implementation complexity (e.g. cross-slots soft combination). Thus, the parameter for testing can be {1, 2}.

In summary, as examples in the table below several PRS patterns depending on these parameters combinations above can be defined.

Table 1: PRS configuration patterns for NR positioning measurement

|  | PRS Pattern 1 | PRS Pattern 2 | PRS Pattern 3 |
| --- | --- | --- | --- |
| SCS | FR1, 15k | FR1, 30k | FR2, 120k |
| PRS periodicity | 160ms |  |  |
| PRS periodicity offset | 10ms |  |  |
| PRS transmission bandwidth in RBs (System Bandwidth) | 52 (10MHz) | 132 (50MHz) | 128(200MHz) |
| Number of PRS symbol | 4 | 4 | 12 |
| Comb size | 4 |  |  |
| Repetition factor | 4 | 2 | 1 |
| Number of PRS slots | 1 |  |  |

Proposal 1: The PRS configuration patterns in Table 1 for NR Positioning measurement tests can be specified in the Annex A.3.x of TS 38.133.

Beside parameters above which can be included in the common PRS configuration patterns, the other parameters depending on measured cell/TRPs below shall be also considered.

- SRS configuration

When measuring UE Rx-Tx time difference, the SRS transmission parameters shall be also specified as[6]:

Table 2: general SRS configuration for UE Rx-Tx RRM test cases

| SRS-Resource | SRS-ResourceId | 0 |
| --- | --- | --- |
|  | nrofSRS-Ports | Port1 |
|  | transmissionComb | n4 |
|  | combOffset-n4 | 0 |
|  | cyclicShift-n4 | 0 |
|  | resourceMapping startPosition | 0 |
|  | resourceMapping nrofSymbols | n4 |
|  | resourceMapping repetitionFactor | n1 |
|  | freqDomainPosition | 0 |
|  | freqDomainShift | 0 |
|  | freqHopping c-SRS | Matches NRB,c |
|  | groupOrSequenceHopping | Neither |
|  | resourceType | Periodic |
|  | periodicityAndOffset-p | 80*2^u, 20*2^u |
|  | sequenceId | 0 |

Proposal 2: SRS configuration can be specified as Table 2.

# Updated Test Cases List

- Different deployment scenarios

In the previous meetings, whether the test case for NR-DC is needed in Rel16 was debated.

- Option 1: Only need to define the test cases for SA - Option 2 : RAN4 will define NR positioning test cases for NR-DC, where PCell is in FR1, and PSCell is in FR2; otherwise define test cases for other deployments with both FR1 and FR2 (e.g., CA or SA with FR1 PCell and FR2 neighbors).

In our view, NR positioning measurements configured by NR PCell are supported only in Rel-16. That is currently it is sufficient to define the test cases for NR standalone scenarios.

Proposal 3: It is enough to define the test cases for NR standalone cells.

- Additional test cases for RSTD

In the last meeting, whether the test cases for the differential RSTD requirements is needed was discussed.

- Specify three RSTD measurement accuracy (sub-) tests: Absolute RSTD, Differential RSTD

In our views, it is unnecessary to test differential RSTD. The difference between absolute RSTD and differential RSTD is only in the report signaling, but UE’s measurements are same. Additional path is an assistant report, and RAN4 has never discussed the accuracy requirement for it.

Proposal 4: Absolute measurement reporting is tested for all PRS measurements. It is unnecessary to define the test case for differential RSTD.

- Subsets of accuracy tests :FFS

In the last meeting, how to define the subsets of accuracy test case was discussed.

- Option 1. Define two subtests per accuracy test cases corresponding to different combinations of {Es/Iot, PRS BW}. - Option 2. In each test case, the NR measurements are tested for multiple measurement bandwidths, with at least: - the smallest bandwidth, - a bandwidth from the medium bandwidths range (e.g., >48 PRBs in FR1 or >32 PRBs in FR2), - a bandwidth from the large bandwidths range (e.g., >132 PRBs in FR1 or >64 PRBs in FR2).

Currently the accuracy requirements were not finalized. If the accuracy requirements for RSTD/PRS RSRP was differentiated for the more than 2 bandwidth the less test case will be highly expected. Thus, the Option 1 which can minimize the necessary testing number is more preferable.

Proposal 5:  In order to minimize the testing efforts, we can define two subtests per accuracy test cases corresponding to different combinations of {Es/Iot, PRS BW}.

# Test case design principles

- SINR set for the different SINR side condition

In the last meeting, SINR side condition for the different SINR side condition for UE Rx-Tx time difference requirements was agreed as:

- Option 1 - Both SINR side conditions for UE Rx-Tx are tested in the same test with two cells. - For PRS-RSRP, there can be separate tests (e.g., Test 1, Test 2, …) inside the test case, one for each side condition. - Option 2 - by multiple sub-tests in each TC, or by associating different cells with different Es/Iot in a single test

In principle, in order to minimize the number of the test cases the approach by associating different cells with different Es/Iot in a single test is more preferable. And regarding to the test cases for UE Rx-Tx time difference and PRS RSRP needs more than one cell to strictly align the practical usage (e.g. to emulate the interference from the other cells), such alternative to test the different SINR side condition within a single test is possible. Thus our recommendation is:

Proposal 6 : For the different SINR side condition , we can associate different cells with different Es/Iot in a single test when testing PRS RSRP and UE Rx-Tx time difference.

- Number of cells/TRPs for NR Positioning test case

In the last meeting, the following options on this open issue were proposed.

- Option 1a(Rohde & Schwarz, ZTE) - RSTD measurement requirements, test cases with 3 cells are developed: NR Pcell (cell 1) and two NR neighbor cells (cell 2, cell 3); - for RSTD measurement accuracy requirements, test cases with 2 cells can be sufficient, provided separate test cases are developed for measurements on the same and different frequency layers: NR Pcell (cell 1) and one NR neighbor cell (cell 2) - for PRS-RSRP (DL-AoD) and UE Rx-Tx time difference measurement requirements and measurement accuracy requirements, the same test set-up as for RSTD can be used - Option 3 (Ericsson, ZTE, CATT): - The number of cells in RSTD test cases: - 3 for measurements testing, - 2 for accuracy testing (provided there are separate test cases for the same and different frequencies). - The number of cells in PRS-RSRP test cases: - 2 for measurements testing, - 2 for accuracy testing (provided there are separate test cases for the same and different frequencies), with separate tests in the same test case for the different signal levels in the requirements; - Note: absolute measurements (on two cells) and relative measurements (on 1 cell) accuracy are tested in the same test - The number of cells in UE Rx-Tx test cases: - 2 for measurements testing, - 2 for accuracy testing (provided there are separate test cases for the same and different frequencies). - NOTE: the signal levels in the two cells correspond to the different signal levels in the requirements

For RSTD measurement, preferably the total number of cells including the serving cell should be limited to 3 in order to avoid testing complexity, i.e. reporting of RSTD for up to 2 neigbour cells can be settable. We also assume that the serving cell is the reference cell used for RSTD measurements. The related parameter (e.g. ) can be defined in the cell-specific parameters. For other types of NR positioning measurement (e.g. UE Rx-Tx timing difference and PRS RSRP) the 2 cells (one is serving cell, the other neighbor cell) are needed.

Furthermore, for the test cases of RSTD accuracy requirements, only one RSTD can be tested. That is the 2 cells can be assumed in such test.

Proposal 7-1:  3 cells deployment scenarios (one is serving/reference cell , the other two neighbor cells) can be used for RSTD measurement test.

Proposal 7-2:  2 cells deployment scenarios (one is serving/reference cell , the other neighbor cell) can be used for UE Rx-Tx time difference, PRS RSRP measurement tests.

Proposal 7-3:  2 cells deployment scenarios (one is serving/reference cell , the other two neighbor cells) can be used for RSTD/PRS RSRP/UE Rx-Tx time difference accuracy test.

- Synchronous/Asynchronous

- Option 1. (Intel, Huawei, Qualcomm): The synchronous cells will be tested for the measurement delay requirements test.

The positioning sub-frames between the cells can be synchronized or asynchronized within a slot (± 0.5 ms). However, the same measurement delay requirements will be applied for both scenarios [2]. On the other hand, in order to reduce the number of tests we propose that all tests are done using synchronous positioning sub-frames. If necessary all or some of the test cases are performed using asynchronous positioning sub-frames at a later stage for the accuracy requirements tests.

Proposal 8: The synchronous cells will be tested for the measurement delay requirements test.

- Muting

Currently, as there is no any difference of the core requirements (e.g. reporting delay) between the scenarios with and w/o muting), muting could not be applied in the positioning tests.

Proposal 9: For the core requirements test cases, only the non-muting PRS configuration will be used.

However, in the last meeting, whether the separated requirements for the cases with muting was extensively discussed without agreements. Therefore, alternatively, we can support the muting PRS configuration if the requirements were differentiated

Proposal 9a: For the core requirements test cases, the muting PRS configuration will be used if the corresponding requirements for muting defined.

- Positioning frequency layer

In the last meeting, some companies proposed that extend the agreed test cases list to two sub  test below:

| TC1a | RSTD measurement requirements for FR1 in SA, for the same frequency | A6.6.x RSTD measurements |  |  |
| --- | --- | --- | --- | --- |
| TC1b | RSTD measurement requirements for FR1 in SA, for different frequencies | A6.6.x RSTD measurements |  |  |
| TC 2a | RSTD measurement requirements for FR2 in SA, for the same frequency | A7.6.x RSTD measurements |  |  |
| TC 2b | RSTD measurement requirements for FR2 in SA, for different frequencies | A7.6.x RSTD measurements |  |  |
| TC 3a | PRS-RSRP measurement requirements for FR1 in SA, for the same frequency | A6.6.xx PRS-RSRP measurements |  |  |
| TC 3b | PRS-RSRP measurement requirements for FR1 in SA, for different frequencies | A6.6.xx PRS-RSRP measurements |  |  |
| TC 4a | PRS-RSRP measurement requirements for FR2 in SA, for the same frequency | A7.6.xx PRS-RSRP measurements |  |  |
| TC 4b | PRS-RSRP measurement requirements for FR2 in SA, for different frequencies | A7.6.xx PRS-RSRP measurements |  |  |
| TC 5a | UE Rx-Tx time difference measurement requirements for FR1 in SA, for the same frequency | A6.6.xxx UE Rx-Tx time difference measurements |  |  |
| TC 5b | UE Rx-Tx time difference measurement requirements for FR1 in SA, for different frequenciesv | A6.6.xxx UE Rx-Tx time difference measurements |  |  |
| TC 6a | UE Rx-Tx time difference measurement requirements for FR2 in SA, for the same frequency | A7.6.xxx UE Rx-Tx time difference measurements |  |  |
| TC 6b | UE Rx-Tx time difference measurement requirements for FR2 in SA, for different frequencies | A7.6.xxx UE Rx-Tx time difference measurements |  |  |
| TC 7a | RSTD measurement accuracy for FR1 in SA, for the same frequency | A6.7.x RSTD measurements |  |  |
| TC 7b | RSTD measurement accuracy for FR1 in SA, for different frequencies | A6.7.x RSTD measurements |  |  |
| TC 8a | RSTD measurement accuracy for FR2 in SA, for the same frequency | A7.7.x RSTD measurements |  |  |
| TC 8b | RSTD measurement accuracy for FR2 in SA, for different frequencies | A7.7.x RSTD measurements |  |  |
| TC 9a | PRS RSRP measurement accuracy for FR1 in SA, for the same frequency | A6.7.xx xx PRS-RSRP measurements |  |  |
| TC 9b | PRS RSRP measurement accuracy for FR1 in SA, for different frequencies | A6.7.xx xx PRS-RSRP measurements |  |  |
| TC 10a | PRS RSRP measurement accuracy for FR2 in SA, for the same frequency |  | A7.7.xx xx PRS-RSRP measurements |
| TC 10b | PRS RSRP measurement accuracy for FR2 in SA, for different frequencies |  | A7.7.xx xx PRS-RSRP measurements |
| TC 11a | UE Rx-Tx time difference measurement accuracy for FR1 in SA, for the same frequency |  | A6.7.xxx UE Rx-Tx time difference measurements |
| TC 11b | UE Rx-Tx time difference measurement accuracy for FR1 in SA, for different frequencies |  | A6.7.xxx UE Rx-Tx time difference measurements |
| TC 12a | UE Rx-Tx time difference measurement accuracy for FR2 in SA, for the same frequency |  | A7.7.xxx UE Rx-Tx time difference measurements |
| TC 12b | UE Rx-Tx time difference measurement accuracy for FR2 in SA, for different frequencies |  | A7.7.xxx UE Rx-Tx time difference measurements |

In order to simplify our testing design , the test case for a single positioning frequency layer is preferred. That is a single PRS pattern is needed for all tested cells. On the other hand, to verify the measurement cases with multiple positioning frequency layers with minimum efforts and complexity, the more than one positioning frequency layer shall be considered in the testing parameters.

Therefore, we can propose that:

Proposal 10: The number of positioning frequency layers measured can not be larger than 2.

In our views, we need not to test the PRS in same or different layer indeed. If there are multiple cells configured in a single test, it is better to associated them with the different positioning frequency layer. For an instance, if in the test there are 3 cells (e.g. cell 1, 2,3) to be measured, the PRS pattern 1 shall be used for cell1 and pattern 2 for both cell2 and cell3.

- Gap configuration

Since the new gap pattern for PRS measurement was introduced in Rel16, both the legacy measurement gap pattern and new gap pattern shall be tested for the core requirements.

Proposal 11: Gap pattern #0 and #24 can be used for NR Positioning tests.

In summary, as an example, several cell specific test parameters for FR1 RSTD measurement can be listed in the table below. The more detailed parameters configuration for these RSTD test case are also given in Appendix.

Table 2-1: Cell-specific test parameters NR RSTD measurement reporting delay

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| NR postioining frequency layer |  | 1 | 2 | 2 |
| PRS configuration Index |  | 0 | 12 | 171 |
|  | dBm/ 15 kHz | -95 |  |  |
| PRS | dB | -5.5 | -11.5 | -11.5 |
| Es/Iot | dB | -6.0 | -12.8 | -12.8 |
| Propagation Condition |  | TDL-A (30 ns delay spread, 5Hz) |  |  |
| GAP |  | #0/#24 |  |  |
| PRS muting |  | No muting |  |  |
| Power boosting |  | No power boosting |  |  |
| time shifts between the reference cell and neighbor cell | us | - | 0 | 3 |
| ExpectedRSTD-Uncertainty | us | 500 | 500 | 500 |

# Test Procedure

Nevertheless, the testing procedure can reuse that of LTE OTDOA[4, TS36.133]. That is the test consists of three consecutive time intervals, with duration of T1, T2 and T3. Cell 1 is active in T1, T2 and T3, whilst Cell 2 and Cell 3 are activated only in the beginning of T2. Cell 2 is active until the end of T3, and Cell 3 is active until the end of T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the reference cell. Cell 1 transmits PRS in T2, while Cell 2 transmits PRS only in T3, and Cell 3 transmits PRS only in T2.

Proposal 12: The test procedure for LTE OTDoA[4] can be reused for NR positioning measurement testing.

# Summary

This paper proposes the main PRS configurations parameters and test cases list to verify the RRM requirements related to NR positioning.

Proposal 1: The PRS configuration patterns in Table 1 for NR Positioning measurement tests can be specified in the Annex A.3.x of TS 38.133.

Proposal 2: SRS configuration can be specified as Table 2.

Proposal 3: It is enough to define the test cases for NR standalone cells.

Proposal 4: Absolute measurement reporting is tested for all PRS measurements. It is unnecessary to define the test case for differential RSTD.

Proposal 5:  In order to minimize the testing efforts, we can define two subtests per accuracy test cases corresponding to different combinations of {Es/Iot, PRS BW}.

Proposal 6 : For the different SINR side condition , we can associate different cells with different Es/Iot in a single test when testing PRS RSRP and UE Rx-Tx time difference.

Proposal 7-1:  3 cells deployment scenarios (one is serving/reference cell , the other two neighbor cells) can be used for RSTD measurement test.

Proposal 7-2:  2 cells deployment scenarios (one is serving/reference cell , the other neighbor cell) can be used for UE Rx-Tx time difference, PRS RSRP measurement tests.

Proposal 7-3:  2 cells deployment scenarios (one is serving/reference cell , the other two neighbor cells) can be used for RSTD/PRS RSRP/UE Rx-Tx time difference accuracy test.

Proposal 8: The synchronous cells will be tested for the measurement delay requirements test.

Proposal 9: For the core requirements test cases, only the non-muting PRS configuration will be used.

Proposal 9a: For the core requirements test cases, the muting PRS configuration will be used if the corresponding requirements for muting defined.

Proposal 10: The number of positioning frequency layers measured can not be larger than 2.

Proposal 11: Gap pattern #0 and #24 can be used for NR Positioning tests.

Proposal 12: The test procedure for LTE OTDoA[4] can be reused for NR positioning measurement testing.

# References

- R4-2103586, WF on NR Positioning Performance Requirements

- 3GPP TS 38.133 v16.6.0

- R4-2014579, Link-level simulation results for PRS RSRP measurement

- 3GPP TS 36.133 v16.5.0

- R4-2102776, Discussion on RRM test case for UE positioning requirements