3GPP TSG-RAN WG4 Meeting # 98-bis-e 	                                         R4-2106456

Electronic Meeting, Apr. 12-20, 2021

Agenda Item:	5.5.2.2.2.2

Source:	Intel Corporation

Title:	Discussion on PRS RSRP accuracy requirements for NR Positioning

Document for:		Discussion

# Introduction

In the last RAN4 meeting, PRS RSRP requirements were discussed, and some agreements achieved [1]. In this contribution we will focus on the discussion of remaining issue below which are unique for PRS RSRP measurement.

- Measurement accuracy requirements

# Discussion

According to the simulation result in Figure 1 below, we can see:

Figure 1. Simulation results for different SINR

Observation 1a: With the same PRS BW, when SINR is higher enough (>= -3dB) the performance gap because of DL-PRS-NumSymbols ,DL-PRS_ResourceRepetitionFactor and DL-PRS-CombSizeN can be less than about 1dB.

Observation 1b: With the same PRS BW, when SINR is higher enough (>= -13dB) the performance gap because of DL-PRS-NumSymbols ,DL-PRS_ResourceRepetitionFactor and DL-PRS-CombSizeN can be less than about 2dB.

Therefore, we can conclude that:

Proposal 1 : The accuracy requirements of RSRP can be independent with the following parameters:

- DL-PRS-NumSymbols

- DL-PRS_ResourceRepetitionFactor

- DL-PRS-CombSizeN

On the other hand, regarding to the simulation results above depending on the different PRS BWs, we can also see:

Figure 2. Simulation results for different PRS BW

Observation 2a: When SINR is higher enough (>= -3dB) the performance gap because of PRS BW can be less than 1dB.

Observation 2b: When SINR is higher enough (>= -13dB) the performance gap because of PRS BW can be less than 5dB.

Thus, we can conclude that:

Proposal 2-1 : The accuracy requirements of RSRP when SINR is larger than [-3dB] can be independent with PRS BS.

Proposal 2-2 : The accuracy requirements of RSRP when SINR is larger than [-13dB] can be dependent with PRS BS.

Moreover, base corresponding simulation results in Appendix, we can conclude that :

Proposal 3 : The absolute accuracy requirements of PRS RSRP can be specified by the Table 1 below.

Table 1: PRS RSRP Absolute Accuracy

| FR | PRS BW, MHz (or PRBs) | Accuracy (dB) (SINR= -3dB) | Absolute Accuracy (dB) (SINR= -13dB) |
| --- | --- | --- | --- |
| FR1 | ≥24 | [±2.0] | [±6.5] |
|  | ≥ 52 |  | [±6.5] |
|  | ≥104 |  | [±5.0] |
|  | ≥ 268 |  | [±4.5] |
| FR2 | ≥32 | [±2.0] | [±6.0] |
|  | ≥64 |  | [±6.0] |
|  | ≥128 |  | [±5.5] |

Proposal 4 : The relative accuracy requirements of PRS RSRP can be specified by the Table 21 below.

Table 2: PRS RSRP Relative Accuracy

| FR | PRS BW, MHz (or PRBs) | Accuracy (dB) (SINR= -3dB) | Absolute Accuracy (dB) (SINR= -13dB) |
| --- | --- | --- | --- |
| FR1 | ≥24 | [±2.0] | [±3.5] |
|  | ≥ 52 |  | [±3.0] |
|  | ≥104 |  | [±3.0] |
|  | ≥ 268 |  | [±3.0] |
| FR2 | ≥32 | [±1.5] | [±3.0] |
|  | ≥64 |  | [±2.0] |
|  | ≥128 |  | [±1.0] |

# Conclusion

In this contribution, some considerations on PRS RSRP measurement accuracy requirements are provided and the following observations and proposals can be drawn:

Observation 1a: With the same PRS BW, when SINR is higher enough (>= -3dB) the performance gap because of DL-PRS-NumSymbols ,DL-PRS_ResourceRepetitionFactor and DL-PRS-CombSizeN can be less than about 1dB.

Observation 1b: With the same PRS BW, when SINR is higher enough (>= -13dB) the performance gap because of DL-PRS-NumSymbols ,DL-PRS_ResourceRepetitionFactor and DL-PRS-CombSizeN can be less than about 2dB.

Proposal 1 : The accuracy requirements of RSRP can be independent with the following parameters:

- DL-PRS-NumSymbols

- DL-PRS_ResourceRepetitionFactor

- DL-PRS-CombSizeN

Observation 2a: When SINR is higher enough (>= -3dB) the performance gap because of PRS BW can be less than 1dB.

Observation 2b: When SINR is higher enough (>= -13dB) the performance gap because of PRS BW can be less than 5dB.

Proposal 2-1 : The accuracy requirements of RSRP when SINR is larger than [-3dB] can be independent with PRS BS.

Proposal 2-2 : The accuracy requirements of RSRP when SINR is larger than [-13dB] can be dependent with PRS BS.

Proposal 3 : The absolute accuracy requirements of PRS RSRP can be specified by the Table 1 below.

Table 1: PRS RSRP Absolute Accuracy

| FR | PRS BW, MHz (or PRBs) | Accuracy (dB) (SINR= -3dB) | Absolute Accuracy (dB) (SINR= -13dB) |
| --- | --- | --- | --- |
| FR1 | ≥24 | [±2.0] | [±6.5] |
|  | ≥ 52 |  | [±6.5] |
|  | ≥104 |  | [±5.0] |
|  | ≥ 268 |  | [±4.5] |
| FR2 | ≥32 | [±2.0] | [±6.0] |
|  | ≥64 |  | [±6.0] |
|  | ≥128 |  | [±5.5] |

Proposal 4 : The relative accuracy requirements of PRS RSRP can be specified by the Table 21 below.

Table 2: PRS RSRP Relative Accuracy

| FR | PRS BW, MHz (or PRBs) | Accuracy (dB) (SINR= -3dB) | Absolute Accuracy (dB) (SINR= -13dB) |
| --- | --- | --- | --- |
| FR1 | ≥24 | [±2.0] | [±3.5] |
|  | ≥ 52 |  | [±3.0] |
|  | ≥104 |  | [±3.0] |
|  | ≥ 268 |  | [±3.0] |
| FR2 | ≥32 | [±1.5] | [±3.0] |
|  | ≥64 |  | [±2.0] |
|  | ≥128 |  | [±1.0] |

# References

- R4-2103586, “WF on requirements on NR Pos performance requirements”

- 3GPP TS 38.133 v16.5.0: "NR: Requirements for support of radio resource management "

- R4-2101274, “Discussion on NR PRS RSTD measurement accuracy requirements”

# Appendix: Link level simulation results

Table A1: RSRP accuracy simulation results in the fading channel (ns)

| FR | PRS BW (PRBs) | SCS(kHz) | DL-PRS-NumSymbols | DL-PRS-CombSizeN | DL-PRS_ResourceRepetitionFactor | PRS_NormLenthPerSlot | Accuracy @ 5% (dB), SINR [-3dB] | Accuracy @ 95% (dB), SINR [-3dB] | Accuracy @ 5%, (dB), SINR -13dB | Accuracy @ 95% (dB), SINR -13dB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Intel | Intel | Intel | Intel |
| FR1 | 24 | 15 | 2 | 2 | 1 | 1 | -1.9 | 1.7 | -5.5 | 0.1 |
|  |  |  | 2 | 2 | 4 | 4 | -1.2 | 1.2 | -5.1 | 0.1 |
|  | 52 | 15 | 4 | 4 | 1 | 1 | -1.3 | 1.3 | -5.5 | -1.2 |
|  |  |  | 2 | 2 | 2 | 2 | -1.5 | 1.3 | -5.9 | -1.1 |
|  |  |  | 2 | 2 | 4 | 4 | -1.4 | 1.2 | -5.6 | -1.2 |
|  |  |  | 2 | 2 | 6 | 6 | -1.4 | 1.2 | -5 | -0.6 |
|  |  |  | 12 | 4 | 1 | 3 | -1.2 | 1.2 | -5.1 | -1.8 |
|  | 48 | 30 | 4 | 4 | 1 | 1 | -1.2 | 1.3 | -5.1 | -1.5 |
|  |  |  | 2 | 2 | 2 | 2 | -1.2 | 1.2 | -6.5 | -3.3 |
|  |  |  | 2 | 2 | 4 | 4 | -1.1 | 1.1 | -6.1 | -3.5 |
|  |  |  | 2 | 2 | 6 | 6 | -1.1 | 1.1 | -5.6 | -3.3 |
|  | 104 | 15 | 4 | 4 | 1 | 1 | -1.4 | 1.3 | -4.3 | -0.1 |
|  |  |  | 2 | 2 | 2 | 2 | -1.4 | 1.3 | -4.2 | 1 |
|  |  |  | 2 | 2 | 4 | 4 | -1.4 | 1.3 | -4 | 1.1 |
|  |  |  | 2 | 2 | 6 | 6 | -1.3 | 1.3 | -3.2 | 1.1 |
|  |  |  | 12 | 4 | 1 | 3 | -1.3 | 1.3 | -4.4 | -2.1 |
|  | 132 | 30 | 4 | 4 | 1 | 1 | -1.4 | 1.2 | -3.1 | 1.6 |
|  |  |  | 2 | 2 | 2 | 2 | -1.4 | 1.1 | -4.6 | -0.8 |
|  |  |  | 2 | 2 | 4 | 4 | -1.4 | 1.1 | -4.2 | -0.8 |
|  |  |  | 4 | 4 | 1 | 1 | -1.3 | 1.1 | -3.1 | 0 |
|  | 268 | 15 | 2 | 2 | 2 | 2 | -1.6 | 1.2 | -2.3 | 3 |
|  |  |  | 2 | 2 | 4 | 4 | -1.5 | 1.2 | -1.1 | 4.5 |
|  |  |  | 2 | 2 | 6 | 6 | -1.5 | 1.1 | -1.1 | 4.4 |
|  |  |  | 2 | 2 | 6 | 6 | -1.5 | 1.1 | 0 | 4.5 |
|  |  |  | 12 | 4 | 1 | 3 |  |  |  |  |
|  | 272 | 30 | 4 | 4 | 1 | 1 | -0.9 | 1 | -0.5 | 4.6 |
|  |  |  | 2 | 2 | 2 | 2 | -0.9 | 1 | -3.3 | 1.5 |
|  |  |  | 2 | 2 | 4 | 4 | -0.9 | 1 | -2.2 | 1.9 |
|  |  |  | 2 | 2 | 6 | 6 | -0.9 | 1 | -1.9 | 1.9 |
| FR2 | 24 | 120 | 2 | 2 | 1 | 1 | -1.8 | 0.8 | -5.6 | -0.9 |
|  | 32 | 120 | 4 | 4 | 1 | 1 | -1.8 | 0.7 | -4.7 | -1.6 |
|  |  |  | 2 | 2 | 2 | 2 | -1.4 | 0.9 | -5.2 | -3.2 |
|  |  |  | 2 | 2 | 4 | 4 | -1.2 | 0.9 | -5.1 | -3.7 |
|  |  |  | 2 | 2 | 6 | 6 | -1.1 | 0.9 | -4.7 | -3.4 |
|  |  |  | 12 | 4 | 1 | 3 | -1.3 | 0.6 | -4.5 | -2.5 |
|  |  |  | 12 | 6 | 1 | 2 |  |  |  |  |
|  | 64 | 120 | 2 | 2 | 2 | 2 | -1.3 | 0.9 | -6 | -4.3 |
|  |  |  | 2 | 2 | 4 | 4 | -1.3 | 0.9 | -6 | -4 |
|  |  |  | 2 | 2 | 6 | 6 | -1.3 | 0.9 | -5.6 | -3.8 |
|  |  |  | 12 | 4 | 1 | 3 | -1.3 | 0.4 | -5.5 | -3.5 |
|  | 128 | 120 | 4 | 4 | 1 | 1 | -1.6 | 0.1 | -4.1 | -0.6 |
|  |  |  | 2 | 2 | 2 | 2 | -1 | 0.8 | -5.5 | -2.9 |
|  |  |  | 2 | 2 | 4 | 4 | -0.8 | 0.8 | -5.5 | -3.5 |
|  |  |  | 2 | 2 | 6 | 6 | -0.8 | 0.8 | -4.8 | -3.2 |
|  |  |  | 12 | 4 | 1 | 3 | -1.6 | 0 | -5.4 | -4.1 |