3GPP TSG-RAN WG4 Meeting#98bis-e	R4- 2107033

Online, 12th Apr – 20th Apr, 2021

Source: 	Qualcomm Incorporated

Title: 	Test Methodology for Application Layer Throughput Tests

Agenda item:	9.5.2

Document for:	Discussion

- Introduction

In RAN#90-e, RAN4 objectives were added to SID [1] to study the feasibility of defining requirements with link adaptation. In this paper, we provide our views on methodology to align simulation results and define requirements.

- Test Methodology

During the RAN plenary discussion on defining link adaptation requirements in RAN4, one of the major concerns was how to align the simulation results between different companies and how to define requirements with absolute throughput test metric.

To define link adaptation requirements in RAN4, SNR needs to be fixed. RAN4 can choose to define the requirements for high SNR (for example). At that SNR, Rank will most likely be aligned between different companies. So, we looked at some PMI reporting simulation results for Rel-15 Single Panel Type I codebook around 90% of peak throughput since UEs try to operate in 10% BLER regime with link adaptation. We provide some examples of throughput alignment from Rel-15 Single Panel Type I PMI reporting simulation results below.

Table 1: Throughputs in bits/s with following PMI for 4x2 FR1 FDD [2]

| SNR (dB) | Company A | Company B | Company C | Span/Average |
| --- | --- | --- | --- | --- |
| 2 | 8112525.00 | 8355760.00 | 8291101.50 | 2.95% |
| 4 | 8748300.00 | 8842778.00 | 8860101.35 | 1.27% |
| 6 | 8972175.00 | 8996890.00 | 9002496.14 | 0.34% |
| 8 | 9011925.00 | 9026388.00 | 9026943.79 | 0.17% |
| 10 | 9030000.00 | 9029398.00 | 9028165.67 | 0.02% |

Table 2: Throughputs in bits/s with following PMI for 4x2 FR1 TDD [3]

| SNR (dB) | Company A | Company B | Company C | Span/Average |
| --- | --- | --- | --- | --- |
| 2 | 24953357.14 | 25279752.14 | 26123509.17 | 4.60% |
| 4 | 27466189.29 | 27829611.39 | 27789855.21 | 1.31% |
| 6 | 28155121.43 | 28254997.75 | 28208368.91 | 0.35% |
| 8 | 28262400.00 | 28277127.67 | 28262400.00 | 0.05% |

Table 3: Throughputs in bits/s with following PMI for 2x2 FR2 TDD [4]

| SNR (dB) | Company A | Company B | Company C | Span/Average |
| --- | --- | --- | --- | --- |
| 4 | 33234072.48 | 32901655.15 | 35866371.89 | 8.72% |
| 6 | 41468476.00 | 41994476.22 | 40863824.74 | 2.73% |
| 8 | 42314800.00 | 42353403.36 | 42254839.84 | 0.23% |
| 10 | 42314800.00 | 42353403.36 | 42314800.00 | 0.09% |

Based on above examples, we notice that span of absolute throughputs across companies is not very large compared to average throughput. Therefore, we propose the following.

Proposal 1: For alignment of absolute LA throughput simulation results, span across companies should be within X% of average LA throughput.

Proposal 2: X can be determined based on simulation results.

Proposal 3: Use X = [5]% or [10]% as possible values.

To define the requirements, RAN4 needs to account for the span in alignment results. Therefore, we propose the following.

Proposal 4: If simulation results are aligned, define the absolute throughput requirements by averaging the impairment results across companies and adding a margin of Y% of average impairment LA throughput.

Proposal 5: Use Y = [5]% or [10]% as possible values.

- Conclusions

This paper provides views on test methodology for defining LA throughput requirements in RAN4. Following has been proposed.

Proposal 1: For alignment of absolute LA throughput simulation results, span across companies should be within X% of average LA throughput.

Proposal 2: X can be determined based on simulation results.

Proposal 3: Use X = [5]% or [10]% as possible values.

Proposal 4: If simulation results are aligned, define the absolute throughput requirements by averaging the impairment results across companies and adding a margin of Y% of average impairment LA throughput.

Proposal 5: Use Y = [5]% or [10]% as possible values.

# References

[1] RP-202849, “Study on 5G NR UE Application Layer Data Throughput Performance”, 3GPP TSG RAN Meeting #90-e, Online, Dec 7 - Dec 11, 2020.

[2] R4-1903382, “Summary of simulation results of NR UE CSI (FR1 FDD)”, 3GPP TSG RAN WG4 Meeting #90bis, Xi’an, China, Apr 8 - Apr 12, 2019.

[3] R4-1903383, “Summary of simulation results of NR UE CSI (FR1 TDD)”, 3GPP TSG RAN WG4 Meeting #90bis, Xi’an, China, Apr 8 - Apr 12, 2019.

[4] R4-1903384, “Summary of simulation results of NR UE CSI (FR2 TDD)”, 3GPP TSG RAN WG4 Meeting #90bis, Xi’an, China, Apr 8 - Apr 12, 2019.