3GPP TSG-RAN WG4 Meeting # 98-bis-e 	R4-2107335

Electronic Meeting, Apr. 12-20, 2021

Source:	T-Mobile USA

Title:	TP for TR38.841 for PC2 CA_n41A-n71A

Agenda item:	7.20.2

Document for:	Approval

- Introduction

This contribution is a text proposal to introduce PC2 2DL/2UL CA_n41A-n71A according to the request in [1].

The power class 2 case a MSD is calculated by calculating the interference power for 11 dB MSD, increasing the interference by 3 dB, and then calculating the resulting MSD of 13.8 dB. The spreadsheet is attached.

- Reference

[1] RP-210476, Revised basket WID: High power UE for NR inter-band Carrier Aggregation with 2 bands downlink and x bands uplink (x =1,2)

- Text Proposal

<Beginning of changes>

### 5.7.4		∆TIB and ∆RIB values

For the ∆TIB,c and ∆RIB,c values, same PC3 CA_n66A-n77A requirements are applied for PC2 CA_n66A-n77A.

## 5.x	CA_n41A-n71A

### 5.x.1	Configurations

Table 5.x.1-1: NR CA configurations and bandwidth combinations sets for supporting power class 2

| NR CA configuration | Uplink CA configuration | NR Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n41A-n71A | CA_n41A-n71A | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |

### 5.x.2	Maximum output power

Table 5.x.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n41A-n71A | CA power class | Carrier n41 power class | Carrier n71 power class |
| --- | --- | --- | --- | --- |
| CA_n41A-n71A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |

### 5.x.3	REFSENS requirements

According to the PC3 CA_n41A_n71A study, there is a  4th harmonic of n71 that falls in n41, but since n71 is only PC3 in this combination, the MSD due to harmonic interference is already captured in 38.101-1.

There is also an IMD4 that falls into n71. The PC3 MSD is 11 dB.

#### 5.x.3.1	Power class 2 case a

The power class 2 case a MSD is calculated by calculating the interference power for 11 dB MSD, increasing the interference by 3 dB, and then calculating the resulting MSD of 13.8 dB.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n71 | n41 | 2614 | 5 | 25 | 2614 | N/A | TDD | N/A |
|  | n71 | 665 | 5 | 25 | 619 | 13.8 | FDD | IMD4 |

#### 5.x.3.2	Power class 2 case c

For case B  there is no additional MSD due to IMD4.

### 5.x.4	Void

<Editor’s note: If no change by comparing to the values for power class 3 CA, this section will be void.>

<End of changes>