3GPP TSG-RAN WG4 Meeting # 98-bis-e	R4-210xxxx

- Electronic meeting, 12 – 20 April 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	TP to TR 38.849 on NR-ARFCN and GSCN points

Agenda item:	7.38.1

Document for:	Approval

# Introduction

In this contribution, we provide Text proposal for TR 38.849 with NR-ARFCN and GSCN points for 6GHz EU band, that were agreed during RAN4#98-bis-e meeting.

# TP

**************************************** Start of the TP *******************************************

# 5	NR Frequency band definition

## 5.1	Band definition

## 5.2	NR-ARFCN and GSCN

Operation in the 6GHz EU band is to be aligned with other technologies operation in the same shared spectrum restricted to the following NR-ARFCN and GSCN points.

Applicable GSCN in Europe:

- GSCN = {9548, 9562, 9576, 9590, 9603, 9617,9631, 9645, 9659, 9673, 9687, 9701, 9714, 9728, 9742, 9756, 9770, 9784, 9798, 9812, 9826, 9840, 9853, 9867}

Applicable NR-ARFCN in Europe:

- For 20 MHz channel bandwidth, NREF = {797000, 798332, 799668, 801000, 802332, 803668, 805000, 806332, 807668, 809000, 810332, 811668, 813000, 814332, 815668, 817000, 818332, 819668, 821000, 822332, 823668, 825000, 826332, 827668}

- For 40 MHz channel bandwidth, NREF = {797668, 800332, 803000, 805668, 808332, 811000, 813668, 816332, 819000, 821668, 824332, 827000}

- For 60 MHz channel bandwidth, NREF = {798332, 799668, 803668, 805000, 809000, 810332, 814332, 815668, 819668, 821000, 825000, 826332}

- For 80 MHz channel bandwidth, NREF = {799000, 804332, 809668, 815000, 820332, 825668, 831000}

***************************************** End of the TP *******************************************

# Conclusion

In this contribution a TP capturing the NR-ARFCN and GSCN points for 6GHz EU band is provided for agreement.

# References

- R4