3GPP TSG-RAN WG4 Meeting # 98-bis-e        	R4-21xxxxx

- Electronic Meeting, 12-20 April, 2021

Title:	 LS on SCell activation requirements for NR-U

Release:	Release 16

Work Item:	NR_unlic-Core

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

#### Name:	Riikka Dimnik

E-mail Address:	riikka.dimnik@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

RAN4 has discussed how to define NR-U SCell activation/deactivation requirement applicability when sCellDeactivationTimer is not configured by the network. Based on the discussion, RAN4 would like to inform RAN2 that the following agreements have been made to address the issue:

    - Applicability of SCell activation requirements when sCellDeactivationTimer is NOT configured

      - SCell activation delay requirements are applicable in

        - Scenario A (CA with NR PCell and NR SCell) with any LBT type

        - Scenario B and C (E-UTRAN-NR-U DC/SA NR-U) with LBT type 2C.

        - In all scenarios, if the UE does not experience any UL LBT failures during SCell activation/deactivation.

      - For all other scenarios the SCell activation requirements for NR-U do not apply, when the SCell activation delay exceeds the pre-defined time period T = 1280 ms.

        - Note 1: UE behavior for this case is left undefined

        - Note 2: Pre-defined time period T = 1280ms corresponds to the longest possible value of sCellDeactivationTimer

      - Send LS to RAN2 to inform on the agreements

To RAN2:

RAN4 respectfully asks RAN2 to take the presented RAN4 agreements into account.

3. Date of Next TSG-RAN4 Meetings:

TSG-RAN WG4 Meeting #99e			19th - 27th May, 2021

TSG-RAN WG4 Meeting #100-e			16th – 27th August, 2021