3GPP TSG-RAN WG4 Meeting # 98bis-e    												 R4-21xxxxx

Electronic Meeting, Apr. 12-20, 2021

Title:	Reply LS on temporary RS for efficient SCell activation in NR CA

Response to:	R1-2009798

Release:	Rel-17

Work Item:	LTE_NR_DC_enh2

Source:	RAN WG4

To:	RAN WG1

Cc:

Contact Person:

#### Name:	Han Jing

#### E-mail Address:	hw.hanjing@huawei.com

Attachments:

1. Overall Description:

RAN4 thanks the LS from RAN1 on temporary RS for efficient SCell activation in NR CA. In RAN4#98e, an LS reply on partial questions was sent to RAN1. During this RAN4 meeting, RAN4 further discussed the remaining questions and achieved the following conclusions:

Q1: to expedite SCell activation, RAN1 is studying whether and under which conditions (e.g. FR1/FR2, known/unknown cell, etc.), how many temporary RS bursts/symbols are required to achieve both UE AGC setting and time/frequency tracking. Does RAN4 have any information to share for these aspects?

[RAN4 Response]: In RAN4#98e, the following agreements are achieved:

- SCell to be activated is known and belongs to FR1 - If SCell measurement cycle is equal to or smaller than 160ms - temporary RS can be used for time/frequency tracking - 1 burst (2-slot with four CSI-RS resources) is required based on RAN1 working assumptions on temporary RS design provided in the LS R1-2009798. - If SCell measurement cycle is larger than 160ms - temporary RS can be used for AGC - 1 burst (2-slot with four CSI-RS resources) is required - temporary RS can be used for time/frequency tracking - 1 separate burst (2-slot with four CSI-RS resources) is required in addition to the one burst required for AGC - The agreements above apply based on RAN1 working assumptions on temporary RS design provided in the LS R1-2009798.

In this meeting RAN4 further reached the following conclusions:

    - Minimum gap between the RS symbol(s) for AGC and the RS symbols for time/frequency acquisition is needed to account for UE AGC application time delay

      - The minimum gap length is 2 [ms].

- SCell is unknown and belongs to FR1

  - When SCell is contiguous to an active serving cell in the same band (Intra-band continuous CA)

    - UE can perform AGC adjustment based on temporary RS;

      - Option1: Two temporary RS bursts are required.

      - Option2: One temporary RS burst is required.

    - No cell detection provided the conditions specified for intra-band contiguous CA case in TS38.133 section 8.3.2 are satisfied;

    - UE can perform time-frequency tracking based on temporary RS

      - One temporary RS burst is required.

  - So far there is no conclusion on whether/how much benefit can be achieved for the temporary RS based SCell activation in other scenarios (when SCell is non-contiguous to an active serving cell in the same band, and when SCell to be activated and active serving cell are in the different band) RAN4 will continue the discussion and provide feedback to RAN1 if there is conclusion..

- SCell to be activated belongs to FR2

  - If there is at least one active serving cell on that FR2 band and temporary RS for the target SCell is provided, no matter whether the SCell to be activated is known or unknown

    - Temporary RS can be used for time/ frequency tracking

      - 1 burst is required (“1-slot with two CSI-RSs resources (2 samples) or 2-slot with four CSI-RSs resources (4 samples)” or only “2-slot with four CSI-RSs resources (4 samples)”)

  - If there is no active serving cell on that FR2 band, and the SCell to be activated is known to UE

    - Temporary RS can be used for fine timing tracking

      - 1 burst is required (“1-slot with two CSI-RSs resources (2 samples) or 2-slot with four CSI-RSs resources (4 samples)” or only “2-slot with four CSI-RSs resources (4 samples)”)

  - If the SCell being activated is unknown and there is no active serving cell on that FR2 band,

    - Temporary RS can not be used for AGC

    - Whether temporary RS can be used for time/ frequency tracking is under discussion

Q2: for AGC setting in intra-band CA comprising of a to-be-activated SCell and an activated serving cell, when a temporary RS is transmitted on the to-be-activated SCell, whether and under which conditions (e.g., FR1/FR2, known/unknown cell, etc.) the UE may require to receive another RS transmitted also on the other activated serving cell in the same band?

[RAN4 Response]: In RAN4 the following.

- Based on the legacy requirement assumption, UE expects another RS and/or SSB (burst) is also transmitted on the other activated serving cell, having all the RSs time-aligned within MTRD requirement for intra-band CA

- RAN4 is still discussing whether the legacy assumption has to be updated.

2. To RAN WG1 group.

ACTION: 	RAN4 respectfully asks RAN1 to take the above agreements into account.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN4 Meeting #99-e		Online

TSG-RAN4 Meeting #100-e		Online