3GPP TSG-RAN4 Meeting #99-e                                                    R4-2111508

Electronic Meeting, 19th May – 27th May

Title:		        FR2 Extreme temperature conditions applicability

Source:	        Keysight Technologies UK Ltd

Agenda Item:	4.1.2.2

Document for:	Discussion and Endorsement

# 1.	Introduction

FR2 enhanced test method study item included among their objectives the feasibility of FR2 extreme testing conditions as, when Rel-15 core requirements were defined, there were some concerns about whether there could be some restrictions testing extreme temperature conditions in an OTA measurement set-up, especially for spherical measurements.

Now that ETC testability has progressed in either RAN4 and RAN5 ([2], [3], [4], [5], [6], [7], [8]) and some details are finalized on measurement uncertainties under ETC ([9], [10]), a full revision of ETC applicability to FR2 test cases is required.

# 2.	Extreme temperature conditions in TS38.101-2 [1]

Core requirements for FR2 Standalone ([12]) includes the following statement in Annex E:

Observation 1: Unless otherwise stated, all core requirements are applicable under nominal and extreme environmental testing conditions.

Proposal 1: RAN4 to confirm that, unless otherwise stated, all core requirements are applicable under nominal and extreme environmental testing conditions.

[1] only includes the following explicit exceptions to extreme testing conditions:

Some of these exceptions are thought to be caused by FR2 ETC testing feasibility concerns raised during Rel-15 core requirements definition, specifically those affecting EIRP and EIS spherical coverage as well as those for beam correspondence. It should be noticed that in [2], the following details on test procedure were added for 3D scans:

Hence spherical measurements are feasible, and it sounds reasonable that either 3D scans as well as EIRP/EIS spherical coverage and beam correspondence measurement are performed under the same testing conditions.

Proposal 2: RAN4 to confirm that existing EIRP/EIS spherical coverage core and beam correspondence core requirements apply under extreme temperature conditions. Hence changes described in [11] are agreeable.

# 3.	Conclusion

This document proposes a revision on FR2 core requirements under extreme temperature conditions as follows:

Observation 1: Unless otherwise stated, all core requirements are applicable either under nominal or extreme environmental testing conditions.

Proposal 1: RAN4 to confirm that, unless otherwise stated, all core requirements are applicable under nominal and extreme environmental testing conditions.

Proposal 2: RAN4 to confirm that existing EIRP/EIS spherical coverage core and beam correspondence core requirements apply under extreme temperature conditions. Hence changes described in [11] are agreeable.

# 4.	References

[1] 38.101-2 v16.7.0, “User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”

[2] R4-2106129, “TP to TR38.884 v0.2.0 on ETC system “, Vivo, RAN4#98-bis-e

[3] R5-204739, “Preliminary QoQZ and EIRP MU results for ETC testing “, Keysight Technologies, RAN5#88-e

[4] R5-206819, “QoQZ and EIRP/EIS MU results for ETC testing”, Keysight Technologies, RAN5#89-e

[5] R5-206821, “CR to 38.521-2 on ETC Testing”, Keysight Technologies, RAN5#89-e

[6] R5-206835, “CR to 38.903 on ETC Testing”, Keysight Technologies, RAN5#89-e

[7] R5-211184, “On ETC MUs”, Keysight Technologies, Rohde & Schwarz, RAN5#90-e

[8] R5-211185, “CR to 38.903 on ETC Testing”, Keysight Technologies, RAN5#90-e

[9] R5-211935, “CR to 38.903 on ETC Testing”, Keysight Technologies, RAN5#90-e

[10] R5-211925, “Update of ETC MTSU”, Rohde & Schwarz, RAN5#90-e

[11] R4-2111507, “FR2 Extreme Temperature Conditions revision”, Keysight Technologies, RAN4#99