3GPP TSG RAN WG4 Meeting NR-AH#1807		R4-1808769

Montreal, Canada, 2nd – 6th July, 2018

Source: 	NTT DOCOMO, INC.

Title: 	PH report mapping table for NR

Agenda item:	5.3.2

Document for:	Discussion

- Introduction

At the April meeting, RAN4 agreed that PH range would need to be expanded based on LTE PHR [1]. In this contribution, we provide our views on how to expand the mapping table.

- Discussion

LTE PH report mapping table is defined as table 1, where value range is defined from -23 to 40 with 1dB step. In NR case, the maximum number of RBs and Pcmax,c for mm-wave are larger than those in LTE. This is why RAN4 agreed that PH range would need to be expanded based on LTE PHR. However, it has already agreed that the number of reported values was 64 (i.e., by using 6-bits) in RAN2, which is the same value as LTE. In order to define the PH report mapping table, RAN4 at first needs to decide how to expand the value range by using the same table size, e.g., applying larger step size to at least the part of the mapping table

In our view, more accurate reporting is needed for smaller PH value range since small PH value means that transmission power is close to maximum value and fine control is needed. Thus 1dB resolution should be kept for small PH values and larger step size should be applied for large PH values.

Table 1: Power headroom report mapping for LTE

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -23 PH -22 |
| POWER_HEADROOM_1 | -22 PH -21 |
| POWER_HEADROOM_2 | -21 PH -20 |
| POWER_HEADROOM_3 | -20 PH -19 |
| POWER_HEADROOM_4 | -19 PH -18 |
| POWER_HEADROOM_5 | -18 PH -17 |
| POWER_HEADROOM_57 | 34 PH 35 |
| POWER_HEADROOM_58 | 35 PH 36 |
| POWER_HEADROOM_59 | 36 PH 37 |
| POWER_HEADROOM_60 | 37 PH 38 |
| POWER_HEADROOM_61 | 38 PH 39 |
| POWER_HEADROOM_62 | 39 PH 40 |
| POWER_HEADROOM_63 | PH ≥ 40 |

Proposal 1:

1dB resolution should be kept for small PH values.

Proposal 2:

Larger step size should be applied for large PH values. The step size can be discussed based on the minimum and maximum PH values.

In order to define the PH report mapping table for NR, RAN4 also needs to decide the possible maximum and minimum PH values since higher value of maximum output power than that in LTE is supported, especially for FR2 [2]. According to [2], UE maximum output power is specified by TRP and EIRP for each power class. TRP means the total transmit power including all of directions and EIRP means the maximum peak power including beamforming gain at the certain direction. From operator’s perspective, PH value calculated by EIRP is needed in order to know the appropriate path loss at the UE point.

Proposal 3:

The value of UE maximum output power specified by EIRP should be considered when PH value range is decided.

Table 2 shows how to define PH report mapping table based on our proposals. The maximum and minimum PH values in table 1, which are highlighted by red, are just example value.

Change step size,

e.g.2dB step

Keep 1dB step

Table 2: The principle of power headroom report mapping for NR

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | PH -32 (just an example) |
| POWER_HEADROOM_1 | -32 PH -31 |
| POWER_HEADROOM_2 | -31 PH -30 |
| POWER_HEADROOM_3 | -30 PH -29 |
| POWER_HEADROOM_4 | -29 PH -28 |
| POWER_HEADROOM_5 | -28 PH -27 |
| POWER_HEADROOM_54 | 26  PH  27 |
| POWER_HEADROOM_55 | 27  PH  28 |
| POWER_HEADROOM_56 | 28  PH  30 |
| POWER_HEADROOM_57 | 30  PH  32 |
| POWER_HEADROOM_58 | 32  PH  34 |
| POWER_HEADROOM_59 | 34  PH  36 |
| POWER_HEADROOM_60 | 36  PH  38 |
| POWER_HEADROOM_61 | 38  PH  40 |
| POWER_HEADROOM_62 | 40  PH  42 |
| POWER_HEADROOM_63 | PH ≥ 42 (just an example) |

1dB step

- Conclusion

In this contribution, we provided our view on PH report mapping table. Our proposals are as follows:

Proposal 1:

1dB resolution should be kept for small PH values.

Proposal 2:

Larger step size should be applied for large PH values. The step size can be discussed based on the minimum and maximum PH values.

Proposal 3:

The value of UE maximum output power specified by EIRP should be considered when PH value range is decided.

- Reference

[1] RAN4#86bis chairman’s note

[2] TS38.101-2