# 3GPP TSG-RAN WG4 Meeting AH-1807 LTE/NR perf                                  R4-1808822

# Montreal, Canada, 2th – 6th July, 2018

Source: 	ZTE Corporation

Title: 	TP to TS 38.141-1: Test configurations for multi-band(Sections 4.8.4)

Agenda Item:	5.1.3

Document for:	Approval

1	Introduction

In the #86bis meeting,  the WF [1] for NR BS test configurations was agreed, where the following five NRTCs were agreed:

NRTC1: NR contiguous spectrum operation

NRTC2: Contiguous CA occupied bandwidth

NRTC3: NR non-contiguous spectrum operation

NRTC4: NR multi-band test configuration for full carrier allocation

NRTC5: NR multi-band test configuration with high PSD per carrier

Since the TS38.141-1 spec is mainly focus on the conductive test requirements, and the generation for NRTCs are use the similar principle as E-UTRA(or Rel-13 AAS), and also the test configuration for each requirements for multi-carrier and multi-band should be specified.

In this contribution, we provide a TP to TS38.141-1[2] on the test configurations (NRTC1~5) for each requirements multi-band in section 4.8.4 based on contribution of test configuration generated in[3]

2	Reference

[1] R4-1805872, WF on Test configuration for NR BS conformance testing, Huawei, Ericsson, Nokia

[2] 38.141-1 v0.3.0

[3] R4-1808820, TP to TS38.141-1: Test configuration(Sections 4.7) , ZTE Corporation

--------------Start of TP  -------------

4.8.4	Test configurations for multi-band

For a NR BS declared to be capable of multi-band operation, the test configuration in Table 4.8.4-1 shall be used for testing. In the case where multiple bands are mapped on multi-band connector, the test configuration in the second column of Table 4.8.4-1 shall be used. In the case where multiple bands are mapped on single-band connectors, the test configuration in the third column of Table 4.8.4-1 shall be used.

Table 4.8.4-1: Test configuration for a NR BS capable of multi-band operation

| BS test case | Test configuration |  |
| --- | --- | --- |
|  | single-band connector | multi-band connector |
| 6.2 Base station output power | NRTC1/NRTC3 (Note 1), NRTC4 | NRTC1/NRTC3 (Note 1), NRTC4 |
| 6.3 Output power dynamics |  |  |
| 6.3.1 RE Power control dynamic range | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude |
| 6.3.2 Total power dynamic range | SC | SC |
| 6.4 Transmit ON/OFF power | NRTC4 (only applied for NR TDD BS) | NRTC4(only applied for NR TDD BS) |
| 6.5 Transmitted signal quality |  |  |
| 6.5.1 Frequency error | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude |
| 6.5.2 Error Vector Magnitude | NRTC1/NRTC3 (Note 1), NRTC4 | NRTC1/NRTC3 (Note 1), NRTC4 |
| 6.5.3 Time alignment error | NRTC1/NRTC3 (Note 1), NRTC5 (Note 2) | NRTC1/NRTC3 (Note 1), NRTC5 (Note 2) |
| 6.6 Unwanted emissions |  |  |
| 6.6.2 Occupied bandwidth | SC, NRTC2 (Note 3) | SC, NRTC2 (Note 3) |
| 6.6.3 Adjacent Channel Leakage power Ratio (ACLR) | NRTC1/NRTC3 (Note 1, 5), NRTC5 (Note 4, 5) | NRTC1/NRTC3 (Note 1), NRTC5 (Note 4) |
| 6.6.3 Cumulative ACLR requirement in non-contiguous spectrum (CACLR) | NRTC3 (Note 1, 5) | NRTC3 (Note 1), NRTC5 (Note 4) |
| 6.6.4 Operating band unwanted emissions | NRTC1/NRTC3 (Note 1, 5), NRTC5 (Note 5) | NRTC1/NRTC3 (Note 1), NRTC5 |
| 6.6.5 Transmitter spurious emissions | NRTC1/NRTC3 (Note 1, 5), NRTC5 (Note 5) | NRTC1/NRTC3 (Note 1), NRTC5 |
| 6.7 Transmitter intermodulation | NRTC1/NRTC3 (Note 1, 5) | NRTC1/NRTC3 (Note 1) |
| 7.2 Reference sensitivity level | SC | SC |
| 7.3 Dynamic range | SC | SC |
| 7.4 In-band selectivity and blocking |  |  |
| 7.4.1 Adjacent Channel Selectivity(ACS) | NRTC1/NRTC3 (Note 1), NRTC5 (Note 6) | NRTC5 |
| 7.4.2 In-band Blocking | NRTC1/NRTC3 (Note 1), NRTC5 (Note 6) | NRTC5 |
| 7.4.2 Narrowband blocking | NRTC1/NRTC3 (Note 1), NRTC5 (Note 6) | NRTC5 |
| 7.5 Out-of-band Blocking | NRTC1/NRTC3 (Note 1), NRTC5 (Note 6) | NRTC5 |
| 7.6 Receiver spurious emissions | NRTC1/NRTC3 (Note 1, 5), NRTC5 (Note 5) | NRTC1/NRTC3 (Note 1), NRTC5 |
| 7.7 Receiver intermodulation | NRTC1/NRTC3 (Note 1), NRTC5 (Note 6) | NRTC5 |
| 7.8 In-channel selectivity | SC | SC |
| Note 1: NRTC1 and/or NRTC3 shall be applied in each supported operating band according to Tables 4.8.3-1 . Note 2: NRTC5 is only applicable when inter-band CA is supported. Note 3: NRTC2 is only applicable when contiguous CA is supported. Note 4: NRTC5 may be applied for Inter RF Bandwidth gap only. Note 5: Single-band requirement apply to each antenna connector for both multi-band operation test and single-band operation test. For single-band operation test, other antenna connector(s) is (are) terminated. Note 6: NRTC5 is only applicable for multi-band receiver. |  |  |

--------------End of TP ------------